# 3GPP TSG-SA5 Meeting #157	S5-245406

# 14 - 18 October 2024, Hyderabad, India

# Source:	Nokia

# Title:	pCR 28.867 Avoidance of action-execution-time conflicts

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.867-041 “Closed control loop management”.

# 3	Rationale

TR28.867 has studied a use case on CCL conflicts management with a requirement to support a capability to avoid CCL conflicts including action-execution-time conflicts. No solution has been provided for this requirement for action-execution-time conflicts. This pCR adds a solution for Avoidance of action-execution-time conflicts

# 4	Detailed proposal

Start of modification

# 5. 			Use Cases

## 5.6	Use case 6: CCL conflicts management

### 5.6.3			Potential Solutions

#### 5.6.3.6 	Action-execution-time conflict coordination

##### 5.6.3.6.2 Avoidance of action-execution-time conflicts

###### 5.6.3.6.2.1	Required capabilities and interactions.

In a network with several CCLs, it can be that each CCL focuses on a smaller scope of the network problem space. And thus propose changes that are suboptimal to the full problem space. To support the full problem space, several CCLs may be executed but it is important that their actions do not collide with each other in any one given network scope. The CCLs can be explicitly scheduled to act by the CCL coordination entity which has a wider view of the network problems, the capabilities of the CCLs and their likely impacts. The CCL coordination entity obtains the capabilities of the available CCLs including their desired scopes and execution times. The CCL coordination entity evaluates network data and analytics to identify the nature of the problem and best CCLs to be scheduled at specific times to address the problem but without their execution conflicting with one another.

Figure 5.6.3.6.2.1-1: Avoiding action-execution-time conflicts by identifying and triggering the appropriate CCLs

###### 5.6.3.6.2.2	Information objects to realize required capabilities and interactions

-

The solution resues the attributes that enable the CCLs to informe the coordination entity (CE) about the proposed actions and the time under which the reconfiguration is planned to be executed. The coordination entity can use this information to derive a schedule for when the CCLs should be executed. For this,

-	Introduce on the CCL coordination entity an attribute for a schedule representing the schedule in which a set of CCL are proosed to be executed.

The schedule indicates thetime for when a CCL may be exeucte actions for a given network scope. The planned schedule time for each CCL can be notified to the respective CCLs.

Next modifications

Annex A:
PlantUML Code for figures

# A.2	Procedures for CCL Management

## A.2.x9	Avoiding action-execution-time conflicts (Figure 5.6.3.6.2.1-1)

@startuml Avoiding action-execution-time conflicts

skinparam Shadowing false

autonumber

skinparam monochrome true

skinparam maxMessageSize 300

collections "CCL \n(CCL MnS producers &\n Coordination MnS Consumers)" as CCLs

participant "CCL Coordination entity \n(CCL Coordination MnS Producer)" as Orch

CCLs -> Orch: Provide CCL’s desired control scope and execution time

Orch -> Orch: Read network performance data, obtain analytics, identify network problem

Orch -> Orch: Match network problem to best CCLs, schedule CCL execution

Orch -> CCLs: Schedule CCL and notify execution time and control scope.

@enduml

End of modifications