3GPP TSG-SA5 Meeting #157	S5-246041

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Enhancements to the evaluation of the potential solutions for configuring discovery information

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to enhance the evaluation of the potential solutions of configuring discovery information for an external MnS consumer to point out what will be required at the CCF authorization server to support the potential solutions.

# 4	Detailed proposal

It is proposed that the following changes be made to clause 5.1.3.4 of TR 28.879 [1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]            	3GPP TS 28.533: “Management and orchestration; Architecture Framework”.

[3]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

[4]           	3GPP TS 28.537: “Management and orchestration; Management capabilities”.

[5]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[6]	SP-231669: "LS on collaboration and alignment of 3GPP defined application enablers with GSMA Open Gateway".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.255: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[9]	3GPP TS 23.286: "Application layer support for Uncrewed Aerial Systems (UAS) services; Functional architecture and information flows".

[10]	3GPP TS 23.545: "Application layer support for Factories of the Future (FF) ".

[11]	3GPP TS 23.542: "Application layer support for Personal IoT Networks".

[12] 	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[13]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[14]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP Northbound APIs".

[15]	 "The Ecosystem for Open Gateway NaaS API Development", white paper, June 2023 [link]

[16]	"GSMA Operator Platform Group – Requirements and Architecture", version 5.0, July 2023 [link]

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.531: "Management and orchestration; Provisioning"

[19]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service"

[20]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[21]	3GPP TS 28.550: "Management and orchestration; Performance assurance"

[22]	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)"

[23]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management"

[24]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

[25]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and 3"

[26]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[27]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management"

[28]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and 3"

[29]	3GPP TS 28.319: “Management and orchestration; Access control for management services”

[30]                      3GPP TS 32.158: " Management and orchestration;Design rules for REpresentational State Transfer (REST) Solution Sets (SS)”.

[31]                      IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[32]                       IETF RFC 7519: “JSON Web Token (JWT)”.

[X]                        3GPP TS 32.161: "Management and orchestration; JSON expressions (Jex)".

Second Change

#### 5.1.3.4	Evaluation of potential solutions

##### 5.1.3.4.1	Evaluation of potential solutions #1 and #1b

Solutions #1 and #1b allow fulfilling the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03 and PREQ-FS_MExpo-Disc-05, providing controllable granularity on which published MnS information can be consumed by an external MnS consumer.

However, there are gaps with the proposed solutions.

Firstly, it is noteworthy that PREQ-FS_MExpo_Disc-04 cannot be fulfilled with the current MSAC solution. The “operations” attribute only supports CRUD operations, but no notifications. Before working on fulfilling PREQ-FS_MExpo_Disc-04, there are open questions that need to be answered first:

-    Notification types: it should be clarified whether the intention is to set the visibility over certain notification types, and if so, which notification types are eligible. Are Configuration Management related notifications (e.g., notifyMOICreation, notifyMOIAttributeValueChange, notifyFileReady) included? Are Fault Management related notifications (e.g., notifyNewAlarm, notifyAckStateChanged, notifyClearedAlarm) included? Are Threshold Crossing notifications included?

-    Notification content: it should be clarified whether it is the intention to set the visibility over notification content; accessing to notification content can reveal secret.

-    “dataNodeSelector” attribute in AccessRule class: it should be clarified the role of this attribute with notification support: In every notification, there is parameter “objectInstance” in the notification header. Is “dataNodeSelector” attribute evaluated against this parameter “objectInstance”?

Secondly, it is not clear what is the default system behavior when the operator does not configure any rule, and therefore any policy. For example, what happens when a node (specified by “dataNodeSelector”) is not covered by any rule?. Should the system allow or deny visibility over such a node? The default system behavior for this situation is not described in this TR nor in the TS 28.319 [29].

Thirdly, the potential solutions propose to use AccessRule and the Role classes defined in clauses 7.3.3 and 7.3.2 of TS 28.319 [29], respectively, to define the discovery information at the CCF authorization server. However, the proposed solutions do not go into detail to describe how the proposed solutions will be used to restrict the discovery of service APIs at the CCF.

Also, the use of the AccessRule class to define the discovery information at the CCF implies that the CCF authorization server should be able to resolve Jex Expressions (since the dataNodeSelector attribute of AccessRule class is expressed using Jex (see TS 32.161 [X])). This raises the following concerns:

- Is it practical to put a requirement on the CCF authorization server to understand and resolve Jex expressions to configure the discovery information?

- If the use of the AccessRule class to define the discovery information at the CCF is the preferred option by the 3GPP management system, then there is a need to analyse and document how this would work at the CCF authorization server.

End of Changes