3GPP TSG-SA5 Meeting #157	S5-246044

Hyderabad, India, 14 - 18 October 2024

# Source:	China Mobile

# Title:	Add the solution of 5G LAN topology visualization and user plane traffic analysis for TR28.915

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP draft TR 28.915: “Management and orchestration; Study on management aspects of Network Digital Twin v0.1.0”.

[2]	SP-231727 "New Study on management aspects of Network Digital Twin"

3	Rationale

The potential solution for use case 8 is still empty. This contribution proposes to add the potential solution and evaluation for the use case 8 and take the 5G LAN as an example for the visualization of network topology and traffic.

4	Detailed proposal

First Changes

5.8	Use case8: Visualization of network topology and traffic

5.8.1	Description

The visualization of the network is helpful in some management capabilities for the network operators.

EXAMPLE 1:	The visualization of network shows the network topology and information of each contained NFs including the overall performance statistics information (including, the number of simultaneous UEs and PDU sessions), this helps to knowledge the real time status and performance related information of the network. E.g., 5G LAN-type service defined in 5GS enables Ethernet or IP unicast, multicast or broadcast communication via local switch, N6-based or N19-based forwarding methods. Several UPFs may construct the LAN topology via N19 interface. The 5G LAN topology visualization, will be helpful in management capabilities for the network operation.

EXAMPLE 2:	Based on the visualization of user or signaling traffic, it helps to quickly detect abnormal traffic and root cause of a service failure.

By collecting and the synchronizing real time data from the mobile network, the management system can create a network digital twin. The created network digital twin can provide the capability of network visualization, which not only shows the topology of the network, but also displays the simulation image of the real network which includes both  network elements (e.g. 5GC NFs or gNB) information and infrastructure resource information.

In case of 5G LAN topology visualization, the management system can use NDT to simulate a digital twin of 5G LAN group by collecting and the synchronizing real time topology data from the mobile network.

-	When the 5G VN group members' PDU Sessions are served by different PSA UPFs and N19-based forwarding is applied, after the SMF creates a group-level N4 Session with each involved UPF to enable N19-based forwarding and N6-based forwarding, the NDT can simulate the UPF connection topology for this specific 5G VN group.

-	When utilizing Ethernet-based 5G LANs, once the UPF detects devices behind UE, the NDT can simulate the connection status of these devices according to the UPF detected information, while also reflecting the topological connection with the UEs. With such capability, when a new device behind UE connected/removed from the UE, and even a device moved from one UE to another, the NDT can simulate the topological connection change.

The consumer could request the NDT for the supported capability of visualization of network topology and traffic and may further receive the detail information (e.g. the location information) for the consumer to obtain the visualization information of the network.

5.8.2	Potential requirements

REQ-VISUAL_NDT-01: NDT should have a capability to indicate its support visualization of network topology and traffic (e.g., 5G LAN network topology and traffic, including both UPF connection topology and the UE/device behind UE connection topology).

REQ-VISUAL_NDT-02: NDT should have a capability to report the visualization information of the network (e.g., 5G LAN network topology and traffic).

5.8.3	Potential solutions

This solution addresses the following issues of use case 8. When the visualization of network topology and traffic is required by MnS consumer, NDT is used for topology visualization and traffic simulation and analysis. E.g., 5G LAN topology visualization and user plan traffic simulation. The NDT utilizes network related information to generate a report of topology visualization and traffic simulation results with the following approach.

Figure 5.8.3.1-1: NDT for visualization of network topology and traffic

1.	The MnS consumer sends a request to NDT as the MnS provider for visualization of network topology and the traffic modelling, including the simulated network objects (e.g. 5G VN group) and the required actions on the simulated network objects (e.g. topology visualization, simulation information, user plane traffic modelling).

2.	The NDT as the MnS provider provides a response to MnS consumer indicating the status of the request based on a feasibility check (success or failure).

3.	The NDT as the MnS consumer synchronizes the network objects related information from MnS providers for network topology visualization and traffic modelling. In case of 5G VN group, the 5G VN group identifier, 5G VN group topology, N19 tunnel information, device behind UE status, device information may be included.

4.	The NDT executes the network topology simulation and visualization and optionally the traffic modelling and generates the report.

5.	The NDT as the MnS provider sends the notification of report including the results to MnS consumer. The report can include:

-	Visible network topology: report the visualization information.

-	traffic modelling: Use of network simulation to simulate the traffic model.

5.8.4	Evaluation of potential solutions

The solution described in clause 5.8.3 provides the NRM extension needed for the NDT to provide the network topology and traffic visualization modelling of network behaviour. The solution allows NDT MnS consumer to request the NDT to do the network simulation, visualization and traffic modelling. Therefore, the normative work on NDT support to visualization of network topology and traffic should progress following the outline in solution in clause in clause 5.8.3. To support the topology of visibility, the existing SMF IOC and the corresponding KPIs and performance measurements reported by UPF can be utilized.

End of Changes