3GPP TSG-SA5 Meeting #157	S5-246175

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.318 | CR | 0011 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.318 Implement readonly attributes for openAPI SS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-09-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The property “isWritable=False” for several attributes are described in Stage 2 NRM definition, however, it is missing in open API Solution Set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the readOnly attributes in open API Solution Set to align with stage2 according to the endorsed DP S5-244773 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The property “isWritable=False” is not implemented in openAPI Solution Set. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | The following YAML files are normatively stored in 3GPP forge: - TS28318_DsoNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1368 at commit a2273144e49d6480af672ca94274aa71005ad66e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1368 at commit a2273144e49d6480af672ca94274aa71005ad66e

*** START OF CHANGE 1 ***

*** OpenAPI/TS28318_DsoNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP NRM

version: 18.1.0

description: >-

OAS 3.0.1 specification of the OutageAndRecoveryInfo NRM

© 2024, 3GPP Organizational OutageAndRecoveryInfo Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.318; OutageAndRecoveryInfo NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.318/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EnergyServiceLocation:

type: object

properties:

CGI:

type: string

energySupplyId:

type: string

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-ncO-DsoNrm:

type: object

properties:

OutageAndRecoveryInfo:

$ref: '#/components/schemas/OutageAndRecoveryInfo-Multiple'

DsoThresholdMonitor:

$ref: '#/components/schemas/DsoThresholdMonitor-Multiple'

OutageAndRecoveryInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

outageStartTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

expectedOutageEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRapidInterventionTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRapidInterventionDuration:

type: integer

mnoInterventionTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

mnoInterventionDuration:

type: integer

readOnly: true

mnoMaxServiceDuration:

type: integer

readOnly: true

affectedArea:

$ref: '#/components/schemas/EnergyServiceLocation'

isAffectedAreaPriority:

type: boolean

actualOutageEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

DsoThresholdMonitor-Single:

allOf:

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Single'

- type: object

properties:

targetThresholdLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

OutageAndRecoveryInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/OutageAndRecoveryInfo-Single'

DsoThresholdMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/DsoThresholdMonitor-Single'

#--------------------------------- Definition ------------------------------------

resources-DSORecovery:

oneOf:

- $ref: '#/components/schemas/OutageAndRecoveryInfo-Single'

- $ref: '#/components/schemas/DsoThresholdMonitor-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***