3GPP TSG-SA5 Meeting #157	S5-246252

Hyderabad, India, 14 - 18 October 2024

# Source:	NEC, Intel

# Title:	pCR TR 28.858 add clarifications on ML pre-training use case

# Document for:	Approval

Agenda Item:	6.19.1

# 1	Decision/action requested

The group is asked to discuss and approve the attached proposal.

# 2	References

[1]	3GPP TR 28.858 v0.2.0; Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2

# 3	Rationale

# Current description of the ML pre-training [1] lacks details and is very abstract and can benefit from some further clarifications. In addition, some relevant corrections are also introduced where necessary.

# 4				Detailed proposal

Add the following changes to TR 28.858 [1]:

1st change

### 5.1.2 	ML pre-training

#### 5.1.2.1 	Description

Pre-training refers to training an ML model using a dataset that supports multiple types of inference.

The intent is to develop an ML model with foundational knowledge that support multiple types of inference across multiple domains, rather than focusing on a single, narrow task.

For example, an ML model could be pre-trained using dataset from SLS analysis capability group covering type of inference including ServiceExperienceAnalysis, NetworkSliceThroughputAnalysis, NetworkSliceTrafficAnalysis, NetworkSliceLoadAnalysis and E2ElatencyAnalysis [see TS 28.104 [3].

The pre-training is not intended to support a specific aIMLInferenceName but rather leveraging on commonality among number of use cases.

The ML model training process defined in TS 28.105 [2] also covers pre-training.

#### 5.1.2.2	Use cases

##### 5.1.2.2.1		Consumer-requested and producer-initiated ML pre-training

The pre-training process adopts the common procedure of consumer-requested and producer-initiated ML model training, defined already in TS 28.105 [2]. However the information contained in the training request and the resulting trained ML model differs. For example, instead of targeting a single, specific use case or inference task, the request for pre-training is broader, typically specifying datasets and objectives that are more generalized.

#### 5.1.2.3	Potential requirements

Editor’s note:  It is for further discussion if the existing relevant requirements in TS 28.105 [2] Table 6.2b.3 also applies for pre-training.

#### 5.1.2.4	Possible solutions

##### 5.1.2.4.1	Possible solution #1

The exsting MLTrainingRequest IOC and MLModel IOC can be enhanced to support pre-traning.

Enhancement on MLTraingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML pre-training, it is proposed to introduce following attributes into MLTrainingRequest IOC:

- preTrainingIndication, indicates that the request model is a Pre-training Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- potentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

Enhancement on MLModel IOC:  Since the pre-training is not aimed to support a specific inference capability but focuses on commonality in use cases, it’s proposed to introduce following attributes:

- preTrainingModelIndication, indicates the ML Model is a Pre-training Model. Besides, the support qualifier of “aIMLInferenceName” should be changed to CM, which means this attribute should be present when the mLModel MOI represents a Pre-training Model.

- potentialInferenceScope. Definition is documented above.

End of changes