3GPP TSG-SA5 Meeting #157	S5-245221

Hyderabad, India, 14 - 18 October 2024

# Source:	AsiaInfo

# Title:	pCR TR 28.866 Add solution evaluation and conclusion for Edge computing performance analytics UC

# Document for:	Approval

Agenda Item:	16.9.2

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.866 V1.0.1 Study on Management Data Analytics (MDA) – Phase 3

3	Rationale

This provides the solution evaluation and conclusion for Edge computing performance analytics UC.

4	Detailed proposal

It proposes to make the following changes to TR 28.866.

First Change

5.2	End-to-End performance analytics including Edge computing domain

5.2.1	Use case 1: Edge computing performance analytics

5.2.1.1	Description

For edge applications such as remote control and automation vehicles, the end-to-end performance (e.g. latency) to an end user is contributed by both the network side and the Edge Computing side. We can guarantee the end-to-end latency between UE and EAS to satisfy the consumer requirements for guarantee user service experience. In R18, the MDA capability include E2E latency analysis, including CN latency analysis and RAN latency analysis, but not include latency between UE and EAS.

It is desirable that the end-to-end latency can be predicted by MDA. MDA consumer sends the request for end-to-end latency analytics to MDA producer, MDA producer correlates and analyses multi-fold data (such as EDN NF (e.g. EAS, EES) performance measurements, 5GC NF measurement and alarm related to edge computing performance, together with the geographical and configuration data of edge computing). The MDAS producer provides the analytics report that include predicting end-to-end latency for edge application. The MDAS producer may provide the recommendations that may be for example to adjust the configuration data of EDN NF.

Note: if the UPF and DN are not co-located, the input of MDA can require consultation with SA6.

5.2.1.2	Potential requirements

REQ-EDGE-CON-1:	MDA capability for edge computing performance analytics should provide the prediction related to E2E latency between UE and EAS.

5.2.1.3	Potential solutions

The solution is to introduce the enabling data for edge computing performance analytics, the enable data are provided in table 5.2.1.3-1

Table 5.2.1.3-1: Enabling data for edge computing performance analytics

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Packet Delay on air-interface | Average delay DL air-interface(clause 5.1.1.1.1 TS 28.552 [5]);(Average delay UL on over-the-air interface(clause 5.1.1.1.3 TS 28.552 [5]) |
|  | Round-trip GTP Data Packet Delay on N3 interface | Round-trip delay on a N3 interface on PSA UPF(clause 5.4.1.9 TS 28.552 [5]) |
|  | GTP packets delay in UPF | GTP packets delay within the PSA UPF(clause 5.4.5 TS 28.552[5]). |
|  | Round-trip Packet Delay | Round-trip packet delay between PSA UPF and NG-RAN (clause 5.4.8 TS 28.552 [5]). |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (clause 6.3.1.2 in TS 28.554 [6]); Integrated uplink delay in RAN (clause 6.3.1.7 in TS 28.554 [6]). |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [7]). |
| EAS Service location | It defines the location where the EAS service should be available | requiredEASservingLocation（see TS see clause 7.3.3.6 in TS 28.538 [8] |

The solution is also to introduce a data type for edge computing performance analytics, called edgeComputingPerformanceAnalysis. The data type can be the contents of the analytics report that representing the MDA for edge computing performance correlation analytics

Attributes may configure the analytics context in MDARequest. The context may include attributes.

- An attribute may indicate the EAS identifier that identifies a particular application (e.g. Video, Game, … etc.).

- An attribute may indicate the latency of UE and EAS.

5.2.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

Second Change

6	Conclusions

6.2 End-to-End performance analytics including Edge computing domain

The use case, requirements and solution for Use case: Edge computing performance analytics in clause 5.2.1. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics for Edge computing performance analytics, which can be used to predict related to E2E latency between UE and EAS. The detailed solution is described in clause 5.2.1.3.

The use case, requirements and solution for Use case :  Edge application deployment location analytics in clause 5.2.2. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics for edge application deployment location, which can be used to recommend the deployment location that meets user experience.

The detailed solution is described in clause 5.2.2.3.

End changes