3GPP TSG-SA5 Meeting #157	S5-245328

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	DP on using plan management to support time-based configuration for NTN scenarios

# Document for:	Endorsement

Agenda Item:	6.19.15

1	Decision/action requested

The group is asked to discuss and endorsement.

2	References

[1]	3GPP TR 28.872: "Study on Management of planned configurations"

[2]	3GPP TS 28.874: "Study on Management Aspects of NTN Phase 2"

3	Rationale

3.1	Background

The time-based pre-configuration is widely to support most of the use cases documented in TR 28.874, following are the summary on the use case, requirements and potential solutions:

| Use case | Requirements | Solution |
| --- | --- | --- |
| 5.1.1 Use case #1: Connections between RAN node on-board satellite and CN (regenerative mode) | REQ-NTN-REGCON-1：time based configuration for the connections between RAN nodes on-board satellite and 5GC on an unreliable management interface. | Potential solution #<1>: add time window for attribute “remoteAddress” for EP_RP_EPS IOC, EP_NgC IOC and EP_N2 IOC. Potential solution #<2>: add time window for EP_RP_EPS IOC, EP_NgC IOC and EP_N2 IOC. |
|  | REQ-NTN-REGCON-2：time based configuration for NRCellCU and NRCellDU configurations on RAN nodes on-board satellite due to NTN (quasi-earth-fixed) cells, on an unreliable management interface | Potential solution #<3.1>: The NRCellCU IOC is modified so that all current attributes in NRCellCU IOC are replaced by one single attribute NRCellCUInfoList, which is a list with entries consisting of a timeWindow, and NRCellCUInfo with the same attributes as the current attributes in NRCellCU. Same update for NRCellDU Potential solution #<3.2>: add time window for NRCellCU IOC and NR CellDU IOC. |
| 5.1.2 Use case #2: Associations between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground (transparent mode) | REQ-NTN-TRANSCON-1：time based configuration for the associations between SectorEquipmentFunction on-board satellite and NRSectorCarrier in the RAN nodes (gNB/eNB) on ground on an unreliable management interface. | Potential solution #<1>: add time window for attribute “relatedSector” for EUtranGenericCell IOC, attribute “sectorEquipmentFunctionRef” for NRSectorCarrier IOC, attribute “theCellList” and “theNRSectorCarrierList” for SectorEquipmentFunction IOC Potential solution #<2>: add time window for NRSectorCarrier IOC and SectorEquipmentFunction IOC |
| 5.2.1 Use case #1: NTN neighbour cell management | REQ-NTN-FUN-0X: time based configuration for the NTN neighbour cells | Potential solution #<1>: add time window for NRCellRelation IOC |
| 5.2.2 Use case#2: NTN Tracking area management | REQ-NTN-FUN-01: time based configuration TACs for NTN earth-moving cells | Potential solution #<1>: add time window for attribute "nTNTAClist" for NRCellDU IOC |
| 5.4.1 Use case #1: UE-Satellite-UE Communication via UPFs on-board the satellites | REQ- UE-SAT-UE-1: 3GPP management service producer should have the capability to configure the associations between UPF on-board satellite and SMF on the ground. | Potential solution #<2>: adds new attributes representing the connection availability duration information in EP_N4 IOC |
| 5.5.1 Use case #1: Connectivity between non-terrestrial network node and security gateway | REQ-NTN-PnC-1: time based configuration for secure communications channel association between RAN nodes on-board satellite and SEG | Potential solution #<1>: add time window for IP Autoconfiguration |

Following are the observations based on above summary for use case, requirements and potential solutions for time-based configuration.

Observation#1: Above NTN configuration is pre-configuration based on the planned flight path for the NTN satellite Node. All network configuration parameters are the parameters in the existing NR or 5GC NRM, no additional network configuration parameters introduced.

Observation#2: TimeWindow attribute introduced individually for a bulk of IOCs for existing NR or 5GC NRM as optional or condition mandatory attributes. For some potential solutions (e.g. Potential solution #<1> for 5.1.1 	Use case #1), time window is introduced for each item of list for one single IOC.

Issues#1: The update of NRM in several solutions (e.g. Potential solution #<1> for 5.1.1 	Use case #1, Potential solution #<1> for 5.1.2 	Use case #2: Associations between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground (transparent mode)), will have backward compatible issue for non-NTN Node.

Issues#2: Maintain timeWindow attributes for a number of IOCs and attributes with single IOC individually will increase configuration complexity (e.g. MnS producer needs to check whether the configured conditions are fulfilled or not for each time-based attribute and IOC) and risk of inconsistent configurations (using gNB on the Satellites for example, how to ensure the same time window configured for different attributes and IOCs for the same gNB).

Potential configuration solution to address above issues to better support NTN configuration feature without impact on non-NTN configuration feature.

Management of planned configurations is described in TR 28.872 [1] to provide the capability to support creating, reading, updating, and deleting planned configurations for managed systems. A planned configuration may include only the configuration for the part of the managed system that shall be reconfigured. A planned configuration is represented by a data node tree. The data node tree is compliant to the existing NRM schema that also describes the data node tree of the corresponding current configuration. The values of the configuration data nodes in the data node tree are those values with which the managed system shall be reconfigured. Many management problems would benefit from the possibility of creating planned configurations that are not active yet and that can be manipulated without changing the current configuration of the managed system. When ready, the planned configurations can be activated. The capability of conditional activation of planned configurations also is described in TR 28.872 [1], which allow MnS consumer to specify when planned configurations are activated.

Based on observation#1, above NTN configuration is pre-configuration based on the planned flight path for the NTN satellite Node, which means a set of configurations is planed for each time window derived from the planned flight. The set of configurations planed for each time window for an NTN node or multiple NTN nodes can be modelled as one plan with the specified time window as condition.  Following is one example to illustrate different plans are created for one gNB for different time windows. The plan X for time window X only contain the configuration parameters which are different with those for time window X-1. This solution will have no impact on NR NRM and 5GC NRM.

Figure X Example to illustrate different plans are created for one gNB for different time windows

Following are the detailed procedures for using plan management to support time-based configuration for NTN scenarios.

Figure Y procedures for using plan management for time based NTN configuration

4	Detailed proposal

It proposes to use plan management solution described in TR 28.872 [1] for following time based NTN configuration related use case and requirements:

| Use case | Requirements |
| --- | --- |
| 5.1.1 Use case #1: Connections between RAN node on-board satellite and CN (regenerative mode) | REQ-NTN-REGCON-1：time based configuration for the connections between RAN nodes on-board satellite and 5GC on an unreliable management interface. |
|  | REQ-NTN-REGCON-2：time based configuration for NRCellCU and NRCellDU configurations on RAN nodes on-board satellite due to NTN (quasi-earth-fixed) cells, on an unreliable management interface |
| 5.1.2 Use case #2: Associations between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground (transparent mode) | REQ-NTN-TRANSCON-1：time based configuration for the associations between SectorEquipmentFunction on-board satellite and NRSectorCarrier in the RAN nodes (gNB/eNB) on ground on an unreliable management interface. |
| 5.2.1 Use case #1: NTN neighbour cell management | REQ-NTN-FUN-0X: time based configuration for the NTN neighbour cells |
| 5.2.2 Use case#2: NTN Tracking area management | REQ-NTN-FUN-01: time based configuration TACs for NTN earth-moving cells |
| 5.4.1 Use case #1: UE-Satellite-UE Communication via UPFs on-board the satellites | REQ- UE-SAT-UE-1: 3GPP management service producer should have the capability to configure the associations between UPF on-board satellite and SMF on the ground. |
| 5.5.1 Use case #1: Connectivity between non-terrestrial network node and security gateway | REQ-NTN-PnC-1: time based configuration for secure communications channel association between RAN nodes on-board satellite and SEG |