3GPP TSG SA WG5 Meeting #157 	S5-245334

Hyderabad, India, 14 - 18 October 2024

# Source:	CATT

# Title:	Add store and forward satellite operation charging solutions for Control Plane CIoT

# Document for:	Approval

# Agenda Item:	7.5.1

1	Decision

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.846: " Study on charging aspects of satellite access Phase 3 ".

3	Rationale

According to the conclusion of the SA2 SAT_Ph3_ARCH, the feature of S&F operation only supports the EPC in the Rel-19. Only delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) have been supported in the Rel-19.

This pCR proposes to add S&F operation charging solutions for Control Plane CIoT.

According to the endorsed CR of SA2 for the S&F operation, UE using satellite access with S&F operation indication in the initial attach or the service request procedure can only be known by MME. The P-GW can not get the information about the S&F, therefore, only offline charging triggered by MME can be used to charge the S&F operation of the Control Plane CIoT service.

4	Detailed proposal

The following changes are proposed to be incorporated into the new TR.

1st Modified Section

6.1.4.x	Solution #1.x: MME Charging Trigger Function (CTF) based solution for S&F operation charging with Control Plane CIoT

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the clause 5.3.4B of TS 23.401 [5], when the UE accesses the network with Control Plane CIoT EPS Optimisation, the data is stored in the MME-onboard. The UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach and service request procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach and service request procedure.

For the MME split architecture, the CHF may be deployed on the ground. The MME-ground together with the set of MME-onboard instances deployed in the set of satellites behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard instance is associated with a different Satellite ID identifier. The MME-ground reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT CP Optimizations;

-	Service Request via the MME onboard for the CIoT CP Optimizations.

For the whole EPC on-board architecture, the CHF may be deployed on the satellite. The MME on-board reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT CP Optimizations;

-	Service Request via the MME onboard for the CIoT CP Optimizations.

When the MME obtains the attach request or service request including S&F support indication from the UE and determines to support the S&F-based services, the MME (CTF) sends the Charging Data Request to OFCS for S&F operation charging, which has the following enhancement charging information:

-	RAT type

-	Satellite Access Indicator

-	Store and Forward indicator

-	Satellite information

-	S&F Monitoring list (i.e.Satellite IDs)

-	Store duration

-	Store data volume

The Figure 6.1.4.x-1 describes the high level charging procedure for MME (CTF) Converged charging for S&F operation charging of MME split architecture.

Figure 6.1.4.1-1: Attach procedure message flow for S&F operation charging with Control Plane CIoT

- If the PLMN broadcast that it is in S&F mode, the UE sends the attach request including S&F support indication.

- If the UE indicates that it supports S&F satellite operation, the on-board MME shall consider whether to accept the request.

3-16. Attach procedure as described in clause 5.3.2.1 of TS 23.401 [5].

17. After successful attach, the MME may provide the estimated UL delivery time, a S&F wait timer and S&F monitoring list to UE in the Attach Accept message as described in clause 5.3.2.1 of TS 23.401 [5].

17ch-a. The MME sends Charging Data Request [Event] including Satellite Access Indicator, Store and Forward indicator and S&F Monitoring list to OFCS for the UE's successful attach for the S&F-based services.

17ch-b. The CHF creates the CDR.

17ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MME.

The high level charging procedure for whole EPC on-board architecture is similar to above. The only difference is that all the NFs are on the satellite.

The MME-CDR shall be produced for S&F operation charging with Control Plane CIoT via the MME on-board and would be:

Table 6.1.4.x MME-CDR

| Field | Category | Description |
| --- | --- | --- |
| RAT type | OC | This field contains the RAT type for the satellite access. |
| Satellite Access Indicator | OC | This field holds the use of 4G Satellite Access. |
| Store and Forward indicator | OC | This field holds the use of Store and Forward Satelllite operation. |
| Satellite information | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| S&F Monitoring list (1..max) | OC | This field holds the satellite IDs that used for the Store and Forward Satelllite operation. |
| Store duration | OC | This field holds the storage duration of data on the satellite. |
| Store data volume | OC | This field stores the amount of data stored on the satellite. |

End of Modified Sections