3GPP TSG-SA5 Meeting #157	S5-245347

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei, Nokia，Deutsche Telekom

# Title:	DP on observations and issues for using of VNF generic OAM functions in SBMA

# Document for:	Endorsement

Agenda Item:	6.19.6

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TR 28.869: "Study on Cloud Aspects of Management and Orchestration v1.0.1"

[2]	3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[3]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification"

[4]	S5-153364 Guidelines for cooperation between 3GPP TSG SA WG5 and ETSI ISG NFV

3	Rationale

3.1 Background and observations

3.1.1 Guidelines for cooperation between 3GPP TSG SA WG5 and ETSI ISG NFV

Following cooperation principles are documented in S5-153364 Guidelines for cooperation between 3GPP TSG SA WG5 and ETSI ISG NFV [4]:

4	Cooperation principles

The MMN-VNF shall be managed with emphasis on cooperation, reaching compromises and achieving consensus.

The following cooperation principles shall be used:

- For NS LCM (clause 7.1.2 of [1]), ETSI owns, produces and maintains ETSI stage 1/2/3 specifications on Os-Ma-nfvo reference point (defined in [1]) and would consider 3GPP SA5 inputs on requirements.

- For VNF package management (clause 7.2.1 of [1]), VNF software image management (clause 7.2.2 of [1]) and VNF LCM (clause 7.2.4 of [1]), ETSI owns, produces and maintains ETSI stage 1/2/3 specifications on Ve-Vnfm-em reference point (defined in [1]) and would consider 3GPP SA5 inputs on requirements.

- For VR fault information (clause 7.3.5 of [1]) and PM information (clause 7.3.4 of [1]), ETSI owns, produces and maintains ETSI stage 1/2/3 specifications on Ve-Vnfm-em reference point (defined in [1]) and would consider 3GPP SA5 inputs on requirements.

- For FM, PM and CM of VNFs/PNFs corresponding to 3GPP-defined NEs, 3GPP owns, produces and maintains 3GPP stage 1/2/3 specifications on all interfaces and would consider ETSI IFA inputs on requirements.

- For FM, PM, CM and policy management of network services realized by VNFs/PNFs corresponding to 3GPP-defined NEs (i.e. Tenant Domain (see [2]) specific services), 3GPP owns, produces and maintains 3GPP stage 1/2/3 specifications on all interfaces and would consider ETSI IFA inputs on requirements.

- Any external communication or announcement (e.g. press releases, website) related to the MMN-VNF cooperation should be agreed by the Stakeholders.

Observation 1: 3GPP owns, produces and maintains 3GPP stage 1/2/3 specifications on all interfaces for FM, PM and CM of VNFs/PNFs corresponding to 3GPP-defined NEs.

3.1.2 Management architecture and procedures of mobile networks that include virtualized network functions

The following management architecture of mobile networks that include virtualized network functions is introduced in TS 28.500 [2] in Release 14 to clarify the relationship between 3GPP management architecture and NFV-MANO framework.

In this Management architecture,

-	NM plays one of the roles of OSS/BSS. It provides the functions for the management of mobile network which includes virtualized network functions. The NM supports FCAPS management functions of the mobile network (e.g. IMS, EPC) and 3GPP service (e.g. data service, voice service) and supports the lifecycle management of mobile networks.

-	EM/DM is responsible for FCAPS management functionality for a VNF on an application level and physical NE on a domain and element level. EM/DM participates in lifecycle management functionality for a VNF. This includes:

-	Request the lifecycle management operation for a VNF to VNFM.

-	Exchange of information regarding the VNF and Virtualized Resources associated with the VNF.

-	NFV-MANO is composed of NFV Orchestrator (NFVO), VNF Manager (VNFM) and VIM. The main functionalities of those are defined in ETSI GS NFV 006, which mainly focus on NS/VNF lifecycle management and virtualized resources management.

The management interactions with NFV MANO in the SBMA is described in TS 28.533 in clause 5.2 as follows

3GPP management system shall be capable to consume NFV MANO interface (e.g. Os-Ma-nfvo, Ve-Vnfm-em and Ve-Vnfm-vnf reference points).

Producer of management services can consume management interfaces provided by NFV MANO for following purposes:

-	Network service LCM.

-	VNF LCM, PM, FM, CM on resources supporting VNF.

Observation 2: 3GPP management system is responsible for VNF application specific parameters management and consumes management interfaces provided by NFV MANO on resources supporting VNF.

3.1.3 VNF generic OAM functions defined in ETSI NFV IFA 0049

The VNF generic OAM functions and other PaaS Services framework is introduced in ETSI NFV IFA 0049 Release 5 as follows:

Figure 4.2.2-1: VNF generic OAM functions and other PaaS Services framework

In ETSI NFV IFA 0049, it is specified that a "VNF generic OAM function" is a "PaaS Service" of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs.

Following are more detailed content abstracted from ETSI NFV IFA 049 clause 4 Overview of the VNF generic OAM functions and PaaS Services framework

VNF generic OAM functions and other PaaS Services can interact with any VNF type and support in a generic way operations related to VNF connectivity, configuration and monitoring.

The use of VNF generic OAM functions and other PaaS Services can relax the restriction that management actions like the configuration of different VNF types is coupled with dedicated management systems (i.e. EMs).

Example: VNF generic OAM functions and other PaaS Services can be used to perform EM related operations like VNF configuration and/or exposure of the Indicator and LCM Coordination interfaces according to ETSI GS NFV-IFA 008 [i.5] in a generic simplified way.

In more detail when it comes to VNF configuration aspects in NFV, VNF configuration is regarded to include two parts, as documented by clause 4.2 of ETSI GR NFV-EVE 022 [i.4]. The first part is about virtualisation-dependent items, and the second part of virtualisation-independent items. The latter are also referred to as "VNF's application configuration". In principle functionalities provided by EMs are targeting VNF's application configuration aspects and lack support for the virtualisation-dependent items. These tasks can be performed by VNF generic OAM functions or other PaaS Services.

Observation 3: ETSI NFV IFA describe that VNF generic OAM function and corresponding interfaces provide NF management functionalities (including configuration functionalities) for any VNF types, including virtualisation-dependent items (VNF non-application specific parameters) and virtualisation-independent items (VNF application specific parameters)

The Interactions between ETSI NFV PaaS Services, NFV-MANO and the 3GPP Service Based Management Architecture (SBMA) are described in Annex B in ETSI IFA 049 as follows:

The Service Based Management Architecture (SBMA) is defined by 3GPP in 3GPP TS 28.533 [i.19]. Like in the case of NFV-MANO, VNF generic OAM functions and other PaaS Services reside outside the SBMA management domain.

Figure B1.1-1 Interactions between ETSI NFV PaaS Services, NFV-MANO and the 3GPP Service Based Management Architecture (SBMA)

Observation 4: ETSI IFA 049 described that VNF generic OAM functions and other PaaS Services reside outside the SBMA management domain

3.1.4 Use of VNF generic OAM functions for cloud native NF management investigated in TR 28.869[1]

Following four use cases and corresponding requirements documented in TR 28.869[1] for using of VNF generic OAM functions for Cloud-native VNF management:

- 5.1.1	Use case #1: Cloud-native VNF configuration management

REQ-CVNF_CM-2 The reference point between 3GPP management system and external OAM entity should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of configuring cloud-native VNF instances.

For each investigated use case, the VNF generic OAM functions and corresponding interfaces are proposed as potential solution.

Observation 5: TR 28.869[1] proposed the requirement to enable 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of configuring cloud-native VNF instances. VNF generic OAM functions (i.e., VNF configuration manager function, network configuration manager function, and the configuration server) and corresponding configuration interfaces as the potential solutions to support this configuration requirement, including both virtualisation-dependent items (VNF non-application specific parameters) and virtualisation-independent items (VNF application specific parameters).

3.2 Issues and potential solutions

Issue: Based on above observations, the VNF generic OAM functions and the corresponding interfaces are overlapping with the 3GPP management function and corresponding MnS for NF management (i.e., the NF provisioning MnS), especially for the management of VNF application specific parameters (i.e., virtualisation-independent items).

To address above identified issue, it proposes to endorse the following guidelines for the use of VNF generic OAM functions and other PaaS Services for 3GPP NFs which contain virtualization or containerized parts:

- 3GPP management system is responsible for VNF application specific parameters (i.e. 3GPP service related) management for 3GPP NFs.

- The 3GPP MnSs (including the Provisioning MnS) for NF management is used for the management of any VNF types for 3GPP NFs on VNF application specific parameters.

- 3GPP management system can consume management interfaces provided by VNF generic OAM functions and other PaaS Services for VNF non-application specific parameters (i.e. non-3GPP service related).

Note: The MO attributes, which are defined by 3GPP, typically are VNF application specific parameters.

The clause 5.1 “Use of VNF generic OAM functions” in TR 28.869[1] needs to be updated to follow above guidelines.

Following is the proposed relation of 3GPP SBMA with ETSI VNF generic OAM functions.

4	Detailed proposal

It proposes to endorse the following guidelines for the use of VNF generic OAM function and other PaaS Services for 3GPP NFs which contain virtualization part or cloud part:

- 3GPP management system is responsible for VNF application specific parameters (i.e. 3GPP service related) management for 3GPP NFs.

- The 3GPP MnSs (including Provisioning MnS) for NF management is used for the management of any VNF types for 3GPP NFs on VNF application specific parameters.

- 3GPP management system can consume management interfaces provided by VNF generic OAM functions and other PaaS Services for VNF non-application specific parameters (i.e. non-3GPP service related).

Note: The MO attributes, which are defined by 3GPP, typically are VNF application specific parameters.

The clause 5.1 “Use of VNF generic OAM functions” in TR 28.869[1] needs to be updated to follow above guidelines.