3GPP TSG-SA5 Meeting #157	S5-245368

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Add evaluation of solution for emergency preparedness

# Document for:	Approval

Agenda Item:	6.19.5

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.915: “Study on management aspects of Network Digital Twin”.

# 3	Rationale

Add evaluation of solution for emergency preparedness.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.915 [1].

1st change

### 5.3.3 Potential solutions

#### 5.3.3.1 solution 1

Introduce an IOC for an NDT, which may be called NDT. This may be name contained in a subnetwork or managed function to respectively represent a standalone NDT and an NDT contained in another function, e.g. in a SON function.

- The consumer can configure on to the NDT instance the network scenario to be modelled. The scenario can include the scope to be considered for evaluating Emergency preparedness.

- introduce a data type and an attribute on the NDT of the scope to be modelled or simulated by the NDT instance. This may be called nDTSimulationScope.

The consumer can configure the parameters of the NDT instance, including the configurations indicating the actions of a natural disaster.

- Introduce a data type and an attribute on the NDT configuration plan . The datatype which may be called nDTConfigurationPlan indicates the parameter values to be applied by the NDT instance.

Note: the specific characteristics of Emergency preparedness can be added as an attribute of the nDTSimulationScope and nDTConfigurationPlan

The NDT can provide output to the MnS consumer, the output including values on PMs and KPIs of all the objects that have been modelled by the NDT instance. These include the values indicating the impact of the actions of a natural disaster.

- Introduce a data type and an attribute on the NDT to represent the output of the NDT instance. This may be called nDTOutput and will contain attributes similar to those of existing network objects like cells

Note: the specific characteristics of reports for Emergency preparedness can be added as an attribute of the nDTOutput

### 5.3.4 Evaluation of potential solutions

The solution described in clause 5.3.3 provides the NRM extension needed for the NDT to provide modelling of network behavior that supports emergency preparedness. The solution satisfies the requirements and is feasible to implement.

End of changes