3GPP TSG-SA5 Meeting #157	S5-245461

Hyderabad, India, 14 - 18 October 2024

# Source:	ZTE Corporation

# Title:	Rel-19 pCR TR 28.858 Add New Requirements and Possible Solution to the Reinforcement Learning Use Case

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.858: " Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2"

3	Rationale

When the RL process is performed in real network, the network performance degradations of unexpected actions selected by the RL agent should be considered and evaluated. To minimize the negative impact of RL process in the real network, AIML MnS Consumer should be allowed to specify fall back conditions for exploration in reinforcement learning.

This contribution proposes to add new requirement to RL use case.

4	Detailed proposal

Start of First change

5.1.7	Management of Reinforcement Learning

5.1.7.1	Description

Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions by taking actions in an environment to achieve a goal. The agent learns from the consequences of its actions, rather than from being explicitly taught and it selects its actions based on its past experiences (exploitation) and also by new choices (exploration), which is essentially a trial-and-error approach. In RL, an agent interacts with the environment in discrete time steps. At each time step, the agent receives the current state of the environment and selects an action. The environment, in turn, provides a reward and the new state. As illustrated in Figure 5.1.6.1, suppose RL is applied for coverage problem analysis use case, the RL agent actions are used to optimize the coverage problem and the RL environment can be the simulation environment. The actions can be changes to the values of network adjustable parameters (e.g. change the transmission power of the NR sector carrier, see TS 28.104 [3]). The states can be the network PMs/KPIs like RSRP distribution etc. The reward can be the score of an RL performance metric to evaluate the PMs/KPIs. The goal of the agent is to learn a policy, which tells it what action to take under what circumstances, that maximizes the sum of rewards. The main advantage of this approach is in the ability to automatically adapt to the characteristics of the environment, making it suitable approach for handling dynamic environments such as mobile networks.

Figure 5.1.6.1: Reinforcement Learning in Domain Management Function

5.1.7.2	Use cases

5.1.7.2.1	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution to the problem. This makes the RL-based approaches very interesting for applications in the mobile networks. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled environment that are not supposed to violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, e.g., to switch from RL-based solution to non-RL-based solution, to fall back to last discrete time step, and to terminate the RL process.

For RL management, the MnS consumer can query the ML Training MnS producer to discover if RL is supported. When RL is supported, a consumer may want to provide a scope (e.g., geographical area, time window) that can aid the producer to select/create the environment when performing RL. In the event RL is supported, the consumer may also want to state their preference for environment type for RL during training i.e. simulated environment or real network.

NOTE:	Support for both environment types can be considered optional in the RL training.

5.1.7.3	Potential Requirements

REQ-RL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.

REQ-RL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized MnS consumer to specify the preferred RL environment type.

REQ-RL_MGMT-03: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the preferred RL environment scope.

REQ-RL_MGMT-04: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify fall back conditions for exploration in reinforcement learning.

5.1.6.4	Possible solutions

5.1.6.4.1	Possible solution #1

This solution proposes to enhance the existing MLTrainingRequest IOC and MLTrainingReport IOC to allow the RL consumer to specify RL requirement including environment selection and fall back condition.

Enhancement on MLTrainingRequest IOC:  Introduce a new <<datatype>> “RLRequirement” to the MLTrainingRequest IOC, which may include following attributes:

- RLEnviromentType, represents preferred RL environment type. The allowed values may be “RealNetwork”, “SimulationEnviroment”:

- RLEnviromentScope, represents RL environment scope, which may be a geographical area.

- RLFallbackCondition, represents the condition to perform fall back exploration in RL, e.g., KPI or PM based condition. This attribute is present only when the RLEnviromentType is “RealNetwork”.

- RLTerminationCondition, represent the condition to terminate exploration in RL process. This attribute is present only when the RLEnviromentType is “RealNetwork”.

Enhancement on MLTrainingReport IOC:  Introduce a new <<datatype>> “RLReport” to the MLTrainingReport IOC, which may include following attributes:

- RLProcessState, represents current RL state. Allowed values may include Completed, Fallback, and Termination,

- RLFallbackReason, represents the reason of RL fallback in RL process. This attribute is only present when the RLProcessState is “FAllback”.

- RLTerminationReason, represents the reason of termination in RL process. This attribute is only present when the RLProcessState is “Termination”.

End of First change