3GPP TSG-SA5 Meeting #157	S5-245578

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Discussion Paper for correction on the non-blocking mode

# Document for:	Discussion

Agenda Item:	7.3

1	Decision/action requested

Discuss the correction on the non-blocking mode.

2	References

[1]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[2]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[3]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[4]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

3	Rationale

3.1			Background

In TS 32.290 [1], the non-blocking mode scenario is introduced for the 5G charging in clause 5.3.2 Charging scenarios. The Converged Charging service offers charging with quota management, including the support for both blocking mode and non-blocking mode. The non-blocking mode allow the service delivery before sending the request to the CHF for the service to be granted authorization to start.

In TS 23.503 [2], the "Service Data flow handling while requesting credit" in cluase 6.3 Policy and charging control rule is introduced for indicating whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request (Values: blocking or non-blocking). In TS 29.512 [3] clause 4.1.4.4.4, the attribute sdfHandl is defined in ChargingData to indicate the blocking or non-blocking mode.

3.2			Observations and Analysis

Whether non-blocking mode applies has an impact on how CHF grant quota/credit to a rating group. For instance,

-	When the non-blocking mode applies to the service data flow within this RG, CHF may refuse the request for quota due to insufficient account balance, which may lead to service termination. If the non-blocking mode remain unchanged for the service data flow, the service may be started and interrupted again and again. To avoid frequent service interruption, it may require CHF to be aware of the non-blocking mode and inform PCF of the situation.

-	The service using non-blocking mode are usually latency sensitive services which may cause serious impact if the service delivery stops due to failure in requesting for quota. However, in normal handling procedure, if the account balance is exhausted for a rating group, the CHF may refuse to grant quota to the request. In special cases, when non-blocking mode applies, it may require different handling procedure from CHF to avoid unexpected service interruption.

However, the current charging procedure and charging information defined in TS 32.290 [1] does not provide a clear guidance on how and when does CHF detect the non-blocking mode applied to a rating group.

Question 1: Which NF can report the non-blocking mode information to CHF?

Figure 5.3.2.3.2 in TS 32.290 [1] already provide a session-based charging procedure for non-blocking mode. Based on the existing procedure, the NF(CTF) with SCUR based charging scenario can report the non-blocking mode information to CHF, e.g. SMF. The SMF can receive the sdfHandl attribute indicating the blocking or non-blocking mode from PCF over N7 interface.

Question 2: How to report the non-blocking mode information in the charging data request?

As Figure 3.2-1 depicted, a service session may contain multiple service data flows which may be blocking or non-blocking mode and may be assigned to different RG by PCC rule, while CHF grant quota per RG basis.

For RG-1, since the first service data flow is non-blocking mode, SMF allows the service to start while sending the first quota request to CHF. In this case, the actual network resources may be consumed without quota supervision while SMF waiting for the response from CHF. This situation should be known by CHF in order to make quota grant decision considering the non-blocking situation, e.g. grant quota even the account balance is insufficient or inform PCF of the situation which is outside the scope of this discussion.

Based on the above analysis, it is proposed to add an attribute in the MUU level (i.e. the same level with the identifier of RG) to indicate whether the non-blocking mode is applied, which is only applicable for the first quota request of a rating group.

Figure 3.2-1: Relation between service data flows and rating group

4	Detailed proposal

The detailed proposals on the non-blocking mode for TS 32.290 [2] and TS 32.291 [3] are present in this meeting.