3GPP TSG-SA5 Meeting #157	S5-245691

# Hyderabad, India, 14 - 18 October 2024

# Source:	Samsung, NTT DOCOMO

# Title:	NDT Assisted Network Configuration Generation based on Performance Monitoring

# Document for:	Approval

Agenda Item:	6.19.5

# 1	Decision/action requested

Group is requested to discuss and agree this document`

# 2	References

None

# 3	Rationale

This proposes a new use case of NDT.

# 4	Detailed proposal

First Change

## 5.X Use case#A: NDT Assisted Network Configuration Generation based on Performance Monitoring

### 5.X.1 Description

Networks are configured through a combination of hardware and software components. The hardware components include base stations, antennas, and backhaul links, which are strategically placed to provide optimal coverage and capacity. The software components consist of network management systems, orchestrators, and controllers, which are responsible for automating and optimizing network operations. Additionally, 5G networks leverage virtualization and cloud technologies to enable flexible and scalable network deployments.

The abovementioned points needs operator involvement to carefully configure the network components. At present, operator has to rely on repetitive manual configurations to deliver KPI’s like high-speed, low-latency, and reliable connectivity for various use cases and applications. Manual configuration using trial and error can be time-consuming, error-prone, and limited in visibility. It requires repeated attempts to find the optimal configuration, consuming significant time and resources. Manual error can result in incorrect settings, misconfigurations, and network disruptions.   Incorporating adaptive network designs, that can respond to changing conditions can help maintain stable network performance. For example, using dynamic spectrum allocation algorithms that adjust resource allocation based on various performance measurements and external data (e.g weather patterns) can improve network resilience. Digital twin configuration under simulated environment can provide real-time visibility of the network performance, recommend required configuration to attain a particular level of performance, and reduces the risk of errors resulting in improved efficiency and scalability.

The required optimal configuration for the network may depend of various factors e.g load; the optimal configuration of the network would change based on the network load, time; the optimal configuration of the network would change based on the target time of the day, energy saving state; the optimal configuration of the network would change considering whether the simulated node is running in energy saving mode.

### 5.X.2 Potential requirements

REQ-NDTN_PM-1:  The NDT should support a capability enabling an MnS Consumer to decide/find the appropriate network configurations that are required to achieve particular network PM threshold(s).

REQ-NDTN_PM-1:  The NDT should support a capability enabling an MnS Consumer to provide the target performance threshold and the QoS that need to be achieved by the NDT simulation.

REQ-NDTN_PM-1:  The NDT should support a capability enabling an MnS Consumer to provide the load of network entities that are included in the NDT scope.

REQ-NDTN_PM-1:  The NDT should support a capability enabling an MnS Consumer to provide the time-of-the-day for network entities that are included in the NDT scope.

REQ-NDTN_PM-1:  The NDT should support a capability enabling an MnS Consumer to provide the energy saving sate of the network entities that are included in the NDT scope.

### 5.X.3 Potential solutions

The solution proposes to model the following information as part of NDT related NRMs.

- Simulation Scope: Specifies the target network nodes that need to simulated. This can be provided in terms of target ManagedFunction (e.g., base stations, subnetworks) or in terms of a location

- PM Threshold: This defines the target thresholds for PMs that should be achieved by the resultant network configurations.

- Target QoS: This defines the target quality of service (e.g., LTE OCI or NR OFI tables) that should be achieved by the resultant network configurations.

- Granularity: This defines the granularity to collect performance data. This relates to granularityPeriod attribute defined in TS 28.622.

- Target Load: This defines the target load of the simulated network nodes. This can be defined in terms of virtual resource usage or in an implementation specific way with a linear scale from 1 to 10.

- TargetTime: This defines the target time at which the performance of the nodes need to be collected. This can be defined in terms time of day. This relates to the Scheduler IOC defined in TS 28.622.

- Target Energy Saving: This defines whether the energy-saving features should be enabled or disabled for the network nodes that are being simulated.

The simulation shall be created by considering the abovementioned information. After simulating and evaluating multiple configurations, the NDT selects the optimal configuration and reports it back to the MnS Consumer.

The report includes:

- The final network configuration values recommendation (e.g., reconfiguring the gNB to improve DRB setup success rates or adjusting MIMO layers to enhance throughput).

- Predicted performance improvements (e.g lower latency, higher throughput, better energy efficiency).

- Achieved PM/KPI values - Achieved PM/KPI values that are collected and reported by NDT after the behaviour happens in NDT.

- Configuration Schedule -the timeframe in which specific configuration settings should be applied

Note: In order to generate the report, NDT may need amount of knowledge and reasoning capabilities. Other network automation function e.g MDA, CCL can be used for this purpose.

### 5.X.4 Evaluation of potential solutions

Only one solution is proposed satisfying all the requirement and is considered to be a feasible solution.

End of Changes