3GPP TSG-SA5 Meeting #157	S5-245706

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.421 | CR | 0144 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.421 Corrections of RRC reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The VisitedCellList is part of the UE History Information report, and does not to be handled explicitly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove references to VisitedCellList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary function clutters specification and leads to unnecessary implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 7.0, A.18.1, A.18.2, A.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.422 CR 0498 |  |  |
| Other comments: |  | This change involves the present CR, plus TS 32.422 CR 0498, TS 28.622 CR 0477 and TS 28.623 CR 0457. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 1	Scope

The present document describes the requirements for the management of Trace and the reporting of Trace data (including FDD mode and TDD mode) across UMTS, EPS or 5G networks as it refers to subscriber tracing (tracing of IMSI or Public User Identity) and equipment tracing (tracing of IMEI or IMEISV). Trace also includes the ability to trace all active calls in a cell or multiple cells (Cell Traffic Trace). The present document also includes the description of Service Level Tracing (tracing of a specific service). It defines the administration of Trace Session activation/deactivation by the management system, the network or User Equipment (UE) itself via signalling, the generation of Trace results in the Network Elements (NEs) and UE and the transfer of these results to one or more Operations Systems.

GSM Trace is outside of the scope of this specification (see [7]).

The present document also describes the requirements for the management of Minimization of Drive Tests (MDT) across UMTS networks, EPS networks or 5G networks and Radio Link Failure (RLF) and RRC Connection Establishment Failure (RCEF) reporting across EPS networks and 5G networks.

Furthermore, it describes requirements for Random Access (RA) Report, Successful Handover Report (SHR), Successful PSCell Change Report (SPR), and Mobility History Information (MHI) Report reporting for 5G networks.

The present document is built upon the basic Subscriber and UE Trace concept described in clause 4. 
The high-level requirements for Trace data, Trace Session activation/deactivation and Trace reporting are defined in clause 5. Clause 5 also contains an overview of use cases for Trace (the use cases are described in Annex A). Clause 6 defines the requirements for managing MDT. Clause 7 defines the requirements for managing RLF, RCEF, RA, SHR, SPR, and MHI reports. Trace control and configuration management are described in 3GPP TS 32.422 [2], and Trace data definition and management are described in 3GPP TS 32.423 [3].

The present document does not cover any Trace capability limitations within a NE (e.g. maximum number of simultaneous traced mobiles for a given NE) or any functionality related to these limitations (e.g. NE aborting a Trace Session due to resource limitations).

The objectives of the Trace specifications are:

a)	to provide the descriptions for a standard set of Trace and MDT data;

b)	to produce a common description of the management technique for Trace, MDT, RLF, RCEF, RA, SHR, SPR, and MHI administration and result reporting;

c)	to define a method for the reporting of Trace, MDT, RLF, RCEF, RA, SHR, SPR, and MHI results across the management interfaces.

The following is beyond the scope of the present document, and therefore the present document does not describe:

-	tracing non-Subscriber or non-UE related events within an NE;

-	tracing of all possible parties in a multi-party call (although multiple calls related to the IMSI specified in the Trace control and configuration parameters are traceable).

The definition of Trace and MDT data is intended to result in comparability of Trace and MDT data produced in a multi-vendor wireless UMTS, EPS and/or 5G network(s), for those Trace control and configuration parameters that can be standardised across all vendors' implementations.

Vendor specific extensions to the Trace control and configuration parameters and Trace and MDT data are discussed in 3GPP TS 32.422 [2] and 3GPP TS 32.423 [3].

All functions (trace, MDT etc.) specified in this specification support Network Sharing, with the following conditions:

-	It is accepted that the recorded information from the shared nodes is available to the Primary Operator. Recorded information that is collected in a non shared node or cell will only be available to the operator managing the non shared node or cell.

-	It is accepted that the recorded information from the shared network shall be delivered to the Participating Operator whose PLMN recording is requested, taking user consent into account. Operators must also agree on sharing the information, but how that agreement is done is outside the scope of this specification. The mapping of TCE IP addresses and TCE addresses must be coordinated among the operators that shares the network. How that coordination is done is outside the scope of this specification.

-	It is accepted that the inter-PLMN recorded information for Logged MDT from the non-shared nodes of Participating Operators may be available to the Primary Operator.

-	For signalling based activation, the operators that share a network must coordinate the TCE IP addresses and the TCE address mapping must be coordinated. How that coordination´ is done is outside the scope of this specification.

For UMTS and EPS the 3GPP Management reference model, 3GPP TS 32.101 [1] is followed.

For 5GS the 3GPP Services Based Management Architecture, 3GPP TS 28.533 [20] is followed.

Next change

# 7	Requirements for managing RRC reports

## 7.0	General

The term RRC reports is used for the RLF, RCEF, RA, SHR, SPR, and MHI reports for NG-RAN. A NG-RAN node can configure the UE so these reports are generated when specific triggers occur. The UE will notify the NG-RAN node when the reports are available, and the NG-RAN node can subsequently retrieve the reports.

Next change

## A.18	Use case #18 Collecting RRC reports for analytics

### A.18.1	Goal

Improving and optimising mobility management configuration in network nodes is important to reduce handover failures, poor coverage and low QoE. These improvements and optimisations can be done in the network nodes and also in the OAM system.

To aid this optimisation, RAN3 has defined a set of RRC [23] reports that can be generated by the UE and retrieved by a network node (for example a gNB). These reports consist of the RLF, RCEF, RA, SHR, SPR, and MHI reports for NR.

These reports can be used by network nodes themselves for use by, for example, a decentralized SON function. In rel-18, two of these reports (RLF and RCEF) can also be used by a consumer of Trace reports, for example a centralised SON function or an AI/ML function in the OAM system.

The goal is to make all reports (RLF, RCEF, RA, SHR, SPR, and MHI) available to MnS consumers, for example centralised SON functions, MDAS or NWDAF.

### A.18.2	Pre-conditions

The consumers of the data are operational.

The consumers have subscribed to one or many of the RLF, RCEF, RA, SHR, SPR, and MHI reports.

### A.18.3	Description/steps

1. The consumer(s) subscribe to Trace job(s), indicating one, many or all of the RLF, RCEF, RA, SHR, SPR, and MHI reports from network nodes.

2. When consumer(s) no longer are interested in the reports, the consumer(s) stops subscribing to the trace(s).

End of changes