3GPP TSG-SA5 Meeting #157 	S5-245724

Hyderabad, India, 14 - 18 October 2024

# Source:	ZTE Corporation

# Title:	Update requirement and add potential solution for Inactive RedCap UEs

# Document for:	Approval

Agenda Item:	6.19.17

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

# 3	Rationale

This contribution proposes to update requirement and add potential solution for Inactive RedCap UEs.

# 4	Detailed proposal

1st Change

## 5.3	Use case #3: Monitoring of RRC connection number for RedCap service

### 5.3.1	Description

Industry sensors scenario is a typical use case of RedCap technology. It's desirable in this scenario to connect different kinds of sensors to 5G network as mentioned in 3GPP TR 38.875 [6]. According to 3GPP TS 22.104 [8] and 3GPP TR 22.804 [9], this use case has a requirement on UE density to ensure enough number of devices enjoying communication service.

RRC connection number is an indispensable performance measurements for 5G NR, indicating the number of UEs connected to gNB simultaneously. It can reflect NR performances to operators.

The current measurements related to RRC connection number specified in 3GPP TS 28.552 [10] are all performed without recognizing the UE type. Therefore, it is difficult to accurately determine the number of RedCap UEs connected to NR, especially when different type of UEs co-exist. Consequently, it is important to investigate how to define measurement for RRC connection number of RedCap UEs. It will assist operators in understanding the resource load brought by RedCap and enable dynamic resource allocation.

The measurements of inactive RRC connection number for RedCap UEs can help operators better understand the status of RedCap UEs in RRC inactive mode. Monitoring of inactive RRC connection number for RedCap UEs can help operators use this information for dynamic frequency resource allocation or load balance purpose.

Monitoring of RRC connection resuming for RedCap UEs, can help operators optimize connection recovery process and improve user experience. The success or failure of a RRC connection resuming directly impacts the quality level for delivering the service by the networks and user experience.

### 5.3.2	Potential requirements

REQ-RedCap_Perf_RRCNum-1: The 3GPP management system should have capability to provide measurements related to RRC connection number for RedCap UEs in NR network.

REQ-RedCap-Perf-RRCNum-2: The 3GPP management system should have capability to provide measurements related to inactive RRC connection number for RedCap UEs in NR network.

REQ-RedCap-Perf-RRCNum-3: The 3GPP management system should have capability to provide measurements related to RRC connection resuming for RedCap UEs in NR network.

### 5.3.3	Potential Solutions

#### 5.3.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to RRC connection number defined in 3GPP TS 28.552 [10] for this use case.

Current measurements about RRC connection number includes Mean number of RRC Connections, Max number of RRC Connections, Mean number of stored inactive RRC Connections, Max number of stored inactive RRC Connections, Number of RRC connection resuming attempts, Successful RRC connection resuming etc. In order to achieve the requirement in clause 5.3.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

In radio access procedure in 5G system, UEs (no matter non-Redcap UEs or RedCap UEs) need to report to gNB about the NR UE Radio Access Capability Parameter using UE-NR-Capability IE, which is specified in clause 6.3.3 in 3GPP TS 38.331 [2]. RedCapParameters is part of UE-NR-Capability IE which can be delivered by the UE capability inquiry process.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequently, the measurements that performed after the inquiry of UE capability can be split into subcounters per UE type.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the filter needs to be enhanced to optionally split the measurement into subcounters to represent the RRC connection number for RedCap UEs.

Take the Mean number of RRC Connection as an example:

-	Introduce a new filter which can be named as NewFilter, then RRC.ConnMean can be optionally split as RRC.ConnMean.NewFilter to represent RRC connection number for RedCap UEs when the value of NewFilter is RedCap.

The subcounters filtered by the new filter with the value of RedCap can be used as measurements of RedCap RRC connection number.

### 5.3.4	Evaluation of potential solutions

The possible solution described in clause 5.3.3.1 enhances the existing measurements related to RRC connection number defined in 3GPP TS 28.552 [10]. A new filter is introduced, which can be optionally used to separate the measurements into subcounters to represent RedCap UEs number. The change is lightweight and and largely reuses existing measurements.

In summary, the solution described in clause 5.3.3.1 is feasible.

End of change