3GPP TSG-SA5 Meeting #157 	S5-245725

Hyderabad, India, 14 - 18 October 2024

# Source:	ZTE Corporation

# Title:	Rel-19 pCR TR28.876 Add conclusion and recommendation for resource load of RedCap service

# Document for:	Approval

Agenda Item:	6.19.17

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

# 3	Rationale

This contribution proposes to add conclusion and recommendation for resource load of RedCap service.

# 4	Detailed proposal

1st Change

## 5.6	Use case #6:  Performance evaluation on resource load of RedCap network service

### 5.6.1	Description

The radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) relative to eMBB (enhanced mobile broadband) devices due to their reduced capabilities and lower data rate requirements. PRBs related measurements may include total PRB usage, PDSCH PRB Usage, PUSCH PRB Usage, etc.

In 3GPP TS 28.552 [10], radio resource utilization metrics are used to evaluate the performance of the entire network, but the current approach combines the resource utilization of RedCap UEs and non-RedCap UEs, which may not accurately reflect the performance of the RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

Therefore, it is essential to investigate an approach for separately calculating the resource load of RedCap UEs and defining the resource load performance of networks that deliver RedCap service. This will ensure a more precise evaluation of the user experience for RedCap UEs, thus make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing for RedCap service.

Similarly, to distinguish RedCap UEs and non-RedCap UEs for metric of the number of the active UEs in each cell is helpful for operators to know how many DRBs are running with buffered data per cell for different kinds of UEs.

Through the metrics of radio resource utilization and active users of RedCap UEs and non-RedCap UEs, operators can better evaluate the overall consumption of network resources of RedCap UEs to facilitate adjustment of related network policies.

### 5.6.2	Potential requirements

REQ-RedCap-Perf-Resource-1: The 3GPP management system should have capability to provide measurements related to resource load of RedCap service.

### 5.6.3	Potential Solutions

#### 5.6.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to radio resource utilization defined in 3GPP TS 28.552 [10] for this use case.

Current PRBs related measurements are specified in clause 5.1.1.2 in 3GPP TS 28.552 [10], including DL Total PRB Usage, UL Total PRB Usage, PRB Usage per SSB, etc. In order to achieve the requirement in clause 5.6.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

Also, clause 5.1.1.23 in 3GPP TS 28.552 [10] Number of Active UEs can also be reused to reflect the load of RedCap service including:

-	Active UEs in the DL per cell.

-	Active UEs in the UL per cell.

-	Active UEs per cell.

-	Etc.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequently, the measurements that performed after the inquiry of UE capability can be split into subcounters per UE type.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the measurements can be optionally split into subcounters to represent PRB number for RedCap UEs.

The related measurements will create a subcounter which can be named as per UE type.

Take the DL Total PRB Usage as an example:

-	RRU.PrbDl.UeType

Where UeType indicates the type of UEs (e.g. RedCap UEs, non-RedCap UE, etc.) and UeType the value of RedCap can be used as measurements of PRB number for RedCap UEs.

### 5.6.4	Evaluation of potential solutions

The possible solution described in clause 5.6.3.1 enhances the existing measurements related to radio resource load in 3GPP TS 28.552 [10]. Measurements related to radio resource load of RedCap service will create a subcounter. The change is lightweight and largely reuses existing measurements.

In summary, the solution described in clause 5.6.3.1 is feasible.

2nd Change

## 6.X	Use case #6:  Performance evaluation on resource load of RedCap network service

This use case identifies that existing cell-level measurements related to radio resource load may not accurately reflect the performance of network delivering RedCap services, especially in co-existence scenarios. This is because these measurements combine all types of UEs together. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by filtering the RedCap UEs. Detailed description about this solution is shown in clause 5.6.3.

It is recommended to make some enhancements on radio resource load performance management related to RedCap service according to this solution in the future normative work.

End of change