3GPP TSG-SA5 Meeting #155 	S5-245782

# Hyderabadu, India, 14th Oct 2024 - 18th Oct 2024

# Source:	Ericsson

# Title:	Rel-19 pCR TR 28.871 Add evaluation of potential solutions (schema retrieval support)

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871  Study on Service Based Management Architecture enhancement phase 3

[2]	3GPP TS 28.532	 Generic management services

[3]	IETF RFC6022 	YANG Module for NETCONF Monitoring

# 3	Rationale

A key enabler to for data-driven producers, and consumers, is the ability to retrieve the supported schemas from the network.

This submission adds Evaluation and Recommendation for the use case.

# 4	Detailed proposal

First change

## 5.8	Schema retrieval enhancements

### 5.8.1	Description

This enhancement proposes methods to support retrieval of schema files/modules.

### 5.8.2	Potential requirements

REQ-Schema-retrieval -1: The MnS Producer shall support advertising its supported schema files/modules.

REQ-Schema-retrieval -2: The schema advertisement shall support files/modules located at any location specified by the MnS Producer.

REQ-Schema-retrieval -3: The MnS Producer shall support retrieval of all files/modules.

REQ-Schema-retrieval -4: The MnS Producer shall support retrieval of specific schema files/modules.

### 5.8.3	Potential solutions

This enhancement proposes methods to support retrieval of schema files/modules.

#### 5.8.3.1	Potential solution#1, Schema list at known location

This enhancement proposes methods to support advertisement and retrieval of schema files/modules using a model- based approach:

- Add objects to define each available schema. E.g. Schema IOC with properties:

  - Schema id (identifier, e.g. file/module name).

  - Schema version (based on versioning information in the file/module).

  - Schema format (Enum: yang, yaml).

  - Schema namespace (URI).

  - Schema location (file URI).

  - Supported feature list (if applicable).

  - Indicator of "implemented" vs. "imported" (if applicable).

- Advertisement of supported schema:

  - Define a specific location from which the list of supported schema is available. Similar to the ../schemas/.. tree defined in [3], clause 2.1.3 this would be a predefined location from which any consumer can query the list.

  - Each schema would be an entry in a 'list' of schema available at this location.

- Schema retrieval:

  - Consumer uses the information in the schema list entries to retrieve the schema file(s) of interest.

  - The actual method to retrieve them (e.g. HTTP, sftp) is defined by the solution and indicated in the <protocol> portion of the schema location URI.

#### 5.8.3.2	Potential solution#2 Schema lists as capabilities

This enhancement is similar to #1, but instead of placing the schema at a known location the schema files would be added to the capabilities advertised by specific MnS and/or managed entities. I.e. the schema file details would be data in the capabilities, not individual MOIs per schema file.

#### 5.8.3.3	Potential solution#3 Solution set specific solutions

This enhancement proposes different methods based on solution set:

- OpenAPI: predefine a resource end point from which the schema list can be queried. Define as a tree allowing single/multiple entries to be queried.

- NETCONF: implement per RFC6022 or RFC8525

### 5.8.4	Evaluation of potential solutions

Solution #1 is recommended as it provides a standardized set of meta data for each supported schema along with standardized mechanisms to store and query such information.  This solution could fulfil the proposed requirements in a consistent manner for all solution sets.

Solution#2 is not recommended.  It introduces an unnecessary coupling between the schema query and the MnS discovery.  This solution could fulfil the proposed requirements in a consistent manner for all solution sets.

Solution #3 is not recommended.  It adds no benefit from a standardization perspective.  It is already possible in current solution.  Whether or not the proposed requirements could be met, and how, would be solution set specific.