3GPP TSG-SA5 Meeting #158	S5-246488

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia, NTT DOCOMO

# Title:	Rel-19 pCR TR 28.858 Add solution and evaluation for ML explainability in inference

# Document for:	Approval

Agenda Item:	6.19.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 			3GPP TR 28.858 “Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2”.

# 3	Rationale

This pCR is to add a solution and an evaluation for the already agreed use case on ML explainability in inference during previous meetings.

# 4	Detailed proposal

### 5.5.6	ML explainability

#### 5.5.6.1		Description

The description for explainable ML is already described in clause 5.1.13.1.

#### 5.5.6.2	Use cases

##### 5.5.6.2.1	Local explanation in AI/ML inference

Once the ML model is trained to generate both outputs and corresponding explanations, it is deployed for inference. The generated local explanations by the AI/ML inference MnS producer may need to be reported to the AI/ML inference MnS consumer. These local explanations provide additional information that the MnS consumer can use to analyze the ML model’s potential impact on network performance. This analysis helps in making informed decisions regarding the deactivation of AI/ML inference or fallback to a previous version of the ML model. By considering network performance PM/KPIs, model performance, and local explanations, the MnS consumer can effectively identify the ML model contributing to network performance degradation when several ML models are active in the network.

#### 5.5.6.3		Potential requirements

REQ-ML-INF-EXP-1: The 3GPP management system should provide the capability for an authorized consumer to receive from the AI/ML inference MnS producer the generated local explanations for inference.

#### 5.5.6.4	Possible solutions

Introduce a new attribute inferenceLocalExplanation in the inferenceOutputs attribute (which is of type inferenceOutput) in the AIMLInferenceReport IoC. This information indicates the local explanation for the generated inference corresponding to a particular inferenceOutputId. The attribute is of type string.

#### 5.5.6.5	Evaluation

The solution described in clause 5.5.6.4 proposes the addition of a new attribute to the AIMLInferenceReport IoC to enable the MnS producer to provide local explanations corresponding to the generated inferences. Any advanced solutions, e.g., identifying a finite set of local explanations for each type of aIMLInferenceName needs further investigation in the normative phase.   Therefore, the solution described in clause 5.5.6.4 is a feasible solution to be developed further in the normative specifications.