3GPP TSG-SA5 Meeting #158	S5-246518

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Discovery UC - updates

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR aims to update the description and requirements for the discovery UC (use case #4).

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [1].

Begin Change

### 5.1.3	Use case #3: Configuring discovery policy information for an external MnS consumer

#### 5.1.3.1	Description

One or more API invokers may want access to published service APIs. For an API invoker to become a recognized user of the CAPIF, there are two stages:

- API invoker enrolment. In this first stage, a subscription for this API invoker is created, based on service agreement between the CAPIF provider and the API invoker (see clause 5.1 of TS 23.222 [5]). This subscription defines the list of published service APIs that the API invoker can discover and access later, together with SLA related to API invocations (e.g., quota, throttling). Following this subscription, the API provider domain generates an onboarding credential for the API invoker. The administrator sends this credential together with CCF details (address, root CA certificate) to the API invoker. As noted in clause 6.1 of TS 33.122 [14], these artefacts will be required by API invoker to initiate the onboarding stage.

- API invoker onboarding. In this second stage, the API invoker onboards itself at the CCF. To that end, the API invoker sends an onboarding request to the CCF (see clause 8.1 of TS 23.222 [5]) over CAPIF-1e interface. This request is sent using CAPIF_API_Invoker_Onboarding_API (see clause 5.5.2.2 of TS 29.222 [13]), with input information represented with APIInvokerEnrolmentDetails data type (see clause 8.4.2.2.3.1 in TS 29.222 [13]). Successful onboarding results in CCF provisioning API invoker profile (which includes identity for the API invoker and information required for the CCF to authenticate and authorize API invoker on subsequent CAPIF-1e interactions) and creating the list of service APIs that the CCF authorizes the API invoker to access (based on the subscription created during the API invoker enrolment).

Upon completion of these two stages, the API invoker is a recognized CAPIF user and can proceed with the discovery. For discovery, the API invoker sends a request to the CCF (see clause 8.7 of TS 23.222 [5]) using CAPIF_Discover_Service_API (see clause 8.8 of TS 29.222 [13]). This request includes the API invoker identifier and query information.

As noted in clause 8.7.3 of TS 23.222 [5], the discovery procedure requires the fulfilment of two pre-conditions. On the one hand, that the API invoker is onboarded and has received an API invoker identity; this is needed for the CCF to authenticate the API invoker on CAPIF-1e interface. On the other hand, that is CCF is configured with a discovery policy information; this is needed for CCF to perform filtering on service APIs matching the query information.

The discovery policy information can be used by CCF to restrict discovery to API categories (see clause 8.7.3 of TS 23.222 [5]). An API category is a name used to group service APIs by the service domain in which they are used, e.g. IoT, V2X (see clause 3.1 of TS 23.222 [5]). It is to the CAPIF administrator’s discretion to decide:

- How published service APIs are grouped into API categories, and the number and name of the API categories.

- How a discovery policy might look like. For example, there can be policies defined per API invoker (e.g., API-invoker-id1: APICategory-id2, APICategory-id4; API-Invoker-id2: APICategory-id1, API-Category-id2). Likewise policies can be associated to a group of API invokers, with the grouping criterium based on business (the API invokers are from the same 3rd party provider) or market segment (the API invokers belong to the same vertical industry), etc.

It is also worth noting that the discovery is performed at Service API level. The atomic information used both by API invoker (when including the query information in the discovery request) and the CCF (when applying the discovery policy information) is service API, not service API resource. Because of this, and because discovery only implies interaction over CAPIF-1e interface, there is no impact on 3GPP management system for the discovery use case.

#### 5.1.3.2	Potential requirements

There are no requirements impacting 3GPP management system for this use case.

#### 5.1.3.3	Potential solutions

##### There are no solutions available for this use case.

-

-

#### 5.1.3.4	Evaluation of potential solutions

-

There are no solution evaluations available for this use case.

End Change