3GPP TSG-SA5 Meeting #158	S5-246548

Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0143 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 add use case and requirements for RAN UE throughput analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 28.866 agree the use case of UE throughput analysis. It is proposed to add the use case and requirements of UE throughput anlaysis in TS 28.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use case and requirements of UE throughput anlaysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.X.1, 7.2.X.2, 7.2.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

### 7.2.X	RAN UE throughput analysis

##### 7.2.X.1	Description

This MDA capability is for analysis of RAN UE throughput to identify the traffic congestions.

##### 7.2.X.2	Use case

With the development of diverse communication services and the increasing number of connections, user data volume demanded by end users grows rapidly. As the user traffic increases, user experience deteriorates significantly. Therefore, UE throughput related measurements need to be analysed to identify the potential traffic congestion time and place by monitoring the network and provide the corresponding recommendation optimization that can be implemented to avoid potential traffic congestion.

Two types of traffic congestion as follows.

- Non-burst traffic congestion: Congestion that have heavy traffic which occurs at a fixed time or place every day, for example, during peak hours of daily morning, heavy traffic will resulting in a degraded user experience

- Burst traffic congestion:  user traffic increases sharply in a specific area, for example, in commercial activities and sports events, a large number of users gather for a period of time, resulting in rate experience deterioration.

The occurrence of such traffic congestion problem may seriously affect user experience and can be detected based on the UE throughput related measurements analysis.

Figure 7.2.X.2-1  RAN UE throughput analytics

Due to the complexity of 5G network and wireless environment, multiple types of performance deteriorate are related with high user traffic volume, e.g. the related performance measurements such as UE throughput, Radio resource utilization(e.g. PRB utilization), RRC connection number (e.g. number of RRC Connections) can be used to analyze, Once the preset threshold is reached, it may be considered that there is a traffic congestion problem as shown in the Figure 7.2.X.2-1. The user experience related performance should be analysed and identified to help to resolve improve the user experience. In this case, the consumer may need to be able to set the policy to help producer to analyze the congestion problems. The policy could be, for example, the threshold of the UE throughput related performance measurements (threshold and absolute deviation of the traffic in the figure).

The MDAS producer for radio network may also provide the recommendations to solve the UE throughput issues identified by the analytics congestion scenarios. The recommended actions may be for example to change the mobility parameters (e.g. MRO or LBO) or antenna RF parameters (e.g. CCO) such as downtilt and azimuth to where they are needed.

##### 7.2.X.3	Requirements

Table 7.2.X.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-UEThR_MDA-01 | MDA capability for UE throughput analysis shall include providing analytics for issues including Non-burst traffic congestion and Burst traffic congestion. | RAN UE throughput analytics |
| REQ-UEThR_MDA-02 | MDA capability for UE throughput analysis shall be able to provide the analytics output including traffic congestion type, traffic congestion severity level, the start time and end time of the traffic congestion, the geographical area and location where the traffic congestion has been happened, as well as the possible recommended actions to solve the potential traffic congestion problem. | RAN UE throughput analytics |

End of change