# 3GPP TSG SA WG5 Meeting #158	S5-246608

# Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei, Nokia

# Title:	Discussion Paper for failure handling mechanism

# Document for:	Discussion

Agenda Item:	7.3

1	Decision/action requested

Discuss the failure handling mechanism.

2	References

[1]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[2]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

3	Rationale

3.1			Background

The failure handling mechanism for charging system is defined in TS 32.290 [1] clause 5.5.1, including three scenarios:

-	CTF detected failure handling,

-	CHF detected failure handling,

-	CHF as NF consumer detected failure handling.

The third scenario was introduced in Release 18, to support the inter-CHF interaction architecture via N107 interface (WI: CHRACHF) or N108 interface (WI: B2B_CH/NETSLICE_CH_Ph2/TSN_CH). However, the mechanism defined for this scenario remains ambiguity, especially in terms of the relation with the first scenario.

3.2			Observations and Proposals

3.2.2	Failure handling mechanism (stage 2)

Observation #1:

According to TS 32.290 [1] clause 5.5.1, both CTF and consumer CHF use “application level failure handling” in case of request times out. However, the role of CTF and consumer CHF, in terms of the handling of failure and service, is not exactly the same.

-	When CTF towards CHF request times out (Figure 1-1), the CTF need to decide how to handle the ongoing service, i.e. whether to allow the service continue or not.

-	When consumer CHF towards producer CHF request times out (Figure 1-2), the consumer CHF need to decide how to handle the session with CTF, i.e. whether to allow the charging session continue or not.

Therefore, the dedicated “application level failure handling” for each scenario shall be clarified, otherwise the failure handling mechanism remains unclear and incomplete.

Furthermore, the application level failure handling mechanism should have a single action, and not covering multiple situations at the same time (e.g. RETRY_AND_TERMINATE), so it can be selected the way the application failure is handled.

Figure 1-1: A demonstration of CTF detected failure handling

Figure 1-2: A demonstration of CHF as NF consumer detected failure handling

Proposal #1:

The “application level failure handling” used by CTF contains three values, as defined in TS 32.291 [2] Table 6.1.6.3.11-1:

-	TERMINATE: the service shall only be granted for as long as there is a connection between NF consumer and the CHF.

-	CONTINUE: the NF consumer should re-send and continue the request to an alternative server in the case of transport temporary failures, provided that a failover procedure is supported in the CHF and the NF consumer, and that an alternative server is available.  Otherwise, the service SHOULD be granted, and Charging Information may be stored, even if charging data request can't be delivered.

- RETRY_AND_TERMINATE: the NF consumer should re-send the request to an alternative server in the case of transport temporary failures at the NF Consumer, provided that a failover procedure is supported in the CHF and NF consumer, and that an alternative server is available. Otherwise, the service should not be granted when the charging data request can't be delivered.

RETRY_AND_TERMINATE and CONTINUE both emphasize the NF Consumer shall attempt to re-sent in the case of transport temporary failures. Given that retry handling is a pre-condition as defined in TS 32.290 [1] clause 5.5.2, TERMINATE and RETRY_AND_TERMINATE could refer to the same result of failure handling. To keep backward compatibility, the differentiation between TERMINATE and RETRY_AND_TERMINATE can be clarified.

In addition, since consumer CHF cannot decide directly whether the service shall continue or not, the above three values cannot be used to specify how consumer CHF handles failure. The following values are proposed to define the possible options for consumer CHF to handle failure in case of request times out to producer CHF:

-	CH_RELEASE: the consumer CHF should respond an error code (i.e. 504 Gateway Timeout) that can instruct NF consumer (CTF) to release the charging session.

-	CH_CONTINUE: the consumer CHF should keep the session continue with NF consumer (CTF), which is implementation specific.

3.2.3	Failure handling attribute (stage 3)

Observation #2:

According to TS 32.290 [1] clause 5.5.1, the application level failure handling may be received from the CHF previously or may be locally configured. In case of received from the CHF previously, the values for failure handling is carried in the Invocation Result within the Charging Data Response, according to TS 32.290 [1] Table 7.2.

In the inter-CHF interaction, as demonstrated in Figure 1-3, the charging data response that is delivered from producer CHF to consumer CHF should indicate how consumer CHF handles failure, while the charging data response that is delivered from consumer CHF to NF consumer (CTF) should indicate how CTF handles failure.

Therefore, the same attribute “failure handling” can be re-used and expanded to support the delivery of failure handling in the inter-CHF interaction scenario.

Figure 1-3: A demonstration of failure handling delivered for inter-CHF interaction

Proposal #2:

The FailureHandling attribute defined in Table 6.1.6.3.11-1 in TS 32.291 [2] can be extended to cover the values for CHF as NF consumer detected failure handling, by indicating the applicability for the new values (i.e. INTER_CHF).

4	Detailed proposal

For proposal 1, a set of CRs (S5-246609/S5-246610) to clarify the CHF as NF consumer detected failure handling in stage 2 is proposed in this meeting for approval.

For proposal 2, a set of CRs (S5-246611/S5-246612) to clarify the CHF as NF consumer detected failure handling in stage 3 is proposed in this meeting for approval.