3GPP TSG-SA5 Meeting #158	S5-246668

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0272 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.312 Enhance stage 3 OpenAPI for multiplicity property in TS28312_IntentNrm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to agreed Rel19 TS32.160 CR0071 (S5-245469 TS32.160 clarify the Attribute property "multiplicity" for OpenAPI) ”Attributes of structured type with multiplicity greater than "1" are mapped to a name/value pair whose value is a JSON array, and the items are JSON objects, whose properties are described by the structured data type. The length of the array can be specified using the "minItems" and "maxItems" keywords, each of which must be a non-negative integer. If the low bound is 0, the keyword "minItems" can be omitted. If there is no upper (e.g, specified as *), the keyword "maxItems" shall not be used.” The OpenAPI stage 3 needs enhancement if the attributes of structured type with multiplicity greater than "1" and mapped to a name/value pair whose value is a JSON array. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the OpenAPI Stage 3 according to clause 6.1.11.2 in TS32.160, in TS28312_IntentNrm.yaml |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiplicity constrain not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (Normative Stage3 is in forge) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1501 at commit f732d26611be8b9c8ee52afd79a118feb0f2b85c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1501 at commit f732d26611be8b9c8ee52afd79a118feb0f2b85c

*** START OF CHANGE 1 ***

*** OpenAPI/TS28312_IntentNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Intent NRM

version: 18.5.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-IntentNrm:

type: object

properties:

IntentHandlingFunction:

$ref: '#/components/schemas/IntentHandlingFunction-Multiple'

#-------Definition of generic IOCs ----------#

Intent-Single:

description: >-

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

uniqueItems: true

minItems: 1

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/EdgeServiceSupportExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/5GCNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioServiceExpectation"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

intentContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions that should apply

for the entire intent even if there may be specific contexts defined for specific parts of the intent

intentAdminState:

type: string

enum:

- ACTIVATED

- DEACTIVATED

description: >-

It describes the intent administrative state.

This attribute is used when MnS consumer-suspension mechanism is supported

intentPriority:

type: integer

minimum: 1

maximum: 100

description: It expresses the priority of the stated intent within a MnS consumer.

intentPreemptionCapability:

type: boolean

observationPeriod:

type: integer

description: >-

It represents the observation period of the fulfilmentInfo for corresponding

ExpectationTargets, IntentExpectations and Intent.

intentReportReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentReport-Single:

description: It represents intent report information from MnS producer to MnS consumer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentFulfilmentReport:

$ref: '#/components/schemas/IntentFulfilmentReport'

intentConflictReports:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/IntentConflictReport'

intentFeasibilityCheckReport:

$ref: '#/components/schemas/IntentFeasibilityCheckReport'

lastUpdatedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

intentReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentHandlingFunction-Single:

description: >-

It represents the intent handling capabilities can be supported by a specific intent

handling function of MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentHandlingCapabilityList:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/IntentHandlingCapability'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

IntentReport:

$ref: '#/components/schemas/IntentReport-Multiple'

#-------Definition of generic IOCs ----------#

#-------Definition of the generic IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

It represents MnS consumer's requirements, goals and contexts given to a 3GPP system

type: object

properties:

expectationId:

type: string

description: A unique identifier of the intentExpectation within the intent.

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationTarget'

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

expectationContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

required:

- expectationId

#-------Definition of the generic IntentExpectation dataType ----------#

#-------Definition of the generic ExpectationObject dataType ----------#

ExpectationObject:

description: >-

It represents the Object to which the IntentExpectation should apply.

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SUBNETWORK  #value for Radio Network Expectation--#

- EDGE_SERVICE_SUPPORT  #value for Edge Service Support Expectation--#

- 5GC_SUBNETWORK  #value for 5GC Network Expectation--#

- Radio_Service  #value for Radio Service Expectation--#

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions to be

used as filter information to identify the object(s) to which a given intentExpectation should apply.

#-------Definition of the generic ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

Selectivity:

type: string

enum:

- ALL_OF

- ONE_OF

- ANY_OF

FulfilmentStatus:

type: string

readOnly: true

enum:

- FULFILLED

- NOT_FULFILLED

default: NOT_FULFILLED

description: It describes the current status of the intent fulfilment result.

NotFulfilledState:

type: string

readOnly: true

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

default: ACKNOWLEDGED

description: It describes the current progress of or the reason for not achieving fulfilment

for the intent, intentExpectation or expectationTarget.

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

FulfilmentInfo:

description: >-

This dataType represents the properties of a specific fulfilment information for an aspect of

the intent (i.e. either an expectation, a target or the whole intent).

type: object

properties:

fulfilmentStatus:

$ref: '#/components/schemas/FulfilmentStatus'

notFullfilledState:

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

type: array

uniqueItems: true

items:

type: string

readOnly: true

description: An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

description: It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Vendor extensions are allowed

Frequency:

description: >-

It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number)

and/or the frequency operating band used for a given direction (UL or DL) in FDD or

for both UL and DL directions in TDD.

type: object

properties:

arfcn:

type: integer

description: >-

This attribute shall be supported, when the frequency information represent RF reference frequency.

The allowed values for NR see TS 38.104 subclause 5.4.2.1; The allowed values for EUTRAN see TS 36.104 [X] subclause 5.7.3;

freqband:

type: string

description: >-

This attribute shall be supported, when the frequency information represent frequency operating band.

The allowed values for NR see TS 38.104 subclause 5.4.2.3; The allowed value for EUTRAN see TS 36.104 subclause 5.7.3

ValueRangeType:

oneOf:

- type: number

- type: string

- type: boolean

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

- $ref: '#/components/schemas/UEGroup'

- $ref: '#/components/schemas/Frequency'

UEGroup:

description: >-

It describes the UE Group, which is

represented by specific 5QI, specific S-NSSAI, or a specific combination

of S-NSSAI and 5QI

type: object

properties:

fiveQI:

type: integer

minimum: 0

maximum: 255

sNssai:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

#-------Definition of the generic dataType --------------#

#-------Definition of the generic ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type represents the target of the IntentExpectation that are required to be achieved.

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: '#/components/schemas/Condition'

targetValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

targetContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: It describes the list of constraints and conditions that should apply for a specific expectationTarget.

#-------Definition of the generic ExpectationTarget  dataType----------#

#-------Definition of the generic Context dataType----------------#

Context:

description: >-

This data type is the "Context" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: '#/components/schemas/Condition'

contextValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

#-------Definition of the generic Context dataType----------------#

#-------Definition of the generic IntentFulfilmentReport dataType----------------#

IntentFulfilmentReport:

description: >-

It includes the intentFulfilmentInfo and expectationFulfilmetResult.

This attribute shall be supported when intent fulfilment information is supported by IntentReport

type: object

properties:

intentFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

expectationFulfilmentResult:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationFulfilmentResult'

#-------Definition of the concrete IntentFulfilmentReport dataType----------------#

#-------Definition of the generic ExpectationFulfilmentResult dataType----------------#

ExpectationFulfilmentResult:

description: >-

It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation.

type: object

properties:

expectaitonId:

type: string

readOnly: true

expectationFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetFulfilmentResult:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/TargetFulfilmentResult'

#-------Definition of the concrete ExpectationFulfilmentResult dataType----------------#

#-------Definition of the generic TargetFulfilmentResult dataType----------------#

TargetFulfilmentResult:

description: >-

This data type includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget.

type: object

properties:

targetName:

type: string

readOnly: true

targetFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetAchievedValue:

type: number

description: >-

It describes the value that has been achieved for the expectation target at the time at which

the report is generated.

readOnly: true

#-------Definition of the concrete TargetFulfilmentResult dataType----------------#

#-------Definition of the generic IntentConflictReport dataType----------------#

IntentConflictReport:

description: >-

It represents the conflict information for the detected conflict

This attribute shall be supported when intent conflict information is supported by IntentReport

type: object

properties:

conflictId:

type: string

readOnly: true

conflictType:

type: string

readOnly: true

enum:

- INTENT_CONFLICT

- EXPECTATION_CONFLICT

- TARGET_CONFLICT

conflictingIntent:

description: >-

This will be present if the value of conflictType is INTENT_CONFLICT. It describes the DN of the conflicting intent

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

conflictingExpectation:

description: >-

This will be present if the value of conflictType is EXPECTATION_CONFLICT. It describes the expectationId of the conflicting IntentExpectation with an Intent

type: string

readOnly: true

conflictingTarget:

description: >-

This will be present if the value of conflictType is TARGET_CONFLICT. It describes the targetName of the conflicting ExpectationTarget with an IntentExpectation

type: string

readOnly: true

recommendedSolutions:

type: string

readOnly: true

enum:

- MODIFY

- DELETE

#-------Definition of the concrete IntentConflictReport dataType----------------#

#-------Definition of the generic IntentFeasibilityCheckReport dataType----------------#

IntentFeasibilityCheckReport:

description: >-

It represents the intent feasibility check information

This attribute shall be supported when intent feasibility check information information is supported by IntentReport

type: object

properties:

feasibilityCheckResult:

type: string

readOnly: true

enum:

- FEASIBLE

- INFEASIBLE

infeasibilityReason:

type: string

readOnly: true

description: An attribute which is used when feasibilityCheckResult is INFEASIBLE

#-------Definition of the concrete IntentFeasibilityCheckReport dataType----------------#

#-------Definition of the generic IntentHandlingCapability dataType----------------#

IntentHandlingCapability:

description: >-

It represents expectation object information and expectation target information

which can be supported by a specific intent handling function of MnS producer.

type: object

properties:

intentHandlingCapabilityId:

type: string

readOnly: true

supportedExpectationObjectType:

type: string

enum:

- RAN_SUBNETWORK

- EDGE_SERVICE_SUPPORT

- 5GC_SUBNETWORK

- Radio_Service

readOnly: true

description: It describes the expectation object type which can be supported by a specific intent handling function of MnS producer.

supportedExpectationTargetNames:

type: array

uniqueItems: true

minItems: 1

items:

type: string

readOnly: true

description: It describes the supported expectation targets for the supported expectation object type.

#-------Definition of the concrete IntentHandlingCapability dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

IntentReport-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentReport-Single'

IntentHandlingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentHandlingFunction-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/IntentHandlingFunction-Single'

- $ref: '#/components/schemas/Intent-Single'

- $ref: '#/components/schemas/IntentReport-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***