3GPP TSG-SA5 Meeting #158	S5-246785

Orlando, USA, 18 - 22 November 2024

# Source:	China Mobile

# Title:	pCR TR 28.858 add some description for Coordination between the ML capabilities

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

The group is asked to review and approve the proposal.

2	References

[1]			3GPP TR 28.858 v0.3.0; Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2

3	Rationale

The proposal is to add some description for Coordination between the ML in clause 5.5.1.

4	Detailed proposal

1st change

# 6	Conclusions and recommendations

5.5.1	Coordination between the ML capabilities

Editor’s Note: The content in this clause may need to be aligned with the latest version of TS28.105

5.5.1.1	Description

For ML in 5GC or RAN, the ML capabilities in 5GC or RAN may be needed to coordinate with 3GPP management analytics and possibly other aspects in order to improve the overall performance.

Typically, due to the type of the collected data used for model training/inference for 5GC or RAN, the performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of longer range and from a wide scope of RAN nodes/5GC, which consequently implies that the predictions calculated by the management system will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system predictions may lack insight of patterns related to specific node behaviour or finer granularity of time. Hence with coordination or alignment of the ML capability among 5GC/RAN and 3GPP management system, the overall performance may be improved.

To enable the coordination between the ML capabilities, the configuration (e.g. a triggering condition, i.e. when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and/or the related context) so the consumer can gain a better understanding regarding the coordination of ML capabilities.

5.5.1.2	Use cases

5.5.1.2.1	Alignment of the ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which may be modeled as "time series data" and the analytics of "time series data" is normally to learn the seasonality, trend, etc., patterns. Different types of seasonality patterns exist, e.g. daily, weekly, monthly, seasonally, annually, etc. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which consequently implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

To be noted, when coordinating with management ML capability, more than one ML capability may be involved. From a single ML capability perspective, one ML model may support one (or a set of) specific type of inference(s) capability. One ML inference function may employ two or more ML models to perform more sophisticated inference(s) instead of developing a new specific ML model. The involved ML models may work together in a coordinated manner, for which a coordination of ML capabilities would be needed.

From an ML consumer perspective, the required ML inference capability or ML models may be vague or unspecified. In such cases, the ML producer may need to provide an AI agent's function that converts the requirement into one or more ML models. For example, a requirement for load prediction for the 5GS from the ML consumer is analyzed by the AI agent provided by the ML producer, which then derives a solution by using the model in RAN and the model in 5GC in a coordinated way for prediction.

End of changes