3GPP TSG-SA5 Meeting #158	S5-246834

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson

# Title:	SCEF CDF/CGF based solution for S&F with CP data transfer

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TS 28.846 Study on charging aspects of satellite access phase 3

# 3	Rationale

Addition of a new solution for topic #1 based on SCEF CDF/CGF for S&F operation with CP data transfer.

# 4	Detailed proposal

This document proposes the following changes in TR 28.846 [1].

First change

#### 6.1.4.x	Solution #1.x: SCEF CDF/CGF based solution for S&F with CP data transfer

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the clause 5.3.4B of TS 23.401 [5], when the UE accesses the network with Control Plane CIoT EPS Optimisation, the data is stored in the MME onboard. The UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach and service request procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach and service request procedure.

Independent of MME split architecture or whole EPC onboard, the SCEF with CDF/CGF may be deployed on the ground. The high level of SCEF architecture is shown below:

Figure 6.1.4.x-1: MME split and whole EPC charging architecture

The SCEF reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	PDN connection creation/release via the SCEF for the Control Plane (CP) data transfer.

End of change