3GPP TSG-SA5 Meeting #158	S5-246895

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	DP on authorization of the external MnS consumer to consume management services at the CCF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss the proposal and approve.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] 3GPP TS 28.319: "Management and orchestration; Access Control for Management services".

# 3	Rationale

Clause 5.4.1 of TR 28.879 describes how to authorize an external MnS consumer to access the management service API without describing exactly how the authorization information for a specific external MnS consumer will be made available at the CCF. According to the CAPIF specifications, the CCF shall be enabled to generate the access token for an external MnS consumer (referred to as API invoker in CAPIF terms) to invoke service APIs. However, how the CCF will be enabled to generate access tokens for external MnS consumers at run-time is still not described.

This contribution aims to propose a potential solution enabling the CCF to provide authorization information to external MnS consumers to invoke service APIs.

## 3.1 Proposed solution for CCF authorization of an external MnS consumer to invoke service APIs

### 3.1.1 Using AccessRule to configure authorization information.

The potential solution assumes that the network operator has a portal with a list of services APIs and their corresponding metadata (e.g., the scope and allowed operations) providing the external MnS consumer with the option to choose what service APIs they would like to subscribe to. Such functionality is already provided by other SDOs, e.g., TM Forum.

After the external MnS consumer has identified the service APIs they would like to subscribe to, the CAPIF API invoker enrolment phase can start.  During the enrolment phase, the external MnS consumer provides a list of the service APIs they would like to consume on the 3GPP management system (which is MSEF acting as the API provider domain, in this case).  Accordingly, depending on the agreed SLA between the external MnS consumer and the network operator, the 3GPP management system administrator does the following:

- Creates new AccessRule(s); if no existing 	apply, that will govern the interactions with the granted service API(s). It should be possible that the requested service API(s) and the granted service API(s) are different.

- Creates a new Role(s) to associate with the newly AccessRule(s) or associates an old Role with new AccessRule(s).

- Create an MSAC Identity associated with the external MnS consumer and assigns the associated Role(s).

These data are stored in the authentication and authorization MnS producer of the 3GPP management system.

Subsequently, when generating the CAPIF enrolment details (e.g., CCF URL and CCF security information) for the external MnS consumer, the authentication and authorization MnS producer also generates the onboarding access token to enable the CAPIF onboarding procedure.  Finally, the authentication and authorization MnS producer maintains a mapping between the assigned MSAC Identity for the external MnS consumer and the generated onboarding access token.

### 3.1.2 Potential solution #x: Identity class made available to the CCF

#### 3.1.2.1 Introduction

From the 3GPP management system perspective, the Identity class represents information that can be made available to any access control system. The Role class is associated to one or more access rules based on the AccessRule class whose notation is specific to the 3GPP management system (see TS 28.319[2]).

In this regard, the MSAC information that is eligible to be provided to the CCF is limited to the MSAC Identities associated to the different API invokers/external MnS consumers.

#### 3.1.2.2 Description

From clause 3.1.1, the MSAC identity and generated onboarding access token for an external MnS consumer is known by the authentication and authorization MnS producer. The potential solution focuses on how to make the MSAC authorization information for an external MnS consumer available in the CCF as shown in Figure 3.1.2.2-1.

Figure 3.1.2.2-1: External MnS consumer authorization by the CCF

- As a pre-condition, during the CAPIF enrolment phase preparation, the following occurs offline:

  - The external MnS consumer requests to initiate CAPIF enrolment procedure by providing list of requested service APIs to subscribe to the 3GPP management system administrator.

  - The 3GPP management system administrator analyzes the request and depending on the SLA with external MnS consumer creates the associated MSAC identity & corresponsing role(s).

  - The created MSAC identity and corresponsing role(s) and access rule(s) are configured within the authentication and authorization MnS producer.

- The authentication and authorization MnS producer within the management system generates an access token to be used for onboarding to the CCF (called onboarding-access-token henceforth). Next, the authentication and authorization MnS producer maintains a mapping between the MSAC identity associated to an external MnS consumer and the generated onboarding-access-token. Next, the API invoker onboarding procedure occurs as described in clause 6.1 of TS 33.122.

- The AMF of the MSEF provides the onboarding enrolment information to the API invoker. This includes the CCF URL, CCF security info, public key of the external MnS consumer, onboarding-access-token or granted API list)

- To begin the onboarding procedure, the API invoker establishes a secure TLS connection with the CCF.

- The API invoker sends an onboard API invoker request providing the onboarding enrolment information from step 3.

- If the CCF can validate the onboard API invoker request, it proceeds to generate the API invoker profile (which includes API invoker ID, API list, Authentication & authorization info) and sends the onboard API invoker response to the API invoker.

- If the CCF can’t validate the onboard API invoker request, it acknowledges receipt of the onboard API invoker request to the API invoker.

- The CCF forwards the onboard API invoker request to the AMF of the MSEF providing the received the onboarding enrolment information (see clause 5.5.2.2 of TS 29.222 for more details).

- The AMF validates the onboarding API invoker request which includes checking onboarding-access-token and the API list. If the request is valid, the AMF requests the authentication and authorization MnS producer to provide the MSAC identity associated with the received onboarding-access-token.

- The authentication and authorization MnS producer sends the associated MSAC identity to the AMF of the MSEF.

- The AMF validates the onboard API invoker request and forwards the authentication and authorization information (i.e., the associated MSAC identity) for the API invoker to the CCF.

- The CCF generates API invoker profile and maintains a mapping between the generated API invoker id and associated MSAC identity. Next the CCF sends an onboard API invoker response to the API invoker. The response includes the assigned API invoker ID, the granted API list and associated Authentication & authorization info).

- Whenever, the external MnS consumer wants to invoke specific service APIs, it establishes a secure TLS session with the CCF.

- The external MnS consumer sends an access token request to the CCF to invoke specific service API(s) providing the API invoker id and a list of service API(s) it wants to invoke.

- The CCF validates the request, maps the API invoker id to the saved MSAC identity and generates access token with the access token scope = “corresponding MSAC identity”. The CCF sends the access token response to the API invoker.

- To invoke the service API, the API invoker establishes a secure TLS connection with the AEF.

- Subsequently, in steps 17-19, the external MnS consumer can successfully invoke the service API at the AEF.

# 4	Detailed proposal

It is proposed to endorse the solutions described in clauses 3.1.1 and 3.1.2 to enable the authorization of external MnS consumers to invoke service API at the CCF.