3GPP TSG-SA5 Meeting #158	S5-246903

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.abc Add informative annex for example streaming protocols

# Document for:	Approval

Agenda Item:	6.19.22 MonStra

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]	3GPP TS 28.abc V0.1.0 Signalling traffic monitoring management

# 3	Rationale

Since the systems to collect the copied signalling messages emitted from core NF have been in field since decades, the corresponding protocols do exist and are in large-scale use. As consequence, it does not make sense to establish yet another protocol, but SA5 may list several protocols as non-normative options in an annex.

# 4	Detailed proposal

The following changes are proposed to TS 28.abc [1].

Begin of modifications

# Annex A (informative)	 Possible streaming protocols

## A.1 General considerations

More important than the selection of a plain encapsulation protocol is the possibility to map nformation elements of the signalling messages to corresponding information elements of the encapsulation protocol (suite) in order to enable the receiving monitoring system to evaluate the messages in a meanigful way.

The following paragraphs list some important aspects to be considered.

### A.1.1 Integration into existing environments

Because the existing monitoring soutions are used to monitor all kind of equipment, not only 3GPP GC, the corresponding protocols need to be able to encapsulate any protocol with reasonable overhead.

### A.1.2 Monitoring cloud native network functions

In contrast to 2G – 4G the signallng messages between the NF of the 5G core are encypted, such that the interfaces cannot be tapped anymore. However, over the time the NFs of core where moved as VNF and VNF-components (e.g. micro-services) into the cloud. The adoption of virtual machies, containers, or other cloud-native technologies means that signalling messages may have passed through load balancers, NATs, sidecars, and other elements prior to reaching the NF processing logic. Such that the NF processing logic might capture IP packets with packet headers than are unknown on the interface between the NFs. As result, a plain copy of IP packets on the internal interface of the cloud might not work properly, such that the copied signalling message needs to be augmented by certain meta data (“synthetic” information) to allow for meaningful monitoring.

Thus the reporting protocols need to enable to carry corresponding meta data .

### A.1.1.3 Header compression

Since the signalling traffic might use header compression of the http2 payload. Such header compression must be resolved before sending the message to the collecting entity.

### A.1.1.4		Correlation information

The streaming protocol needs the capability to carry optional correlation information to enable the monitoring system to correlate certain messages, e.g. incoming and outgoing messages.

### A.1.1.5	Vendor-specific enhancements

The protocols need mechanisms to be enhanced by vendor-specific fields.

## A.2 Example protocol 1

tbd

End of modifications