| 3GPP TR 28.846 V0.3.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on charging aspects of satellite access Phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

4.1	General	8

4.2	Regenerative-based satellite access	8

4.3	Store and Forward Satellite operation	9

4.4	UEs- SAT- UEs communications on satellite	10

4.5	Roaming aspects of Satellite access	11

5	Business roles and business scenarios	12

5.1	Business roles	12

5.2	Business scenarios	12

5.2.1	S&F operation with spilt MME architecture	12

5.2.2	S&F operation with full EPC on-board architecture	12

5.2.3	Roaming between MNOs with and without satellite networks 	13

5.2.4	5G satellite backhaul without UPF on-board	13

5.2.5	5G satellite backhaul with UPF on-board	13

5.2.6	MVNO which provide satellite communication services	13

5.2.7	Business scenarios for UE-satellite-UE communication	13

6	Charging scenarios and key issues	14

6.1	Topic 1: Charging scenarios for store and forward satellite operation	14

6.1.1	Use cases	14

6.1.1.1	Use Case #1.1: MNO charges SCC	14

6.1.1.2	Use Case #1.2: MNO charged by SSP	14

6.1.2	Potential charging requirements	14

6.1.3	Key issues	15

6.1.3.1	Key issue #3.1: Charging events and charging information required	15

6.1.3.2	Key issue #3.2: NF/Service suitable to provide charging information	15

6.1.4	Possible solutions	15

6.1.4.1	Solution #1.1: MME Charging Trigger Function (CTF) based solution for S&F operation charging of SMS service	15

6.1.4.2	Solution #1.2: MME Charging Trigger Function (CTF) based solution for S&F operation charging with Control Plane CIoT	17

6.1.4.3	Solution #1.3: MME Charging Trigger Function (CTF) based solution for S&F operation charging with User Plane CIoT	20

6.1.5	Evaluation	26

6.1.5.1	Solutions evaluation for Key issue #1.1	26

6.1.5.2	Solutions evaluation for Key issue #1.2	27

6.1.6	Conclusion	27

6.2	Topic 2: Roaming charging of satellite access	27

6.2.1	Use cases	27

6.2.1.1	Use Case #2.1: MNO’s subscriber roams to MNO with satellite network	27

6.2.1.2	Use Case #2.2: MNO’s SSC subscriber roams to MNO	28

6.2.2	Potential charging requirements	28

6.2.3	Key issues	28

6.2.3.1	Key issue #2.1: Charging procedure for satellite roaming	28

6.2.4	Possible solutions	28

6.2.4.1	Solution #2.1: Roaming architecture for satellite roaming case	28

6.2.4.1.1	General description	28

6.2.4.1.2	Architecture description	28

6.2.4.1.3	Procedures description	29

6.2.5	Evaluation	30

6.2.5.1	Solutions evaluation for Key issue #2.1	30

6.2.6	Conclusion	30

6.3	Topic 3: Charging for satellite resource usage between SSP and MNO	30

6.3.1	Use cases	30

6.3.1.1	Use Case #1.1: SSP charging MNO for satellite used for backhaul	30

6.3.1.2	Use Case #1.2: SSP charging MNO for edge computing via UPF deployed on the satellite	30

6.3.2	Potential charging requirements	31

6.3.3	Key issues	31

6.3.3.1	Key issue #3.1: Charging information required	31

6.3.3.2	Key issue #3.2: Charging architecture and procedure	31

6.3.4	Possible solutions	31

6.3.4.1	Solution #3.1: CDR in MNO for SSP to perform wholesale to MNO	31

6.3.4.1.1	General description	31

6.3.4.1.2	Architecture description	31

6.3.4.1.3	Procedures description	32

6.4	Topic 4: Charging scenarios for UE-satellite-UE communication	32

6.4.1	Use cases	32

6.4.1.1	Use Case #4.1: MNO charges SCC	32

6.4.1.2	Use Case #4.2: MNO charged by SSP	32

6.4.2	Potential charging requirements	32

6.4.3	Key issues	33

6.4.3.1	Key issue #4.1: Charging events and charging information required	33

6.5	Topic 5: Charging scenarios for MVNO which provide satellite communication services	33

6.5.1	Use cases	33

6.5.2	Potential charging requirements	33

6.5.3	Key issues	34

6.5.3.1	Key issue #5.1: Charging information, architecture and procedure	34

6.5.4	Possible solutions	34

6.5.4.1	Solution #5.1: MNO performs wholesale charging and MVNO performs retail charging	34

6.5.4.1.1	General description	34

6.5.4.1.2	Architecture description	34

6.5.4.1.3	Procedures description	35

6.5.5	Evaluation	35

6.5.6	Conclusion	36

7.	Conclusions and Recommendations	36

Annex <A>: Change history	37

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies on charging aspects of satellite access Phase 3 based on the TS 22.261[2] and TR 23.700-29 [3]:

This study item is to investigate the following aspects of the satellite access:

- Possible business scenarios;

- Possible charging scenarios and potential charging requirements based on business scenarios;

- Identify the potential charging solutions.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

[4]	3GPP TR 28.844: "Study on charging aspects of satellite in the 5G System"

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TR 22.822: "Study on using Satellite Access in 5G ".

[7]	3GPP TS 32.251: " Packet Switched (PS) domain charging".

[8]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[9]	3GPP TS 23.502: "	Procedures for the 5G System (5GS)".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[12]	3GPP TS 32.253: "	Control Plane (CP) data transfer domain charging ".

[13]	3GPP TS 32.299: "	Diameter charging applications".

[14]	3GPP TS 32.298: "	Charging Data Record (CDR) parameter description".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

S&F	Store and Forward

4	Background

4.1	General

For Rel-19, SA1 has specified the following requirements on charging aspects for satellite access Phase 3 in the TS 22.261[2]:

- A 5G system with satellite access supporting Store and Forward (S&F) Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites).

- A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, for UE-Satellite-UE communication.

- In a 5G system with satellite access, charging data records associated with satellite access(es) shall include the location of the associated UE(s) with satellite access.

4.2	Regenerative-based satellite access

As introduced in the clause 4.5 of TR 28.844[4], the regenerative-based satellite access architecture involves deploying some 5GC network functions on the satellites. The following from figure 6.2.1-1 of TR 23.700-29[3] shows the high-level 5G network architecture for the regenerative-based satellite access.

Figure 4.2-1: Regenerative-based satellite access

4.3	Store and Forward Satellite operation

As specified in the TS 22.261[2], the new capability that S&F operation is an operation mode of a 5G system with satellite-access, allows the satellite to store and forward data when satellite connectivity is intermittently/temporarily unavailable(e.g. when the satellite is not connected via a feeder link or via ISL to the ground network). The following from Figure J-1 of TS 22.261[2] shows the high-level network architecture for S&F operation.

Figure 4.3-1: Illustration of "S&F Satellite operation" modes in a 5G system with satellite access

According to the conclusion of the 5GSAT_ph3 in the TR 23.700-29 [3], SA2 have approved two optional architectures to support the S&F operation:

- Split MME architecture

- full CN onboard the satellite architecture

For split MME architecture, the eNB and part of MME functions on the satellite, and all the other network functions including the other part of MME functions and HSS are deployed on the ground. The example of the S&F architecture based on MME split is shown in the Figure 4.3-2：

Figure 4.3-2: S&F architecture based on MME split

For full CN onboard the satellite architecture, the whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. Each satellite includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE.

Figure 4.3-2: S&F architecture based on full CN onboard the satellite

4.4	UEs- SAT- UEs communications on satellite

As specified in the TS 22.261[2], a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not. The UE-satellite-UE communication scenario is that UEs can communicate using satellite access without the user plane traffic going to the ground network. The following from Figure 6.28.1-1 of TR 23.700-29[3] shows the high-level network architecture for UE-satellite-UE communication.

Figure 4.4-1: UEs- SAT- UEs communications on satellite in same cell with ISL

Editor's note:	SA5 will align with the architecture and procedure for supporting UEs- SAT- UEs communications specified by SA2.

As specified in the TS23.501[10], only IMS voice/video services between two UEs belonging to same PLMN and in non-roaming scenario are considered by UE-satellite-UE communication in this release.

As specified in the TS 23.228[11], to support IMS satellite media plane optimization, the IMS-AGW may be deployed on the satellite(s) that host the gNB and UPF (UL CL/BP and L-PSA) of the 5GC. It is assumed that the satellite(s) can always connect to the ground with IP transport networks. The P-CSCF is responsible for determining whether to activate UE-satellite-UE communication. The following figure shows the reference architecture of IMS satellite media plane optimization.

Figure 4.4-2: Reference architecture of IMS satellite media plane optimization

As specified in the TS 23.228[11], the architecture deployment assumes that ISL(s) can be set-up within the same satellite constellation or across different constellations depending on satellite operator's deployments (SLA). The set of ISL(s) builds up an IP network which is out of 3GPP scope.

In addition, the routing between Onboard AGWs across satellite via ISL is assumed to be based on IP routing and the Onboard AGWs in satellite need to have non-conflicting IP address during resource allocation. It is up to the deployment to manage IP routing across ISL links and is out of 3GPP scope.

4.5	Roaming aspects of Satellite access

As per the TS 22.261[2], clause 6.46.4, the roaming aspects for a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access that support roaming of UE for both satellite access and terrestrial access between 5G satellite networks and 5G terrestrial networks.

-	UEs supporting satellite access that support optimized network selection and reselection to PLMNs with satellite access, based on home operator policy.

The Figure 4.5-1 shows the high-level network architecture for Roaming aspects of transparent satellite access.

Figure 4.5-1: Roaming aspects of transparent satellite access

5	Business roles and business scenarios

5.1	Business roles

The business roles for satellite charging in 5G can be split accordingly:

-	Mobile Network Operator (MNO): an operator who can provide satellite communication services for SCC.

-	Satellite Service Provider (SSP): a provider of satellite network services for MNO, e.g. satellite companies.

- 	Satellite Communication Customer (SCC): a consumer who uses satellite communication services from an MNO, e.g. UE.

-	Mobile Virtual Network Operator (MVNO): an operator that does not have its own radio access or satellite network, but resells communication services, typically under their own brand name, using the network of a host MNO.

Depending on the scenarios an enterprise can play one or several roles simultaneously, e.g. MNO and SSP can be provided by the same enterprise, and apply business rules based on corresponding business agreements and relationships, e.g.:

- 	MNO has agreements with SCC for using satellite communication services.

- 	SSP has agreements with MNO for using satellite network services.

-	MNO has agreements with other MNOs for inbound and outbound roamers.

-	MVNO has agreements with MNO for use of terrestrial network service or satellite network services.

5.2	Business scenarios

5.2.1	S&F operation with spilt MME architecture

Business scenario#1a: S&F operation with spilt MME architecture

A UE with the store and forward capability attaches to the satellite in the store and forward mode, the data could be stored on the satellite when the feeder link is not available. When the feeder link is available, the UE data could be forwarded to the ground DN. Several satellites may be involved to finish the attach procedure or data transmission procedure.

An MNO rents satellites to deploy the eNB and part of MME functions from an SSP, to be able to provide service to its customer. This means that there would be a wholesale agreement between MNO and SSP, where the MNO is the charged party and the SSP is the charging party. The MNO could be charged by SSP based on the usage of involved satellites.

An end user has a subscription with the MNO which allows it to use its services including the satellite store and forward service. The subscriber is charged party and the MNO is the charging party.

5.2.2	S&F operation with full EPC on-board architecture

Business scenario#1b: S&F operation with full EPC on-board architecture

A UE with the store and forward capability attaches to the satellite in the store and forward mode, the data could be stored on the satellite when the feeder link is not available. When the feeder link is available, the UE data could be forwarded to the ground DN. Several satellites may be involved to finish the attach procedure or data transmission procedure.

An MNO rents satellites to deploy the eNB and core NFs from an SSP, to be able to provide service to its customer. This means that there would be a wholesale agreement between MNO and SSP, where the MNO is charged party and the SSP is the charging party. The MNO could be charged by SSP based on the usage of involved satellites.

An end user has a subscription with the MNO, , which allows it to use its services including the satellite store and forward service. The subscriber is the charged party and the MNO is the charging party.

5.2.3	Roaming between MNOs with and without satellite networks

Business scenario#2a: Roaming between MNOs with and without satellite networks.

Subscriber of an MNO without satellite network roams into an MNO network with satellite network. The MNO without satellite network has a wholesale agreement with the MNO with satellite network where the MNO without satellite network would be the charged party and the MNO with satellite network would be the charging party for this scenario. The SCC can have subscription with the MNO that allows roaming into the MNO with satellite network. The SCC subscriber would be charged by the MNO, while the MNO with satellite network would do interconnect charging towards the MNO without satellite network. Likewise, an SCC can have a subscription with an MNO with satellite network allowing it to roam into an MNO without satellite network, where the MNO without satellite network would do interconnect charging towards the MNO with satellite network and the MNO with satellite network would charge the SCC.

5.2.4	5G satellite backhaul without UPF on-board

Business scenario#3a: 5G Satellite Backhaul.

SSP can lease satellites to MNO that can be used as part of the backhaul between (R)AN and 5GC. An MNO use satellites from SSP to achieve satellite backhaul. MNO could be charged by SSP based on the total data volume transferred via the satellite. In this scenario, UPF is not deployed on the satellite.

5.2.5	5G satellite backhaul with UPF on-board

Business scenario#3b: Edge Computing.

SSP can lease satellites to MNO that can be used to deploy, UPF on satellite. An MNO can use satellites from SSP to achieve edge computing via UPF deployed on the satellite. MNO could be charged by SSP based on usage of satellite per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

5.2.6	MVNO which provide satellite communication services

Business scenario#4a: MVNO providing satellite communication services

MVNO rents satellite network resource from MNOs and then provide satellite communication services to its subscribers, i.e. it allows the subscribers usage of 5G data connectivity while in the host MNO.

The host MNO will collect charging information related to 5G data connectivity usage for charging the MVNO, and may collect charging information related to the 5G data connectivity usage for MVNO subscribes (per UE) and convey this to the MVNO. MVNO could be charged by host MNO based on the total data volume or other types of resource usage.

5.2.7	Business scenarios for UE-satellite-UE communication

Business scenario#5a: UE-satellite-UE communication

An end user has a subscription with the MNO which allows it to use its services including the UE-satellite-UE communication. The subscriber is charged party and the MNO is the charging party.

An MNO rents satellites from an SSP, to be able to provide service to its customer. This means that there would be a wholesale agreement between MNO and SSP, where the MNO is the charged party and the SSP is the charging party. The MNO could be charged by SSP based on the traffic and the usage of involved satellites.

6	Charging scenarios and key issues

6.1	Topic 1: Charging scenarios for store and forward satellite operation

6.1.1	Use cases

6.1.1.1	Use Case #1.1: MNO charges SCC

This use case focuses on SCC and MNO business scenario.

An SCC has a subscription with an MNO which rents the satellite from an SSP, to allow MNO to provide S&F operation to its subscribers.

The charging party and charged party can be:

-	Charged party: the SCC identified by the UE.

-	Charging party: MNO.

The MNO charges the subscribers based on the usage of satellites to provide the S&F operation service.

Potential charging requirements: REQ-CH_ SAT_PH3-01, REQ-CH_ SAT_PH3-02 and REQ-CH_ SAT_PH3-03.

6.1.1.2	Use Case #1.2: MNO charged by SSP

This use case focuses on MNO and SSP business scenario.

An MNO has a wholesale agreement to use the satellite from an SSP for the deployment of eNB and NFs.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP

The SSP charges MNO based on the usage of satellites to deploy the eNB and NFs.

Potential charging requirements: REQ-CH_ SAT_PH3-01, REQ-CH_ SAT_PH3-02 and REQ-CH_ SAT_PH3-03.

6.1.2	Potential charging requirements

The following are potential high-level charging requirements for satellite in 5GS, derived from the requirements in TS 23.401 [5].

REQ- CH_ SAT_PH3-01: The 5GS should support collecting charging information for S&F operation with SMS  service.

REQ- CH_ SAT_PH3-02: The 5GS should support collecting charging information for S&F operation with CP CIoT service.

REQ- CH_ SAT_PH3-03: The 5GS should support collecting charging information for S&F operation with UP CIoT service.

6.1.3	Key issues

6.1.3.1	Key issue #3.1: Charging events and charging information required

This key issue is for investigating how to support the S&F operation service charging considering REQ- CH_ SAT_PH3-01. This investigation covers the following:

-	identification of the triggers for charging events for S&F operation;

-	identification and classification of the charging information for S&F operation;

6.1.3.2	Key issue #3.2: NF/Service suitable to provide charging information

This key issue is for investigating how to support the S&F operation charging considering REQ- CH_ SAT_PH3-01. This investigation covers the following:

-	determination of which NF in the 4G system are suitable to provide the charging information to support the S&F operation.

6.1.4	Possible solutions

6.1.4.1	Solution #1.1: MME Charging Trigger Function (CTF) based solution for S&F operation charging of SMS service

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As described in the TR 23.700-29 [3], S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) that have been supported in the Rel-19. As described in the TS 23.401 [5], the UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach procedure.

For the MME split architecture, the CTF may be deployed on the ground. The MME-ground together with the set of MME-onboard instances deployed in the set of satellites behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard instance is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.1-1: MME split charging architecture

The MME-ground reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME on-board (MO direction) to the SMSC;

-	Short Message received by a UE via the MME on-board (MT direction) from the SMSC.

For the whole EPC on-board architecture, the CTF may be deployed on the satellite.  SA2 has pointed out that this architecture does not support roaming. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.1-2: Whole EPC on-board charging architecture

The MME on-board reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME on-board (MO direction) to the SMSC;

-	Short Message received by a UE via the MME on-board (MT direction) from the SMSC.

The "SMS over MME Charging" information assignment for Service Information has the following charging information enhancements compared with the Table 6.3.1.1a.1 in TS 32.251 [7]:

-	RAT type extends to include "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)"

-	Store and Forward indicator

-	Satellite information

-	Satellite Access Indicator

-	S&F Monitoring list (i.e.Satellite IDs)

-	Store duration

-	Store data volume

The SMS-MO data in SGSN/MME (S-SMO-CDR) / SMS-MT data in SGSN/MME (S-SMT-CDR) shall be produced for each short message via the MME on-board. The following enhancements are needed compared with the Table 6.1.5.1/6.1.6.1  in TS 32.251 [7]:

Table 6.1.4.1 Extend to S-SMO-CDR/ S-SMT-CDR for S&F (3GPP TS 32.251 [7] – Table 6.1.5.1/6.1.6.1)

| Field | Category | Description |
| --- | --- | --- |
| Satellite Access Indicator | OC | This field holds the use of 4G Satellite Access. |
| Satellite information | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| Store and Forward indicator | OC | This field holds the use of Store and Forward Satelllite operation. |
| S&F Monitoring list (1..max) | OC | This field holds the satellite IDs that used for the Store and Forward Satelllite operation. |
| Store duration | OC | This field holds the storage duration of data on the satellite. |
| Store data volume | OC | This field stores the amount of data stored on the satellite. |

For IP data, this solution and MME-CDR would be specified in the TS 32.251[7].

For non-IP data, this solution and MME-CDR would be specified in the TS 32.253[12].

The corresponding AVPs for S&F operation would be specified in the TS 32.299[13].

The MME-CDR ASN.1 would be specified in the TS 32.298[14].

6.1.4.2	Solution #1.2: MME Charging Trigger Function (CTF) based solution for S&F operation charging with Control Plane CIoT

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the clause 5.3.4B of TS 23.401 [5], when the UE accesses the network with Control Plane CIoT EPS Optimisation, the data is stored in the MME-onboard. The UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach and service request procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach and service request procedure.

For the MME split architecture, the CTF may be deployed on the ground. The MME-ground together with the set of MME-onboard instances deployed in the set of satellites behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard instance is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.2-1: MME split charging architecture

The MME-ground reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT CP Optimizations;

-	Service Request via the MME onboard for the CIoT CP Optimizations.

For the whole EPC on-board architecture, the CTF may be deployed on the satellite. SA2 has pointed out that this architecture does not support roaming. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.2-2: Whole EPC on-board charging architecture

The MME on-board reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT CP Optimizations;

-	Service Request via the MME onboard for the CIoT CP Optimizations.

When the MME obtains the attach request or service request including S&F support indication from the UE and determines to support the S&F-based services, the MME (CTF) sends the Charging Data Request to OFCS for S&F operation charging, which has the following enhancement charging information:

-	RAT type extends to include "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)"

-	Satellite Access Indicator

-	Satellite information

-	Store and Forward indicator

-	S&F Monitoring list (i.e.Satellite IDs)

-	Store duration

-	Store data volume

The Figure 6.1.4.2-1 describes the high level charging procedure for MME (CTF) charging for S&F operation charging of MME split architecture.

Figure 6.1.4.2-1: Attach procedure message flow for S&F operation charging with Control Plane CIoT

- If the PLMN broadcast that it is in S&F mode, the UE sends the attach request including S&F support indication.

- If the UE indicates that it supports S&F satellite operation, the on-board MME shall consider whether to accept the request.

3-16. Attach procedure as described in clause 5.3.2.1 of TS 23.401 [5].

17. After successful attach, the MME may provide the estimated UL delivery time, a S&F wait timer and S&F monitoring list to UE in the Attach Accept message as described in clause 5.3.2.1 of TS 23.401 [5].

17ch-a. The MME sends Charging Data Request [Event] including Satellite Access Indicator, Store and Forward indicator and S&F Monitoring list to OFCS for the UE's successful attach for the S&F-based services.

17ch-b. The CDF creates the CDR.

17ch-c. The CDF acknowledges by sending Charging Data Response [Event] to the MME.

The high level charging procedure for whole EPC on-board architecture is similar to above. The only difference is that all the NFs are on the satellite.

The MME-CDR shall be produced for S&F operation charging with Control Plane CIoT via the MME on-board and would be:

Table 6.1.4.2 MME-CDR

| Field | Category | Description |
| --- | --- | --- |
| Satellite Access Indicator | OC | This field holds the use of 4G Satellite Access. |
| Satellite information | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| Store and Forward indicator | OC | This field holds the use of Store and Forward Satelllite operation. |
| S&F Monitoring list (1..max) | OC | This field holds the satellite IDs that used for the Store and Forward Satelllite operation. |
| Store duration | OC | This field holds the storage duration of data on the satellite. |
| Store data volume | OC | This field stores the amount of data stored on the satellite. |

For IP data, this solution and MME-CDR would be specified in the TS 32.251[7].

For non-IP data, this solution and MME-CDR would be specified in the TS 32.253[12].

The corresponding AVPs for S&F operation would be specified in the TS 32.299[13].

The MME-CDR ASN.1 would be specified in the TS 32.298[14].

6.1.4.3	Solution #1.3: MME Charging Trigger Function (CTF) based solution for S&F operation charging with User Plane CIoT

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.401[5], only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data.

UE using satellite access with S&F operation indication in the initial attach or the service request procedure can only be known by MME, MME offline charging can be used to charge the S&F operation of the User Plane CIoT service. For the whole EPC on-board architecture, the CTF may be deployed on the satellite. The high level of the whole EPC on-board architecture is the same as the Figure 6.1.4.1-2 of the solution#1.2.

The MME on-board reports charging information to OFCS about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT UP Optimizations;

-	Service Request via the MME onboard for the CIoT UP Optimizations.

The charging solution is similar with the solution#1.2 except for all the devices are on the satellite.

Since the MME cannot obtain the store duration and store data volume, the charging information would be:

-	RAT type extends to include "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)"

-	Satellite information

-	Store and Forward indicator

-	S&F Monitoring list (i.e.Satellite IDs)

-	Satellite Access Indicator

#### 6.1.4.4	Solution #1.4: MME CDF/CGF based solution for S&F operation of SMS service

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As described in the TR 23.700-29 [3], S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) that have been supported in the Rel-19. As described in the TS 23.401 [5], the UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach procedure.

For the MME split architecture, the CDF/CGF may be deployed on the ground. The MME ground together with the MME onboard behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.4-1: MME split charging architecture

The MME-ground reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME onboard (MO direction) to the SMSC.

-	Short Message received by a UE via the MME onboard (MT direction) from the SMSC.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.4-2: Whole EPC on-board charging architecture

The MME onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME onboard (MO direction) to the SMSC.

-	Short Message received by a UE via the MME onboard (MT direction) from the SMSC.

Table 6.1.4.4-1 shows the enhancements needed for S-SMO-CDR and S-SMT-CDR compared with the Table 6.1.5.1/6.1.6.1 in TS 32.251 [7].

Table 6.1.4.4-1 Extention to S-SMO-CDR/ S-SMT-CDR for S&F (3GPP TS 32.251 [7] – Table 6.1.5.1/6.1.6.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |
| S&F Duration | OC | This field holds the storage duration of data on the satellite. |
| S&F Data volume | OC | This field holds the data volume stored on the satellite. |

#### 6.1.4.5	Solution #1.5: MME CDF/CGF based solution for S&F operation with CP data transfer

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the clause 5.3.4B of TS 23.401 [5], when the UE accesses the network with Control Plane CIoT EPS Optimisation, the data is stored in the MME onboard. The UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach and service request procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach and service request procedure.

For the MME split architecture, the CDF/CGF may be deployed on the ground. The MME ground together with the MME onboard behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.5-1: MME split charging architecture

The MME ground reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following chargeable events:

-	PDN connection creation/release. via the MME onboard for the Control Plane (CP) data transfer.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC onboard architecture is shown below:

Figure 6.1.4.5-2: Whole EPC on-board charging architecture

The MME onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	PDN connection creation/release. via the MME onboard for the CIoT CP Optimizations.

Table 6.1.4.5-1 shows the enhancements needed for CPDT-SNN-CDR compared with the Table 6.1.3.3.1 in TS 32.253 [12].

Table 6.1.4.5-1 Extension to CPDT-SNN-CDR for S&F (3GPP TS 32.253 [12] – Table 6.1.3.3.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |
| S&F Duration | OC | This field holds the storage duration of data on the satellite. |
| S&F Data volume | OC | This field holds the data volume stored on the satellite. |

#### 6.1.4.6	Solution #1.6: SCEF CDF/CGF based solution for S&F with CP data transfer

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the clause 5.3.4B of TS 23.401 [5], when the UE accesses the network with Control Plane CIoT EPS Optimisation, the data is stored in the MME onboard. The UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach and service request procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach and service request procedure.

For the MME split architecture, the CDF/CGF may be deployed on the ground. The MME ground together with the MME onboard behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.6-1: MME split charging architecture

The SCEF reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	PDN connection creation/release via the SCEF for the Control Plane (CP) data transfer.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC onboard architecture is shown below:

Figure 6.1.4.6-2: Whole EPC charging architecture

The SCEF reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	PDN connection creation/release via the SCEF for the Control Plane (CP) data transfer.

There is no enhancement required for the SCEF CDRs needed.

6.1.5	Evaluation

6.1.5.1	Solutions evaluation for Key issue #1.1

Solutions #1.1 addresses Key issue #1.1, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.2 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT CP Optimizations and Service Request via the MME onboard for the CIoT CP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.3 addresses Key issue #1.1, for the User Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT UP Optimizations and Service Request via the MME onboard for the CIoT UP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and S&F Monitoring lists.

Solutions #1.4 addresses Key issue #1.1, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The CDF/CGF may be deployed on the satellite. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring List, S&F Duration and S&F Data volume.

Solutions #1.5 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite.  The extension to CPDT-SNN-CDR for S&F charging include RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.6 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite.  There is no enhancement required for the SCEF CDRs needed.

6.1.5.2	Solutions evaluation for Key issue #1.2

All address Key issue #1.2. Solution #1.1 enhances the MME over SMS charging that MME provides the charging information. Solution #1.2, Solution#1.3, Solution#1.4 and Solution#1.5  directly uses the MME provides the charging information and provides the message flow. Solution #1.6 uses SCEF to report charging information to CGF or Billing Domain about satellite access running in S&F mode.

6.1.6	Conclusion

It is concluded that the solution #1.4 is the feasible solution for the SMS service using the satellite access running in the S&F mode. New trigger events and new parameters need to be added for the S&F operation charging.

It is concluded that the solution #1.6 is the feasible solution for the CP CIoT service using the satellite access running in the S&F mode. No enhancement for the CDRs is needed.

6.2	Topic 2: Roaming charging of satellite access

6.2.1	Use cases

6.2.1.1	Use Case #2.1: MNO’s subscriber roams to MNO with satellite network

This use case focuses on roaming between MNOs with and without satellite networks business scenario.

An SSC subscriber (identified by the UE) has a subscription with an MNO (i.e. HPLMN), the MNO has a roaming agreement with the MNO without satellite network.

When the UE has roamed to the coverage of the satellite network belonging to the other MNO, it will as an inbound roamer, access the satellite network provided by the MNO. The MNO provides the service access and connectivity for the UE.

When the UE moves back to the coverage of the terrestrial network, the MNO provides the terrestrial communication service access and connectivity for the UE based on the network selection policy, which is described in the TR 22.822[6].

For the retail part the charging party and charged party can be:

-	Charged party: SSC

-	Charging party: MNO

For the wholesale part the charging party and charged party can be:

-	Charged party: MNO

-	Charging party: MNO with satellite network

The potential charging requirements for this Use Case is: REQ-CH_ SAT_RO-01.

6.2.1.2	Use Case #2.2: MNO’s SSC subscriber roams to MNO

This use case focuses on roaming between MNOs with and without satellite networks business scenario.

An SCC has a subscription with the MNO with satellite network, the MNO (with the SSC subscription) has a roaming agreement with the MNO.

When the SCC (identified by the UE) has moved to where it has coverage of both MNOs, it may based be on the network selection policy access the MNO, as the inbound roamer. The MNO provides service access and connectivity for the SCC.

For the retail part the charging party and charged party can be:

-	Charged party: SCC

-	Charging party: MNO

For the wholesale part the charging party and charged party can be:

-	Charged party: MNO (with the SSC subscription)

-	Charging party: MNO

The potential charging requirements for this Use Case is: REQ-CH_ SAT_ RO-01.

6.2.2	Potential charging requirements

The following are potential high-level charging requirements for satellite roaming:

REQ- CH_ SAT_RO-01: The 5GS should support collecting charging information for inbound Roaming UEs and out bound roaming UEs.

6.2.3	Key issues

6.2.3.1	Key issue #2.1: Charging procedure for satellite roaming

This key issue is for investigating how to support the satellite roaming considering REQ- CH_ SAT_RO-01 and REQ- REQ- CH_ SAT_RO-02. This investigation covers the following:

-	identification of the charging architecture for the satellite roaming;

-	identification of triggers and potential roaming charging profile mechanism for satellite roaming;

-	identification of charging information required to support the satellite roaming.

6.2.4	Possible solutions

6.2.4.1	Solution #2.1: Roaming architecture for satellite roaming case

6.2.4.1.1	General description

This solution #2.1 which relying on CHF/5G Converged Charging System for both satellite access charging and satellite backhaul charging in roaming cases, including the Home Routed and Local breakout roaming scenarios, addresses the Key Issue #2.1.

6.2.4.1.2	Architecture description

The 5G System high level charging architecture for SMF charging in roaming cases as described in TS 32.255 [8] clause 4.2 can be used to support the satellite backhaul roaming, including three roaming architectures in Figure 6.2.4.1.2-1, Figure 6.2.4.1.2-2, Figure 6.2.4.1.2-3.

In this solution, the satellite access architecture refers to the Transparent Payload Satellite architecture where the 5GC NFs are deployed on the ground, as described in TR 28.844 [4] Figure 4.2-1. For instance, when VPLMN provide the satellite communication service for the inbound roamer, the V-SMF is deployed on the ground, but can report the satellite access or backhaul information to CHF. Both V-CHF (owned by the MNO that can provide satellite communication service) and H-CHF (owned by the MNO that only provide terrestrial communication service) are deployed on the ground.

Figure 6.2.4.1.2-1 The Home Routed roaming architecture

Figure 6.2.4.1.2-2 The Local breakout with N47 roaming architecture

Figure 6.2.4.1.2-3 The Local breakout with N107 roaming architecture

6.2.4.1.3	Procedures description

For satellite backhaul charging:

- According to the figure 4.3.2.2.2 in TS 23.502 [9], for Home-routed Roaming case, the Nsmf_PDUSession_Create Request or Nsmf_PDUSession_Update Request is transferred from V-SMF to H-SMF in step6, including the Satellite backhaul category. The satellite backhaul category is reported to V-CHF and H-CHF, in the HR roaming scenario.

- According to the figure 4.3.2.2.1 in TS 23.502 [9], for Local breakout Roaming, the Nsmf_PDUSession_CreateSMContext Request and Nsmf_PDUSession_UpdateSMContext Request is transferred from AMF to SMF in step3, including the Satellite backhaul category, GEO Satellite ID. The satellite backhaul category and GEO Satellite ID is reported to V-CHF and H-CHF, in the LBO roaming scenario.

For satellite access charging:

- The satellite access type is reported to V-CHF and H-CHF, in both HR and LBO roaming scenarios.

6.2.5	Evaluation

6.2.5.1	Solutions evaluation for Key issue #2.1

The solution #2.1 addresses Key issue #2.1, using V-SMF/H-SMF to provide the charging information for satellite backhaul charging and satellite access charging in roaming cases, with the required charging information (e.g. Satellite backhaul category, GEO Satellite ID) specified in the TS 32.255 [8].  No extension to the existing charging mechanism.

NOTE: The roaming charging for the Regenerative payload satellite architecture depends on the SA2 system and architecture enhancement, which is outside the scope of this TR.

6.2.6	Conclusion

The solution #2.1 is the only solution and therefore recommended into the normative work with the clarification in the TS 32.255 [8].

6.3	Topic 3: Charging for satellite resource usage between SSP and MNO

6.3.1	Use cases

6.3.1.1	Use Case #1.1: SSP charging MNO for satellite used for backhaul

MNO has an agreement for rental of the satellite from SSP to be used for backhaul.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP.

This charging could be based on the total data volume transferred via the satellite.

6.3.1.2	Use Case #1.2: SSP charging MNO for edge computing via UPF deployed on the satellite

SSP provides the satellite infrastructure resources to terrestrial network operator to enable the EAS to be running on the satellite. The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP.

This charging could be based on usage of satellite per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

6.3.2	Potential charging requirements

REQ-CH_ SAT_PH3_BH-01: The 5GS should support collecting and converging charging information per UE related to satellite backhaul service or edge computing for charging of an SSP.

6.3.3	Key issues

6.3.3.1	Key issue #3.1: Charging information required

This key issue is for investigating how to support the charging for satellite resource usage between SSP and MNO considering REQ-CH_ SAT_PH3_BH-01. This investigation covers the following:

-	identification of charging information required to support the satellite resource usage charging.

6.3.3.2	Key issue #3.2: Charging architecture and procedure

This key issue is for investigating how to support the charging for satellite resource usage between SSP and MNO considering REQ-CH_ SAT_PH3_BH-01. This investigation covers the following:

-	identification of the charging architecture for the satellite resource usage between SSP and MNO;

6.3.4	Possible solutions

6.3.4.1	Solution #3.1: CDR in MNO for SSP to perform wholesale to MNO

6.3.4.1.1	General description

A possible solution for key issue #3.1 and #3.2 covering requirements REQ-CH_ SAT_PH3_BH-01.

6.3.4.1.2	Architecture description

Figure 6.3.4.1.2-1: satellite resource usage charging architecture

The SMF and CEF interact with CHF using Nchf interface. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP. The wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

SMF collect charging data related to satellite backhaul (satellite backhaul category, data volumes) for each subscriber.

CEF collect charging data related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

6.3.4.1.3	Procedures description

The message flows for satellite backhaul for each subscriber would be the same as in clause 5.2.2 of TS 32.255. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

The message flows for infrastructure resource usage would be the same as in clause 5.2.2 of TS 32.257. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

The message flows for EAS deployment would be the same as in clause 5.2.3 of TS 32.257. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

6.4	Topic 4: Charging scenarios for UE-satellite-UE communication

6.4.1	Use cases

6.4.1.1	Use Case #4.1: MNO charges SCC

This use case focuses on SCC and MNO business scenario.

An SCC has a subscription with an MNO which rents the satellite from an SSP, to allow MNO to provide UE-sat-UE to its subscribers.

The charging party and charged party can be:

-	Charged party: the SCC identified by the UE.

-	Charging party: MNO.

The MNO charges the subscribers based on the usage of satellites to provide the UE-sat-UE communication service.

Potential charging requirements: REQ-CH_ SAT_IMS-01.

6.4.1.2	Use Case #4.2: MNO charged by SSP

This use case focuses on MNO and SSP business scenario.

An MNO has a wholesale agreement to use the satellite from an SSP.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP

The SSP charges MNO based on the usage of satellites.

Potential charging requirements: REQ-CH_ SAT_IMS-01.

6.4.2	Potential charging requirements

The following are potential high-level charging requirements for satellite in 5GS, derived from the requirements in TS 23.501[10] and TS 23.228[11].

REQ- CH_ SAT_PH3-01: The 5GS should support collecting charging information for UE-sat-UE communication service.

6.4.3	Key issues

6.4.3.1	Key issue #4.1: Charging events and charging information required

This key issue is for investigating how to support the UE-sat-UE communication service charging considering REQ- CH_ SAT_IMS-01. This investigation covers the following:

-	identification of the triggers for charging events for UE-sat-UE communication;

-	identification and classification of the charging information for UE-sat-UE communication;

-	determination of which NF in the IMS are suitable to provide the charging information to support the UE-sat-UE communication.

6.5	Topic 5: Charging scenarios for MVNO which provide satellite communication services

6.5.1	Use cases

#### 6.5.1.1	Use Case #5.1: MVNO charges SCC

This use case focuses on SCC and MVNO business scenario.

An SCC has a subscription with an MVNO which rents network resource from MNOs and then provide satellite communication services to its subscribers.

The charging party and charged party can be:

-	Charged party: the SCC identified by the UE.

-	Charging party: MVNO.

The MVNO charges the subscribers based on data connectivity usage.

#### 6.5.1.2	Use Case #5.2: MVNO charged by MNO

This use case focuses on MVNO and MNO business scenario.

An MVNO has a wholesale agreement to use the network resource from MNOs and then provide satellite communication services to its subscribers.

The charging party and charged party can be:

-	Charged party: MVNO.

-	Charging party: MNO.

The MNO charges MVNO based on the total data volume or other types of resource usage.

6.5.2	Potential charging requirements

The following are potential high-level charging requirements for MVNO which provide satellite communication services:

REQ-CH_ SAT_MVNO-01:  The charging mechanism should support collecting and conveying charging information related to 5G data connectivity usage for each UE.

REQ-CH_ SAT_MVNO-02: The charging mechanism should support collecting and conveying charging information related to 5G data connectivity usage for each MVNO.

6.5.3	Key issues

6.5.3.1	Key issue #5.1: Charging information, architecture and procedure

This key issue is for investigating how to support MVNO which provide satellite communication services considering REQ-CH_ SAT_MVNO-01 and REQ-CH_ SAT_MVNO-02. This investigation covers the following:

-	identification of charging information required to support MVNO which provide satellite communication services.

-	identification of the charging architecture and procedure for MVNO which provide satellite communication services.

6.5.4	Possible solutions

6.5.4.1	Solution #5.1: MNO performs wholesale charging and MVNO performs retail charging

6.5.4.1.1	General description

A possible solution for key issue #5.1 covering requirements REQ-CH_ SAT_MVNO-01 and REQ-CH_ SAT_MVNO-02.

6.5.4.1.2	Architecture description

The 5G System high level charging architecture for 5G data connectivity converged charging architecture in Local Breakout V-SMF to H-CHF scenario reference point representation or 5G data connectivity converged charging architecture in Local Breakout Inter-CHF scenario reference point representation as described in TS 32.255 [8] clause 4.2 can be used to support the MVNO charging which provide satellite communication services.

Figure 6.5.4.1.2-1: 5G data connectivity converged charging architecture in Local Breakout V-SMF to H-CHF scenario reference point representation

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming, the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N47 reference point is used for the interactions between SMF owned by the MNO and A-CHF owned by the MVNO.

Figure 6.5.4.1.2-2: 5G data connectivity converged charging architecture in Local Breakout Inter-CHF scenario reference point representation

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming, the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N107 reference point is used for the interactions between CHF owned by the MNO and A-CHF owned by the MVNO.

There are following differences on the SMF interactions with the CHF in the MNO and A-CHF in the MVNO:

-	V-SMF in V-PLMN is replaced by the SMF in MNO;

-	 V-CHF in V-PLMN is replaced by the CHF in MNO;

-	 H-CHF in H-PLMN is replaced by the A-CHF in MVNO.

N47 or N107 used by A-CHF owned by an additional actor (i.e. MVNO) to perform retail charging for its own subscribers is operator specific.

The CDRs produced by CHF in MNO may be sent for wholesale charging, which then can be used to aggregate and calculate the amount of charging data per MVNO.

For satellite backhaul charging, the satellite backhaul category is reported to CHF in MNO and A-CHF.

For satellite access charging, the satellite access type is reported to CHF in MNO and A-CHF.

6.5.4.1.3	Procedures description

The message flows for the PDU session establishment, PDU session Modification and PDU session Release for LBO charging specified in clause 5.2.2.18 of TS 32.255 [8] could be applicable depending on the deployment scenarios for additional actor (MVNO), with the following differences:

-	V-SMF in V-PLMN is replaced by the SMF in MNO;

-	 V-CHF in V-PLMN is replaced by the CHF in MNO;

-	 H-CHF in H-PLMN is replaced by the A-CHF in MVNO.

6.5.5	Evaluation

Solutions #5.1 solves key issues #5.1. The satellite information is reported to CHF in MNO and A-CHF to support MVNO charging.

6.5.6	Conclusion

It is concluded that the solution #5.1 is the feasible solution and new parameters related to satellite information need to be added for MVNO charging.

7.	Conclusions and Recommendations

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA5#156 | S5-244186 |  |  |  | Initial skeleton | 0.0.0 |
| 2024-08 | SA5#156 | S5-244187 S5-244188 S5-244504 S5-244505 S5-244506 S5-244507 S5-244508 S5-244509 S5-244510 S5-244528 S5-244529 S5-244530 S5-244531 |  |  |  | Add the skeleton to TR 28.846 Add scope and reference to TR 28.846 Add the background for FS_5GSAT_Ph3_CH to TR 28.846 Introduce the background for satellite access charging Add business roles for satellite charging Introduce the use cases for the roaming charging of satellite access Add business scenarios for roaming from terrestrial operator network to satellite operator network Add business scenarios for satellite resource rental between satellite network operator and terrestrial network operator Charging for satellite resource rental between satellite network operator and terrestrial network operator Charging between satellite operator and satellite MVNO Add business scenarios for store and forward satellite operation Add charging scenarios and charging requirements for store and forward satellite operation Add key issues for store and forward satellite operation charging | 0.1.0 |
| 2024-10 | SA5#157 | S5-245878 S5-245908 S5-245909 S5-245910 S5-245911 S5-245912 S5-245913 S5-245914 S5-245915 |  |  |  | Update of business roles and scenarios Add store and forward satellite operation charging solutions for SMS Add store and forward satellite operation charging solutions for Control Plane CIoT Add evaluation for store and forward satellite operation charging Update charging scenario for Satellite Roaming Charging Introduce the solution for Satellite Roaming Charging Clarification of topic 1 use cases Clarification of topic 2 use cases Clarification of topic 3 use cases | 0.2.0 |
| 2024-11 | SA5#158 | S5-246935 S5-246936 S5-246937 S5-246938 S5-246939 S5-246940 S5-246941 S5-246942 S5-246943 S5-246944 S5-246945 S5-246946 S5-246947 S5-246832 S5-246948 S5-246949 S5-246950 S5-246951 |  |  |  | Clarification on the business roles Resolve EN in the satellite roaming charging solution Evaluation and conclusion for satellite charging topic 2 Add Charging requirements and Key issues for satellite resource usage charging Add Possible Solution, Evaluation and Conclusion for satellite resource usage charging Add Use cases, Potential charging requirements and Key issues for MVNO providing satellite service Add Possible Solution, Evaluation and Conclusion for MVNO charging Update background for UE-satellite-UE communication Add business scenarios for UE-satellite-UE communication Add charging scenarios and charging requirements for UE-satellite-UE communication Add key issues for UE-satellite-UE communication charging Update store and forward satellite operation charging solutions to slove Ens Add store and forward satellite operation charging solutions for User Plane CIoT MME CDF_CGF based solution for S_F operation of SMS service MME CDF_CGF based solution for S_F operation with CP data transfer SCEF CDF_CGF based solution for S_F operation with CP data transfer Update evaluation for store and forward satellite operation charging Add conclusion for store and forward satellite operation charging | 0.3.0 |