3GPP TSG-SA5 Meeting #159	S5-250108

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0493 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.623 Reliable notification transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Implement the reliable notification transfer functionality according to TR 28.871 V19.0.0 (2024-12) clauses 5.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add IOC SentNotifications as a child to NtfSubscriptionControl Add state variable to NftSubscriptionControl - operationalState attribute - availabilityStatus attribute potentially with a “dependency/degraded” value - a list of notification types that are not sent (may be unknown). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... 28622 CR0513 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1568 at commit 9dcf1640d396e42b2c427c4b2a385c862bde1c21 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1568 at commit 9dcf1640d396e42b2c427c4b2a385c862bde1c21

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-subscription-control.yang ***

<CODE BEGINS>

module _3gpp-common-subscription-control {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-subscription-control";

prefix "subscr3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines IOCs for subscription and heartbeat control.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.623";

revision 2025-01-19 { reference CR-0493 ; }

revision 2024-05-18 { reference CR-0359 ; }

revision 2024-01-18 { reference "CR-0309 CR-0329" ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-10 { reference "CR0257 CR0260"; }

revision 2022-10-20 { reference CR-0196;   }

revision 2021-01-16 { reference "CR-0120"; }

revision 2020-08-26 { reference "CR-0106"; }

revision 2019-11-29 { reference "S5-197648 S5-197647 S5-197829 S5-197828"; }

feature ReliableNotificationTransfer {

description "Reliable notification transfer allows a consumer to detect

missing notifications and in some cases retrieve them.";

}

typedef NotificationTypes {

description "An extensible enumeration of notification types";

type union {

type enumeration {

enum NOTIFY_MOI_CREATION;

enum NOTIFY_MOI_DELETION;

enum NOTIFY_MOI_ATTRIBUTE_VALUE_CHANGES;

enum NOTIFY_MOI_CHANGES;

enum NOTIFY_EVENT;

enum NOTIFY_NEW_ALARM;

enum NOTIFY_CHANGED_ALARM;

enum NOTIFY_ACKSTATE_CHANGED;

enum NOTIFY_COMMENTS;

enum NOTIFY_CORRELATED_NOTIFICATION_CHANGED;

enum NOTIFY_CHANGED_ALARM_GENERAL;

enum NOTIFY_CLEARED_ALARM;

enum NOTIFY_ALARMLIST_REBUILT;

enum NOTIFY_POTENTIAL_FAULTY_ALARMLIST;

enum NOTIFY_FILEREADY;

enum NOTIFY_FILE_PREPARATION_ERROR;

enum NOTIFY_THRESHOLD_CROSSING;

enum NOTIFY_NOTIFICATION_ERROR;

}

type string;

}

}

grouping ScopeGrp {

description "This <<dataType>> allows to select data nodes in an object

tree whose root is identified by the so called base managed object

instance. The identification of the base object instance is not part of

this <<dataType>> and needs to be specified by other means. The base

managed object instance is typically a managed object instance in an

object tree.

The 'scopeType' and the 'scopeLevel' attributes allow to select managed

object instances. Attributes, attribute fields and attribute elements

cannot be selected.

The 'dataNodeSelector' attribute allows to select managed object

instances, attributes, attribute fields, attribute elements, or

attribute field elements. Its value contains a solution set specific

expression for specifying the data nodes to be selected.";

choice scope {

description "Describes which object instances are selected with

respect to a base object instance.";

case type-level {

leaf scopeType {

type enumeration {

enum BASE_ONLY;

enum BASE_ALL;

enum BASE_NTH_LEVEL;

enum BASE_SUBTREE;

}

mandatory true;

description "If the optional scopeLevel parameter is not supported

or absent, allowed values of scopeType are BASE_ONLY and BASE_ALL.

The value BASE_ONLY indicates only the base object is selected.

The value BASE_ALL indicates the base object and all of its

subordinate objects (incl. the leaf objects) are selected.

If the scopeLevel parameter is supported and present, allowed

values of scopeType are BASE_ALL, BASE_ONLY, BASE_NTH_LEVEL

and BASE_SUBTREE.

The value BASE_NTH_LEVEL indicates all objects on the level,

which is specified by the scopeLevel parameter, below the base

object are selected. The base object is at scopeLevel zero.

The value BASE_SUBTREE indicates the base object and all of its

subordinate objects down to and including the objects on the level,

which is specified by the scopeLevel parameter, are selected.

The base object is at scopeLevel zero.";

}

leaf scopeLevel {

when '../scopeType = "BASE_NTH_LEVEL" or ../scopeType = "BASE_SUBTREE"';

type uint16;

mandatory true;

description "See description of scopeType.";

}

}

case dataNodeSelector {

leaf dataNodeSelector {

type string;

description "The value shall follow the rules of RFC 8641

filter-spec";

reference "RFC 8641 section 5.";

}

}

}

}

grouping NtfSubscriptionControlGrp {

description "Attributes of a specific notification subscription";

leaf notificationRecipientAddress {

type string;

mandatory true;

}

leaf-list notificationTypes {

type NotificationTypes;

description "Defines the types of notifications that are candidates

for being forwarded to the notification recipient.

If the notificationTypes attribute is not supported or not present

all candidate notifications types are forwarded to the notification;

discriminated by notificationFilter attribute.";

}

list scope {

description "Scopes (selects) data nodes in an object tree.";

key idx;

max-elements 8;

leaf idx { type string; }

uses ScopeGrp;

}

leaf notificationFilter {

type string;

description "Defines a filter to be applied to candidate notifications

identified by the notificationTypes attribute.

If notificationFilter is present, only notifications that pass the

filter criteria are forwarded to the notification recipient; all other

notifications are discarded.

The filter can be applied to any field of a notification.

The format of the string shall confrm to a

JSON expressions (Jex) 'JexConditionsExpr'";

reference "3GPP TS 32.161";

}

leaf operationalState {

if-feature ReliableNotificationTransfer;

type types3gpp:OperationalState;

mandatory true;

config false;

}

leaf-list availabilityStatus {

if-feature ReliableNotificationTransfer;

type types3gpp:AvailabilityStatus;

config false;

description "Provides additional information about the operation status

of a resource. ";

}

leaf-list missedNotificationTypes {

if-feature ReliableNotificationTransfer;

type NotificationTypes;

config false;

description "If a producer knows or suspects that it failed to prepare

some notifications, the types of these notifications should be listed

here.

It can be used to detect cases where only some notificationTypes

are missing, e.g. only FM is unavailable, but CM notifications

are working.";

}

}

grouping PreparedNotificationsGrp {

description "Attributes of PreparedNotifications.";

leaf oldestSequenceNo {

type uint64 ;

config false;

description "The sequence number of the oldest notification in the

sentNotificationList.";

}

leaf oldestEventTime {

type yang:date-and-time;

config false;

description "The eventTime of the oldest notification in the

sentNotificationList";

}

leaf newestSequenceNo {

type uint64 ;

config false;

description "The sequence number of the newest notification in the

sentNotificationList.";

}

leaf newestEventTime {

type yang:date-and-time;

config false;

description "The eventTime of the newest notification in the

sentNotificationList.";

}

leaf numberOfStoredNotifications {

type uint64 ;

config false;

description "The number of notifications in the sentNotificationList.";

}

leaf-list sentNotificationList {

type string ;

config false;

description "An in-sequence, ordered set of notification messages,

each containing the representation of a notification as encoded in

the HTTP body (excluding the HTTP headers).

The order shall be oldest first.

Note: Whether a filtered retrival is needed is FFS. In that case the

attribute may have a strucutured type.";

}

}

grouping HeartbeatControlGrp {

description "Attributes of HeartbeatControl.";

leaf heartbeatNtfPeriod {

type uint32;

mandatory true;

units seconds;

description "Specifies the periodicity of heartbeat notification emission.

The value of zero has the special meaning of stopping the heartbeat

notification emission.";

}

leaf triggerHeartbeatNtf {

type boolean;

default false;

description "Setting this attribute to 'true' triggers an immediate

additional heartbeat notification emission. Setting the value to

'false' has no observable result.

The periodicity of notifyHeartbeat emission is not changed.

After triggering the heartbeat the system SHALL set the value

back to false.";

yext3gpp:notNotifyable;

}

}

grouping SubscriptionControlSubtree {

description "Contains notification subscription related classes.

Should be used in all classes (or classes inheriting from)

- SubNetwork

- ManagedElement

If some YAM wants to augment these classes/list/groupings they must

augment all user classes!";

list NtfSubscriptionControl {

description "NtfSubscriptionControl represents a notification

subscription of a notification recipient.

The scope attribute is used to select managed object instances included

in the subscription. The base object instance of the scope is the

object instance name-containing the NtfSubscriptionControl instance.

When the scope attribute is absent, all objects below and including

the base object are scoped.

For most notification types the scope needs to identify a set of

managed object instances, such as for alarm notifications. For some

notification types one or more attributes, attribute fields or

attribute elements may be specified as well, though, such as for

attribute value change notifications. Details on this matter are

specified together with the definition of the notification type.

Note that a scope may also include objects that are not created yet,

for example, when a complete subtree is scoped, or when all objects

with a specific object class are scoped. Object instances added after

creating the subscription are included in the subscription as well

if not explicitly exluded.

The notifications related to the selected

data are candidates to be sent to the address

specified by the notificationRecipientAddress attribute.

The notificationTypes attribute and notificationFilter attribute

allow MnS consumers to control which candidate notifications are

sent to the notificationRecipientAddress.

If the notificationTypes attribute is present, its value identifies

the notification types that are candidates to be sent to the

notificationRecipientAddress. If the notificationTypes attribute is

absent, notifications of all types are candidates to be sent to

notificationRecipientAddress. Notification types supported in the

NtfSubscriptionControl.notificationTypes attribute are the ones

listed in the attribute SupportedNotifications.notificationTypes.

If supported, the notificationFilter attribute defines a filter that

is applied to the set of candidate notifications. The filter is

applicable to all parameters of a notification. Only candidate

notifications that pass the filter criteria are sent to the

notificationRecipientAddress. If the notificationFilter attribute is

absent, all candidate notificatios are sent to the

notificationRecipientAddress.

To receive notifications, a MnS consumer has to create a

NtfSubscriptionControl instance on the MnS producer. A MnS consumer

can create a subscription for another MnS consumer since it is not

required the notificationRecipientAddress be his own address.

When a MnS consumer does not wish to receive notifications any more

the MnS consumer shall delete the corresponding NtfSubscriptionControl

instance.

When a subscription is created and the notification scope inludes

the created subscription object and the subscribed notification types

include notifications reporting object creation (notifyMOICreation

or notifyMOIChanges), the first notification sent related to the

new subscription shall report the creation of the

NtfSubscriptionControl instance. Likewise, when a subscription is

deleted and the notification scope inludes the deleted subscription

object and the subscribed notification types include notifications

reporting object deletion (notifyMOIDeletion or notifyMOIChanges),

the last notification sent related to the subscription shall report

the deletion of the NtfSubscriptionControl instance.

If multiple NtfSubscriptionControl instances are configured to send

the same notification to the same notificationRecipientAddress, then

a separate notification message shall be sent according to each such

NtfSubscriptionControl instance.

The 'reliable notification transfer feature' ensures detection of

failure of notification transfer. It handles two separate error cases:

- the notification was prepared by the producer, but could not be

delivered to the consumer

- the producer could not prepare the notification e.g. due to overload

or internal error.

If the feature is supported this implies support for all of the

following:

- operationalState, availabilityStatus and missedNotificationTypes

attributes

- all notifications carry the sequenceNo and the subscriptionId

parameters

- the IOC PreparedNotifications is supported.

- the notification notifyNotificationError is supported

If notifications are prepared by the producer but their delivery to

the recipient is unsuccessful, the consumer should detect this based

on the sequenceNo and the subscriptionId and retrieve the missing

notifications from the PreparedNotifications MOI.

If the MnS producer is not able to prepare notifications this

is indicated by the operationalState, availabilityStatus and

missedNotificationTypes attributes and the notifyNotificationError

notification. The notificationType notifyNotificationError

shall be automatically included in all subscriptions.

To fetch the information missed, the consumer should retrieve the

data from the relevant managed object instances (e.g. AlarmList,

Files etc.). The information needed is dependent on the subscription.

Creation and deletion of NtfSubscriptionControl instances by MnS

consumers is optional; when not supported, the NtfSubscriptionControl

instances may be created and deleted by the system or be

pre-installed.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NtfSubscriptionControlGrp;

}

list HeartbeatControl {

description "MnS consumers (i.e. notification recipients) use heartbeat

notifications to monitor the communication channels between themselves

and MnS producers configured to emit notifications.

A HeartbeatControl instance allows controlling the emission of

heartbeat notifications by MnS producers. The recipients of heartbeat

notifications are specified by the notificationRecipientAddress

attribute of the NtfSubscriptionControl instance containing the

HeartbeatControl instance.

Note that the MnS consumer managing the HeartbeatControl instance

and the MnS consumer receiving the heartbeat notifications may not be

the same.

As a pre-condition for the emission of heartbeat notifications, a

HeartbeatControl instance needs to be created. Creation of an instance

with an initial non-zero value of the heartbeatNtfPeriod attribute

triggers an immediate heartbeat notification emission. Creation of an

instance with an initial zero value of the heartbeatPeriod attribute

does not trigger an emission of a heartbeat notification. Deletion of

an instance does not trigger an emission of a heartbeat notification.

Once the instance is created, heartbeat notifications are emitted with

a periodicity defined by the value of the heartbeatNtfPeriod

attribute. No heartbeat notifications are emitted if the value is

equal to zero. Setting a zero value to a non zero value, or a non zero

value to a different non zero value, triggers an immediate heartbeat

notification, that is the base for the new heartbeat period. Setting a

non zero value to a zero value stops emitting heartbeats immediately;

no final heartbeat notification is sent.

Creation and deletion of HeartbeatControl instances by MnS Consumers

is optional; when not supported, the HeartbeatControl instances may be

created and deleted by the system or be pre-installed.

Whether and when to emit heartbeat notifications is controlled by

HeartbeatControl. Subscription for heartbeat is not supported via the

NtfSubscriptionControl.";

max-elements 1;

key id;

uses top3gpp:Top_Grp;

container attributes {

uses HeartbeatControlGrp;

}

}

list PreparedNotifications {

if-feature ReliableNotificationTransfer;

description "This IOC represents the set of notifications prepared for

a specific NtfSubscriptionControl subscription. Notifications are

available independent whether they are successfully delivered to

the consumer or not. The MnS producers shall always maintain an

in-sequence continuous set of notifications for the subscription

starting with the newest notification, and may limit the number of

notifications.

It is the responsibility of the consumer to create the MOI and

thus enable its functionality. If the MOI is present the producer

will update the attributes.

The IOC shall be supported if and only if the reliable notification

transfer feature supported.";

yext3gpp:notNotifyable;

max-elements 1;

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PreparedNotificationsGrp;

}

}

}

}

grouping SupportedNotificationsGrp {

description "Attributes of SupportedNotifications.";

leaf-list notificationTypes {

type string;

config false;

description "List of notification types supported by the MnS producer";

}

leaf-list notificationProtocols {

type enumeration {

enum HTTP;

enum HTTP_VES_ENCAPS;

}

config false;

min-elements 1;

description "List of protocols supported for notifications.";

reference "3GPP TS 28.532";

}

}

grouping SupportedNotificationsSubtree {

description "Contains SupportedNotifications.";

list SupportedNotifications {

description "SupportedNotifications represents the notification related

capabilities of a MnS producer.

The notificationTypes attribute lists notificationTypes supported

by the MnSProducer. Specific IOCs can be the source of a specific

but not necessary every supported notificationType.

The notificationProtocols attribute identifies the notification

transport protocols supported by a MnS producer.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SupportedNotificationsGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***