3GPP TSG-SA5 Meeting #159	S5-250141

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0294 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add procedure for intent feasibility check |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedure for intent feasibility check is documented in TR 28.914 and recommended for normative work. It proposes to add the procedure for intent fulfilment feasibility check in TS 28.312. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedure for intent feasibility check and fulfilment in clause 6.3 Procedures for intent management of TS 28.312 based on clause 5.14 in TR 28.914. - Update the procedure of Create an intent in clause 6.3.2. - Update the uml code in Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.X(new), Annex A.1.1, Annex A.1.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3.2	Create an intent

Figure 6.3.2-1 illustrates the procedure for create a new intent for fulfilment.

Figure 6.3.2-1: Procedure for create an intent

1.	MnS Consumer sends a request to create an intent instance (see createMOI operation defined in TS 28.532 [3]) to MnS Producer, specifying the intentMgmtPurpose as "FULFILMENT" and providing intent information for the new intent to be created. The detailed intent information sees attributes (attribute which "isWritable" property is "True") of the concrete intent IOC defined in clause 6.2.

2.	Based on the received request, the MnS Producer creates the concrete intent instance (i.e., instance of intent IOC) and configure the new created intent MOI with the received intent information.

3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created intent instance.

4.		Based on the created intent instance, MnS Producer performs the feasibility check of the intent instance. MnS Producer can perform the feasibility check and get the results based on latest statistics of network or service performance metrics, historical experience (e.g. experience based feasible value range or threshold of performance gain), current operating status including network resource utilization and availability, prediction results based on network simulation system, and predefined checking rules or policies.

NOTE 1:	Whether to perform the feasibility check can be determined according to the feasibility check enabling policy (e.g., enforce to perform feasibility check in any case, enforce to perform feasibility check in specific cases, not to perform feasibility check in specific cases, not to perform feasibility check in any case). And the feasibility check enabling policy can be predefined/configured in the MnS Producer or sent with the intent creation request from the MnS Consumer.

NOTE 2:	No sequence restriction for above step 3 and step 4.

In case the feasibility check result is 'feasible', steps 5a-step8 are continuously executed:

5a.	Based on the created intent, MnS Producer identifies the MOI for managed entities (e.g. ManagedElement, ManagedFunction) and derives one or more executable management tasks (including deployment and configuration parameters) for these managed entities, then MnS producer deploys or configures corresponding managed entities with deployment and configuration parameters to satisfy the intent instance.

6.	During the execution of the intent, MnS Producer continuously monitors intent fulfilment information.

7.	MnS Producer analyses and adjusts the managed entities to ensure the intent is continuously satisfied.

8.	MnS Producer should notify (see notifyMOIAttributeValueChanges notification) MnS Consumer about attribute "objectInstance" of intent report instance and corresponding intent report information that contains one or any combination of intentFulfilmentReport, intentConflictReport and intentFeasibilityCheckReport which are required by the MnS Consumer.

In case the feasibility check result is 'infeasible', following step 5b is executed.

5b.	MnS Producer notifies MnS consumer about intentFeasibilityCheckReport with infeasible information for the created intent instance. The reasons why the feasibility check result is infeasible (e.g., invalid intent expression, the intent conflict) and corresponding recommendations also can be included in the notification.

Next Change

6.3.X	Intent feasibility check before intent fulfilment

Figure 6.3.X-1 illustrates the procedure for checking the feasibility of an intent.

Figure 6.3.X-1: Procedure for Intent Feasibility Check before intent fulfilment

1)	MnS Consumer sends a request to create an Intent MOI (see createMOI operation defined in TS 28.532 [3]) to MnS Producer, specifying the intentMgmtPurpose as "FEASIBILITYCHECK" and providing the intent information for the intent to be created.

2)	Based on the received request, the MnS Producer creates the concrete intent instance (i.e., instance of intent IOC) and configure it with the received intent information.

3)	MnS Producer sends a response (see createMOI operation defined in TS 28.532[3]) to MnS Consumer with attribute "objectInstance" of the created intent instance.

4)	MnS Producer performs an internal feasibility check as described in clause 5.3.3 to determine whether the requested intent can be achieved within the available network or service resource.

5)	MnS Producer should notify (see notifyMOIAttributeValueChanges notification defined in TS 28.532[3]) Mns Consumer about attribute "objectInstance" of the intent report instance and corresponding intents report information of intentFeasibilityCheckReport, including feasibilityCheckResult and optional inFeasibleExpectationInfos (if the result is "inFeasible"), to inform whether the intent is feasible or not.

NOTE: The process may conclude after obtaining the feasibility check result in Step 5.

6)	In case the feasibility check result is "Feasible" and MnS consumer requires to continue fulfilling this intent instance, MnS Consumer sends a request to modify the intent instance (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS Producer, specifying the "objectInstance" of the intent MOI and setting the intentMgmtPurpose attribute to "FULFILMENT".

7)	Based on the received request, MnS Producer modifies the intent instance, setting the value of intentMgmtPurpose attribute from "FEASIBILITYCHECK" to "FULFILMENT".

NOTE: This modifyMOIAttributes request will not trigger an automatic feasibility check by the MnS Producer.

8)	MnS Producer sends a response (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS consumer with the attribute "objectInstance" of the modified intent instance, confirming that the intentMgmtPurpose attribute has been modified to "FULFILMENT".

9)	Since the intent instance has been set for fulfilment successfully, it is equivalent to creating a feasible intent for fulfilment up to this step. For the following steps about intent fulfilment, please refer to Step 5a through Step 8 of the procedure of "Create an intent" in clause 6.3.3.

Next Change

A.1.1	Create an intent

@startuml

title "[Create an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntity" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (intentMgmtPurpose=FULFILMENT, list of attributes of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the feasibility check of the intent instance

alt feasibility check result is "Feasible"

Ref over MnS_Producer, ManagedEntity: 5a. Perform service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 6. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 7. Adjust to fulfil the intent requirement

MnS_Producer -> MnS_Consumer:8. Notify of intent fulfillment Information\n (DN of intent MOI, FulfilmentInfo)

end

end

else feasibility check result is "inFeasible"

MnS_Producer -> MnS_Consumer: 5b. Notify of intent infeasibile information

End

hide footbox

@enduml

Next Change

A.1.X	Intent feasibility check before intent fulfilment

@startuml

title "[Intent Feasibility Check before intent fulfilment]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "NE(s)" as ManagedEntity

Group Intent Feasibility Check

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (intentMgmtPurpose=FEASIBILITYCHECK, list of attributes of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the feasibility check of the intent instance

MnS_Producer -> MnS_Consumer: 5. Notify of intent feasibility check report (DN of IntentReport MOI, feasibilityCheckResult)

End

opt feasibility check result is "Feasible" and MnS consumer requires to continue fulfilling this intent instance

MnS_Consumer -> MnS_Producer: 6. Request to modify the intent instance ('objectinstance' of intent instance, intentMgmtPurpose=FULFILMENT)

MnS_Producer -> MnS_Producer: 7. Modify intentMgmtPurpose of the intent instance from\n    FEASIBILITYCHECK to FULFILMENT

MnS_Producer -> MnS_Consumer: 8. Response for modify an intent instance ('objectinstance' of intent instance)

Ref over MnS_Consumer, ManagedEntity: 9. Perform the intent fulfilment procedure (referring to Step 5a-8 in clause 6.3.2)

End

hide footbox

@enduml

End of Change