3GPP TSG-SA5 Meeting #159	S5-250151

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0032 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 TS 28.537 Usage of MnS Registry NRM fragment for MnS Registry and Discovery for different deployment scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Usage of MnS Registry for different deployment scenarios is documented in TR 28.871 and recommended for normative work. It is proposed to add Usage of MnS Registry for different deployment scenarios in TS 28.537 based on solution recommended in clause 5.14.2 in TR 28.871. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the REQ-DMS-1 as optional requirement - Add description for the Usage of MnS Registry for different deployment scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.2.2	Requirements

REQ-DMS-1: The 3GPP management system should provide capabilities allowing MnS producers to register their management capabilities (including the endpoint address) at MnS discovery service producer for use by MnS consumers wishing to interact with these MnS producers.

REQ-DMS-2: The 3GPP management system shall provide capabilities allowing MnS consumers to retrieve the management capabilities registered at MnS discovery service producer by MnS producers.

REQ-DMS-3: The 3GPP management system shall provide capabilities allowing to discover MnS producers that are managing a specified managed entity.

REQ-DMS-4: The 3GPP management system shall provide capabilities allowing to discover the managed entities a MnS producer is responsible for.

5.X	Usage of MnS Registry for different deployment scenarios

Following are potential deployment scenarios for MnS Registry (i.e. act as discover MnS producer):

- Deployment scenario #1: Separate MnSRegistry. The MnS Registry and a concrete MnS producer are separate entities with a published interface between them. In this scenario, the MnS Registry is used to store the MnS information for MnS instances provided by different MnFs, especially for different MnFs from different vendors. Following is one example, Domain MnF_1 needs to register MnS_1 information in the MnS Registry implemented in Cross Domain MnF, and domain MnF_2 needs to discover/retrieve the MnS_1 information from Cross Domain MnF to consume MnS_1 provided by MnF_1. In this scenario, MnS Registry needs to support both registry and discovery/retrieve capabilities.

Figure 5.X-1 Example of Separate MnSRegistry deployment scenario

- Deployment scenario #2: Embedded MnSRegistry. The MnsRegistry and a concrete MnS producer are hosted by the same entity with an implementation specific interface between them. The MnS Registry is implemented in an MnF (e.g. RAN domain MnF) and used to store the MnS Information for MnS instances provided by the MnF itself. Following is one example, Cross Domain MnF needs to retrieve/discover MnS_1 information in the MnSRegistry implemented in Domain MnF_1. In this scenario, MnS Registry supports discovery/retrieve capabilities only.

Figure 5.X-2 Example of Embedded MnSRegistry deployment scenario

End of Changes