3GPP TSG-SA5 Meeting #159	S5-250167

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0656 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add new measurements related to failed rate of NWDAF roaming analytics service subscriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When operators take management actions such as load balancing and resource allocation, the failed rates are utilized. Therefore, it is necessary to provide measurements of the failed rates of roaming analytics service subscriptions. The procedure for roaming analytics services is defined in TS 23.288. As described in TS 23.288 (SA2) subclause 4.3 and 6.1.5, analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). Considering the scenario of analytics exposure from VPLMN to HPLMN, failed roaming analytics service subscriptions may be caused in two cases: Upon receiving an analytics information request/subscription from a service consumer NF in HPLMN, the H-RE-NWDAF may reject the subscription. If the subscription is accepted by the H-RE-NWDAF, the H-RE-NWDAF sends a roaming analytics subscribe to the V-RE-NWDAF in VPLMN. The V-RE-NWDAF may also reject the subscription. Similarly, considering the scenario of analytics exposure from HPLMN to VPLMN, failed roaming analytics service subscriptions may be caused in two cases: Upon receiving an analytics information request/subscription from a service consumer NF in VPLMN, the V-RE-NWDAF may reject the subscription. If the subscription is accepted by the V-RE-NWDAF, the V-RE-NWDAF sends a roaming analytics subscribe to the H-RE-NWDAF in HPLMN. The H-RE-NWDAF may also reject the subscription. Measurement “Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN where the subscription is requested” and measurement “Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services” provide precise performance analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN where the subscription is requested” is introduced. A new measurement “Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the failed rate of roaming analytics service subscriptions and to take management actions for configuration and resource allocation in roaming case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.9	Measurements related to RE-NWDAF roaming analytics

5.18.9.x	Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN where the subscription is requested

a)	This measurement provides the failed rate of roaming analytics service subscriptions received and rejected by the RE-NWDAF in the PLMN where the subscription is requested

b)	CC

c)  Considering the scenario of analytics exposure from VPLMN to HPLMN, a failed roaming analytics service subscriptions may happen in two cases：

On receipt of an Nnwdaf_AnalyticsSubscription_Subscribe from a service consumer NF in HPLMN, if the H-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

If the Nnwdaf_AnalyticsSubscription_Subscribe is accepted by the H-RE-NWDAF, the H-RE-NWDAF invokes an Nnwdaf_RoamingAnalytics_Subscribe service operation. On receipt of the Nnwdaf_RoamingAnalytics_Subscribe from the H-RE-NWDAF in HPLMN, if the V-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

In this case, the failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN where the subscription is requested can be calculated according to the number of failed roaming analytics service subscriptions rejected by the H-RE-NWDAF and the number of successful roaming analytics service subscriptions.

Considering the scenario of analytics exposure from HPLMN to VPLMN, a failed roaming analytics service subscriptions may happen in two cases：

On receipt of an Nnwdaf_AnalyticsSubscription_Subscribe from a service consumer NF in VPLMN, if the V-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

If the Nnwdaf_AnalyticsSubscription_Subscribe is accepted by the V-RE-NWDAF, the V-RE-NWDAF invokes an Nnwdaf_RoamingAnalytics_Subscribe service operation. On receipt of the Nnwdaf_RoamingAnalytics_Subscribe from the V-RE-NWDAF in VPLMN, if the H-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

In this case, the failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN where the subscription is requested can be calculated according to the number of failed roaming analytics service subscriptions rejected by the V-RE-NWDAF and the number of successful roaming analytics service subscriptions.

d)	A real value

e)	DANS.RoamingCaseRateAnalyticsSubscriptionRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.18.9.y	Failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services

a)	This measurement provides the failed rate of roaming analytics service subscriptions received and rejected by the RE-NWDAF in the PLMN that provides analysis services.

b)	CC

c)  Considering the scenario of analytics exposure from VPLMN to HPLMN, a failed roaming analytics service subscriptions may happen in two cases：

On receipt of an Nnwdaf_AnalyticsSubscription_Subscribe from a service consumer NF in HPLMN, if the H-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

If the Nnwdaf_AnalyticsSubscription_Subscribe is accepted by the H-RE-NWDAF, the H-RE-NWDAF invokes an Nnwdaf_RoamingAnalytics_Subscribe service operation. On receipt of the Nnwdaf_RoamingAnalytics_Subscribe from the H-RE-NWDAF in HPLMN, if the V-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

In this case, the failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services can be calculated according to the number of failed roaming analytics service subscriptions rejected by the V-RE-NWDAF and the number of successful roaming analytics service subscriptions.

Considering the scenario of analytics exposure from HPLMN to VPLMN, a failed roaming analytics service subscriptions may happen in two cases：

On receipt of an Nnwdaf_AnalyticsSubscription_Subscribe from a service consumer NF in VPLMN, if the V-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

If the Nnwdaf_AnalyticsSubscription_Subscribe is accepted by the V-RE-NWDAF, the V-RE-NWDAF invokes an Nnwdaf_RoamingAnalytics_Subscribe service operation. On receipt of the Nnwdaf_RoamingAnalytics_Subscribe from the V-RE-NWDAF in VPLMN, if the H-RE-NWDAF rejects the subscription, each rejected subscription is added to the corresponding counter.

In this case, the failed rate of roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services can be calculated according to the number of failed roaming analytics service subscriptions rejected by the H-RE-NWDAF and the number of successful roaming analytics service subscriptions.

d)	A real value

e)	DANS.RoamingCaseRateRoamingAnalyticsSubscriptionRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes