3GPP TSG-SA5 Meeting #159	S5-250178

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Add Solution of Management of Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Use Case, Requirement, and Solution of Management of Federated Learning has been discussed and agreed (in TR 28.858). This CR is to normatively propose the use case, requirements, and solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add New Use Case, Requirements, and Solution to support Management of Federated Learning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3a.1.2.1, 7.3a.1.2.2, 7.3a.1.2.3, 7.4.w, 7.4.x, 7.4.y, 7.4.z, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Stage 3 will be provided in the meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Third Changes

7.3a.1.2.1	MLTrainingFunction

7.3a.1.2.1.1	Definition

The IOC MLTrainingFunction represents the model that undertakes ML model training. The MOI of MLTrainingFunction is also the container of the MLTrainingRequest, MLTrainingReport, MLTrainingProcess and ThresholdMonitor MOI(s).

This MLTrainingFunction instance is created by the system (MnS producer) or pre-installed, it can only be deleted by the system.

The ThresholdMonitor contains the list of performance measurements and the corresponding thresholds that are monitored and used to identify the need for ML model re-training by the MnS Producer.

The ML training function represented by MLTrainingFunction MOI supports training of one or more MLModel(s).

supportedMLModelTrainingMethods represents information of the supported ML model training method including federated learning.

7.3a.1.2.1.2	Attributes

The MLTrainingFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [12]) and the following attributes:

Table 7.3a.1.2.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedMLModelTrainingMethods | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRepositoryRef | M | T | F | F | T |

7.3a.1.2.1.3	Attribute constraints

None.

7.3a.1.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

7.3a.1.2.2	MLTrainingRequest

7.3a.1.2.2.1	Definition

The IOC MLTrainingRequest represents the ML model training request that is trigered by the ML training MnS consumer.

To trigger the ML model training process, ML training MnS consumer needs create MLTrainingRequest object instances on the ML training MnS producer. The MLTrainingRequest MOI is contained under one MLTrainingFunction MOI.

The MLTrainingRequest MOI may represent the request for initial ML model training or re-training. For ML model re-training,  the MLTrainingRequest is associated to one MLModel for re-training a single ML model, or associated to one MLModelCoordinationGroup.

The MLTrainingRequest has a source to identify where it is coming from, which is represented with trainingRequestSource attribute. This attribute may be used by a ML Training MnS producer to prioritize the training resources for different sources.

Each MLTrainingRequest indicates the expectedRunTimeContext that describes the specific conditions for which the MLModel should be trained.

In case the request is accepted, the ML training MnS producer decides when to start the ML model training based on consumer requirements. Once the MnS producer decides to start the training based on the request, the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training;

-	prepares and selects the required training data, with consideration of the consumer’s request provided candidate training data if any. The ML training MnS producer may examine the consumer's provided candidate training data and select none, some or all of them for training. In addition, the ML training MnS producer may select some other training data that are available in order to meet the consumer’s requirements for the ML model training;

-	trains the MLModel using the selected and prepared training data.

The MLTrainingRequest may have a requestStatus field to represent the status of the specific MLTrainingRequest:

-	The attribute values are "NOT_STARTED", " IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to " IN_PROGRESS", the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MLT MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED".

The ML training MnS prodcuer shall delete the corresponding MLTrainingRequest instance in case of the status value turns to "FINISHED" or "CANCELLED". The MnS producer may notify the status of the request to MnS consumer after deleting MLTrainingRequest instance.

fLMLModelTrainingReq in the MLTrainingRequest MOI represents the requirements on federated learning from the consumer, which can be used to determine the participator including FL server and FL clients in the FL process.

7.3a.1.2.2.2	Attributes

The MLTrainingRequest IOC includes attributes inherited from Top IOC (defined in TS 28.622 [12]) and the following attributes:

Table 7.3a.1.2.2.1-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLInferenceName | CM | T | F | F | T |
| candidateTrainingDataSource | O | T | T | F | T |
| trainingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | F | F | T |
| expectedRuntimeContext | M | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| fLMLModelTrainingReq | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRef | CM | T | F | F | T |
| mLModelCoordinationGroupRef | CM | T | F | F | T |

7.3a.1.2.2.3	Attribute constraints

Table 7.3a.1.2.2.3-1

| Name | Definition |
| --- | --- |
| aIMLInferenceName | Condition: MLTrainingRequest MOI represents the request for ML model initial training. |
| mLModelRef | Condition: MLTrainingRequest MOI represents the request for ML model re-training. |
| mLModelCoordinationGroupRef | Condition: MLTrainingRequest MOI represents the request for the joint training of an existing mLModelCoordinationGroup. |

7.3a.1.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

7.3a.1.2.3	MLTrainingReport

7.3a.1.2.3.1	Definition

The IOC MLTrainingReport represents the ML model training report that is provided by the training MnS producer. The MLTrainingReport is associated with one MLModel or one MLModelCoordinationGroup.

The MLTrainingReport instance is created by the training MnS producer automatically when creating an MLTrainingRequest instance.

The MLTrainingReport MOI is contained under one MLTrainingFunction MOI.

candidateFLClients identifies the list of DNs of MLTrainingFunctions that are candicate to be further included in the FL process, which means the fLState of these candicate MLTrainingFunctions changes to “AVAILABLE” and they meets the consumer’s requirements on the FL process. The consumer and producer can determine if to include these MLTrainingFunctions acting as FL clients in the future FL process.

7.3a.1.2.3.2	Attributes

The MLTrainingReport IOC includes attributes inherited from Top IOC (defined in TS 28.622 [12]) and the following attributes:

Table 7.3a.1.2.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| usedConsumerTrainingData | CM | T | F | F | T |
| modelConfidenceIndication | O | T | F | F | T |
| modelPerformanceTraining | M | T | F | F | T |
| areNewTrainingDataUsed | M | T | F | F | T |
| modelPerformanceValidation | O | T | F | F | T |
| candidateFLClients | O | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingProcessRef | M | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |
| mLModelGeneratedRef | M | T | F | F | T |
| mLModelCoordinationGroupGeneratedRef | CM | T | F | F | T |
| mLModelRef | M | T | F | F | T |

7.3a.1.2.3.3	Attribute constraints

Table 7.3a.1.2.3.3-1

| Name | Definition |
| --- | --- |
| trainingRequestRef | Condition: The MLTrainingReport MOI represents the report for the ML model training that was requested by the MnS consumer (via MLTrainingRequest MOI). |
| lastTrainingRef | Condition: The MLTrainingReport MOI represents the report for the ML model training that was not ML model initial training (i.e. the model has been trained before). |
| mLModelCoordinationGroupGeneratedRef | Condition: The MLTrainingReport MOI represents the report for ML model joint training. |

7.3a.1.2.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

End of Third Changes

Start of Fourth Changes

7.4.w	SupportedMLModelTrainingMethods <<dataType>>

7.4.w.1	Definition

This dataType represents the information of the supported ML model training method of the MLTrainingFunction.

fLInfo represents the federated learning related information of the MLTrainingFunction, which should be present when federated learning is supported.

7.4.w.2	Attributes

The SupportedMLModelTrainingMethods includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fLInfo | CM | T | F | F | T |

7.4.w.3	Attribute constraints

Table 7.4.3.3-1

| Name | Definition |
| --- | --- |
| fLInfo | Condition: Federated learning is supported. |

.

7.4.w.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

7.4.x	FLInfo <<dataType>>

7.4.x.1	Definition

This dataType represents the federated learning related information of the MLTrainingFunction, which should be present when federated learning is supported.

fLMLModelTrainingType indicates the supported FL training method for ML Model Training by the MLTrainingFunction, which includes HFL, VFL, and both HFL and VFL.

fLCapability represents the capability of the MLTrainingFunction, which indicates whether the MLTraingFunction is supported to act as FL server, FL client, or both FL server and FL client.

fLState represents the state of the MLTrainingFunction, which indicates whether the MLTrainingFunction can be available for a new federated learning process.

MnS Consumer and MnS Producer can query the FLInfo and then determine the candicate participators including the FL server and FL clients for a new FL process. The producer or consumer will determine the FL server and clients that meet the FL requirements from the consumer to participate in the FL process from the candicate participators.

7.4.x.2	Attributes

The FLInfo includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fLMLModelTrainingType | M | T | F | F | T |
| fLCapability | M | T | F | F | T |
| fLState | M | T | F | F | T |

7.4.x.3	Attribute constraints

None.

7.4.x.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

7.4.y	FLMLModelTrainingReq <<dataType>>

7.4.y.1	Definition

This dataType represents the requirement of the federated learning ML model traing.

7.4.y.2	Attributes

The FLInfo includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fLMLModelTrainingType | M | T | F | F | T |
| fLServerID | O | T | F | F | T |
| fLClientID | O | T | F | F | T |
| fLParticipatorSelectivity | O | T | F | F | T |

7.4.y.3	Attribute constraints

None.

7.4.y.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

7.4.z	FLParticipatorSelectivity <<dataType>>

7.4.y.1	Definition

This dataType represents the selection condition for FL participator including both FL server and FL clients.

7.4.y.2	Attributes

The FLParticipatorSelectivity includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fLTime | O | T | F | F | T |
| dataFeature | O | T | F | F | T |

7.4.y.3	Attribute constraints

None.

7.4.y.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

End of Fourth Changes

Start of Fifth Changes

7.5	Attribute definitions

7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLModelId | It identifies the ML model. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceName | It indicates the type of inference that the ML model supports. allowedValues: see clause 7.4.10 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAType | It indicates the type of inference that the ML model for MDA supports. The detailed definition and corresponding allowed values for mDAType see TS 28.104 [2]. | type: MDAType (TS 28.104 [2]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nwdafAnalyticsType | It indicates the type of inference that the ML model for NWDAF supports. The detailed definition and corresponding allowed values for nwdafAnalyticsID see NwdafEvent in TS 29.520 [20]. | type: NwdafEvent (TS 29.520 [20]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngRanInferenceType | It indicates the type of inference that the ML model for NG-RAN supports. The detailed definition and corresponding allowed values for ngRanInferenceType see clause 7.4a.1 | type: NgRanInferenceType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vSExtensionType | It indicates the type of inference that is vendor's specific extension. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML model training. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model(s). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelConfidenceIndication | It indicates the average confidence value (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. allowedValues: { 0..100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingRequestSource | It identifies the entity that requested to instantiate the MLTrainingRequest MOI. This attribute is the DN of a managed entity, otherwise, it is a String. | type: <<Choice>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML model training request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priority | It indicates the priority of the training process. The priority may be used by the ML model training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the ML training MnS producer to terminate a specific training process. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateProcess.cancelProcess | It allows the ML update MnS consumer to cancel the ML update process. Setting this attribute to "TRUE" cancels the ML update process. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateProcess.suspendProcess | It allows the ML update MnS consumer to suspend the ML update process. Setting this attribute to "TRUE" suspends the ML update process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelVersion | It indicates the version number of the ML model. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| modelPerformanceTraining | It indicates the performance score of the ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the “progressStateInfo“ attribute of the “ProcessMonitor“ data type for the “MLTrainingProcess.progressStatus“. When the ML model training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING” + DN of the MLModel being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML model. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN inference output IE name(s), and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML model, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML model when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It allows the ML training MnS consumer to cancel the ML model training request. Setting this attribute to "TRUE" cancels the ML model training request. The request can be resumed by setting this attribute to "FALSE" when it is suspended. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It allows the ML training MnS consumer to suspend the ML model training request. Setting this attribute to "TRUE" suspends the ML model training process. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It allows the ML training MnS consumer to cancel the ML model training process. Setting this attribute to “TRUE“ cancels the ML model training process. Cancellation is possible when the “mLTrainingProcess.progressStatus.status“ is not the “FINISHED“ state. Setting the attribute to “FALSE“ has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It allows the ML training MnS consumer to suspend the ML model training process. Setting this attribute to "TRUE" suspends the ML model training process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML model for inference. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML model e.g. for training or inference | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areNewTrainingDataUsed | It indicates whether new training data are used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLModel is expected to be applied. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specifies the context under which the MLModel has been trained. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or model is being applied. allowedValues: N/A | type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelRef | It identifies the DN of the MLModel requested to be trained. | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingReport.mLModelGeneratedRef | It identifies the DN of the MLModel generated by the ML model training. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRepositoryRef | It identifies the DN of the MLModelRepository. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML model when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.cancelRequest | It allows the ML testing MnS consumer to cancel the ML testing request. Setting this attribute to "TRUE" cancels the ML testing request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.suspendRequest | It allows the ML testing MnS consumer to suspend the ML testing request. Setting this attribute to "TRUE" suspends the ML testing request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.mLModelRef | It identifies the DN of the MLModel requested to be tested. | type: DN Multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceTesting | It indicates the performance score of the ML model when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedPerformanceIndicators | This parameter lists specific PerformanceIndicator(s) of an ML model. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML model training for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML model testing for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML model. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRefList | It is the list of DN of the MLUpdateRequest MOI that represents an ML update request. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| updatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReportID | It identifies the available ML capability report. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateReport.mLModelRefList | It indicates the DN of MLModel instances that can be updated. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateRequest.cancelRequest | It allows the MnS consumer to cancel the ML update request. Setting this attribute to "TRUE" cancels the ML update request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateRequest.suspendRequest | It allows the MnS consumer to suspend the ML update request. Setting this attribute to "TRUE" suspends the ML update request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| memberMLModelRefList | It identifies the list of member ML models within an ML model coordination group. | type: DN multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLModelCoordinationGroup requested to be trained. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingReport.mLModelCoordinationGroupGeneratedRef | It identifies the DN of the MLModelCoordinationGroup generated by ML model joint training. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLModelCoordinationGroup requested to be tested. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLModel. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingRequest.requestStatus | It describes the status of a particular ML model loading request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingRequest.cancelRequest | It allows the MnS consumer to cancel the ML model loading request. Setting this attribute to "TRUE" cancels the ML model loading. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingRequest.suspendRequest | It allows the MnS consumer to suspend the ML model loading request. Setting this attribute to "TRUE" suspends the ML model loading request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelToLoadRef | It identifies the DN of a trained MLModel requested to be loaded to the target inference function(s). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| policyForLoading | It provides the policy for controlling ML model loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML model to be loaded, and the second threshold is related to the existing ML model being used for inference. | type: AIMLManagementPolicy multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdList | It provides the list of threshold. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLModelLoadingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLModelLoadingProcess.progressStatus". When the ML model loading is in progress, and the " MLModelLoadingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " MLModelLoadingProcess.progressStatus.status " = "RUNNING": The allowed values for " MLModelLoadingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " MLModelLoadingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingProcess.cancelProcess | It allows the MnS consumer to cancel the ML model loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingProcess.suspendProcess | It allows the MnS consumer to suspend the ML model loading process. Setting this attribute to "TRUE" suspends the process. The process can be resumed by setting this attribute to "FALSE" when it is suspended. Suspension is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelLoadingRequestRef | It identifies the DN of the associated MLModelLoadingRequest. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelLoadingPolicyRef | It identifies the DN of the associated MLModelLoadingPolicyRef. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| loadedMLModelRef | It identifies the DN of the MLModel that has been loaded to the inference function. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLManagementPolicy.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: ManagedActivationScope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLInferenceFunction.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: AIMLManagementPolicy multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: TimeWindow multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: GeoArea multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| usedByFunctionRefList | It provides the DNs of the functions supported by the AIMLInferenceFunction. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputId | It identifies an inference output within an AIMLinferenceReport. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputs | It indicates the Outputs that have been derived by the AIMLInferenceFunction instance from a specific ML model. Each ML model, inferenceOutputs may be a set of values. allowedValues: N/A. | type: InferenceOutput multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferencePerformance | It indicates the performance score of the ML model during Inference. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputTime | It indicates the time at which the inference output is generated. allowedValues: N/A | type: DateTime multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| outputResult | It indicates the result of an inference. | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: Null isNullable: False |
| mLCapabilitiesInfoList | It indicates information about what an ML model can generate inference for. allowedValues: N/A. | type: MLCapabilityInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| capabilityName | It indicates the name of a capability for which an ML model can generate inference. The capability is defined by Mns producer which can be traffic analysis capability, coverage analysis capability, mobility analysis capability or vendor specific extensions. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLCapabilityParameters | It indicates a set of optional parameters that apply for an aIMLInferenceName capabilityName. allowedValues: N/A | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceReportRefList | It indicates a list of DN of the AIMLInferenceReport MOI that represents an AIML inference report. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| mLModelRefList | It identifies the list of MLModel DN. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| supportedMLModelTrainingMethods | It represents the information of the supported ML model training method of the MLTrainingFunction. allowedValues: N/A | type: SupportedMLModelTrainingMethods multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLCapability | This attribute defines the federated learning capability, i.e., the role the MLTrainingFunction can play in the federated learning process. allowedValues: “FL_SERVER” indicates that the MLTrainingFunction can act only as Federated Learning Server, “FL_CLIENT” indicates that the MLTrainingFunction can act only as Federated Learning Client, “FL_SERVER_AND_CLIENT” indicates that the MLTrainingFunction can act as both Federated Learning Server and Client. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLState | This attribute defines the federated learning state of the MLTrainingFunction. allowedValues: “AVAILABLE” indicates that the MLTrainingFunction is available for a new federated learning process “UNAVAILABLE” indicates that the MLTrainingFunction is unavailable for a new federated learning process | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLMLModelTrainingType | This attribute defines the federated learning type. allowedValues: “HFL” indicates that the ML Model Traning method is HFL. “VFL” indicates that the ML Model Traning method is VFL. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLServerID | It identifies the DN of MLTrainingFunction acting as the FL Server that should be included in the FL process. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLClientID | It identifies the list of DN of MLTrainingFunction acting as the FL client that should be included in the FL process. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| fLParticipatorSelectivity | It indicates the condition to select FL Client and FL Server. | type: FLParticipatorSelectivity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fLTime | This attribute defines the federated learning time. allowedValues: N/A | type: TimeWindow multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |  |
| dataFeature | This attribute defines the feature that should be included in the.dataset for HFL process. The data used for HFL is listed in clause 4a.1. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |  |
| candidateFLClients | It identifies the list of DN of MLTrainingFunction that is caple of acting as the FL client and can be further included in the FL process. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None |  |
| NOTE: When the performanceScore is to indicate the performance score for ML model training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML model testing, the data set is the testing data set. |  |  |

7.5.2	Constraints

None.

End of Fifth Changes

End of Sixth Changes

End of Sixth Changes