3GPP TSG-SA5 Meeting #159	S5-250187

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add solution of using NDT to generate ML training data for TS 28.561 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case of using NDT to generate ML training data is agreed in TR28.915. This CR includes the solution, including the information model definition and procedure for generating ML training data using NDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the solution of using NDT to generate ML training data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution of using NDT to generate ML training data will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause X(new), Clause Y(new), Clause A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

X	Stage 2 definition for NDT capabilities and services

X.1	Management operation for NDT (MnS component type A)

The operations (e.g. createMOI operations) and notifications (e.g. notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [3] can be used for NDT management, including NDT life-cycle management, which can be treated as Managed Object instances.

Following is the IS to support NDT life-cycle management:

Table X.1-1: IS to support NDT life-cycle management

| NDT life-cycle management | IS operation |
| --- | --- |
| Create an NDT | createMOI operation |
| Delete an NDT | deleteMOI operation |
| Configuration an NDT | modifyMOIAttributes operation |
| Re-configuration an NDT | modifyMOIAttributes operation |
| Activate an NDT | modifyMOIAttributes operation |
| Deactivate an NDT | modifyMOIAttributes operation |

X.2	Information model definition for NDT (MnS component typeB)

X.2.1	Generic Information model definition

X.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [6], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.658 [10], DataType, ThresholdInfo | ThresholdInfo |
| 3GPP TS 28.622 [6], DataType, TimeWindow | TimeWindow |

X.2.1.2	Class diagram

FigureX.2.1.2-1: Relationship UML diagram for NDT

X.2.1.3	Class definition

X.2.1.3.1	NDT <<InformationObjectClass>>

X.2.1.3.1.1	Definition

This IOC represents the properties of an NDT management information between MnS consumer and MnS producer.

The attribute "simulationObject" indicates the area of the actual mobile network or the managed object that needs to be simulated in NDT. For instance, a geography area, a network slice, etc.

The attribute "simulationData" indicates the data collected from the managed entities for NDT simulation, e.g. PM data as defined in 3GPP TS 28.552 [7] and 3GPP TS 28.554 [8], CM data as defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [10], etc.

The attribute "simulationTime" is the time window which indicates if the simulation is for the past, present, or future.

The attribute "estimatedTime" indicates when the simulation can be provided

X.2.1.3.1.2	Attributes

The NDT IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table X.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| simulationObject | CM | T | T | F | T |
| simulationData | CM | T | T | F | T |
| simulationTime | CM | T | T | F | T |
| estimatedTime | CM | T | T | F | T |

X.2.1.3.2	NDTService  <<InformationObjectClass>>

X.2.1.3.2.1	Definition

This IOC represents the properties of an NDT service  management information between MnS consumer and MnS producer.

The NDT IOC contains NDTServiceContext which includes MnS consumer's requirements contexts given to a 3GPP system..

X.2.1.3.2.2	Attributes

The NDTService IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table X.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTServiceID | M | T | T | F | F |
| NDTServiceContext | M | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| ndtReference | M | T | T | F | F |

X.2.1.3.3	NDTReport <<InformationObjectClass>

X.2.1.3.3.1	Definition

This IOC represents NDT service report information from MnS producer to MnS consumer. The NDTReport instance is created by MnS producer automatically when creating an NDT service instance. When the MnS producer delete an NDT service instance based on a request from MnS consumer, the corresponding NDT report instance is also deleted by MnS producer automatically. MnS consumers cannot request MnS producer to create or delete NDTReport instance.

The NDTReport IOC includes

- NDTExecutionReport, which represents the execution result information for NDT service requirements.

X.2.1.3.3.2	Attributes

The NDTReport <<IOC>> includes attributes inherited from Top IOC (defined in TS 28.622 [6]) and the following attributes

Table X.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTExecutionReport | CM | T | T | F | T |
| Attribute related to roles |  |  |  |  |  |
| ndtServiceReference | M | T | T | F | F |

X.2.1.4	DataType definition

X.2.1.4.1	NDTExecutionReport <<dataType>>

X.2.1.4.1.1	Definition

NDTExecutionReport <<dataType>>represents execution result information for NDT service requirements.

The NDTExecutionReport <<dataType>> includes NDTExecutionResults used to describe execution result information for NDT service requirements.

X.2.1.4.1.2	Attributes

The NDTExecutionReport includes the following attributes.

Table X.2.1.4.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTServiceID | M | T | T | F | F |
| NDTExecutionResults | CM | T | T | F | T |

X.2.1.4.1.3	Attribute constraints

None.

X.2.1.4.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

X.2.1.4.4	NDTServiceContext <<dataType>>

X.2.1.4.4.1	Definition

The NDTServiceContext <<dataType>> represents the MnS consumer's requirements contexts for NDT service request..

X.2.1.4.4.2	Attributes

The NDTServiceContext includes the following attributes.

Table X.2.1.4.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTServiceContextAttribute | M | T | T | F | F |
| NDTServiceContextValue | M | T | T | F | F |

X.2.1.4.4.3	Attribute constraints

None.

X.2.2	Scenario specific NDTServiceContext definition

X.2.2.1	Scenario specific NDTServiceContext definition

X.2.2.1.1	Date generation NDT Service Context

X.2.2.1.1.1	Definition

Date generation NDT Service Context is an NDTServiceContext which can be used to represent MnS consumer's requirements for using NDT to generate ML training data.

The Date generation NDT Service Context is defined by utilizing the construct of the generic NDTServiceContext <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the NDTServiceContext for Date generation NDT Service Context.

Table X.2.2.1.1.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | Date generation |

X.2.2.1.1.2	NDTServiceContextAttribute

Following provides the concrete NDTServiceContextAttribute for Date generation. The properties of the attributes in the following table should be the same with properties of NDTServiceContextAttribute defined in clause 6.2.1.3.

Table X.2.2.1.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataSource | O | T | T | F | F |
| dataType | O | T | T | F | F |
| dataSubtype | O | T | T | F | F |
| requiredDataPeriod | O | T | T | F | F |
| dataSamplingPeriods | O | T | T | F | F |

X.2.3	Scenario specific NDTExecutionReport definition

X.2.3.1	Scenario specific NDTExecutionReport definition

X.2.3.1.1	Date generation NDT Execution Report

X.2.3.1.1.1	Definition

Date generation NDT Execution Report is an NDTExecutionReport which can be used to represent the execution result information for NDT service requirements for using NDT to generate ML training data.

The Date generation NDT Execution Report is defined by utilizing the construct of the generic NDTExecutionReport <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the NDTExecutionReport for Date generation NDT Service Context.

Table X.2.3.1.1.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | Date generation |

X.2.3.1.1.2	NDTExecutionResults

Following provides the concrete NDTExecutionResults for Date generation. The properties of the attributes in the following table should be the same with properties of NDTExecutionResults defined in clause X.2.1.4.1.

Table X.2.3.1.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| generatedData | O | T | T | F | F |
| generatedDataAddr | O | T | T | F | F |

The next changes

Y	Procedures for NDT

Y.1	Introduction

This clause describes the procedures for NDT.

Y.Z	Generate ML training data using NDT

FigureY.Z-1: Procedure of generating ML training data using NDT

- 1.The MnS consumer, e.g. ML training Function, sends request to the MnS producer who provides NDT to create an NDT service instance. The detailed NDT service requirement information sees attributes (attribute which "isWritable" property is "True",e.g. Date generation NDT Service Context) of the concrete NDTService IOC defined in clause X.2.

- 3.MnS producer receives the request and determines the attributes to create an NDT instance. The attributes  of the concrete NDT IOC are defined in clause X.2, e.g., simulationObject, simulationData, estimatedTime.

- 3.	The MnS producer sends the response to the MnS consumer including attributes to create an NDT instances, e.g., the attribute estimatedTime indicates when the ML training data can be provided.

4.	The MnS consumer sends the feedback to the MnS producer including whether to agree to the MnS producer's configuration of NDT.

If the MnS consumer agrees to the MnS producer's configuration of the NDT, then go to step 5; if not, the MnS producer needs to re-configure the attributes to create an NDT instance, e.g., simulationObject, simulationData, estimatedTime.

5.	MnS producer collects data from the managed entities,e.g. PM data as defined in 3GPP TS 28.552 [7] and 3GPP TS 28.554 [8], CM data as defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [10], etc.

6.	MnS producer executes network simulation and generates the target data corresponding to the NDT service instance.

7.	The MnS producer sends the NDTReport to the MnS consumer.The detailed NDT report information sees attributes (attribute which "isWritable" property is "True",e.g. Date generation NDT Service Execution Report) of the concrete NDTReport IOC defined in clause X.2.

The next changes

A.X	Information model definition for NDT

A.X.1	Relationship UML diagram for NDT (figure X.2.1.2-1)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<ProxyClass>> \n  ManagedEntity " as ManagedEntity{}

class  "<<InformationObjectClass>>\n NDT" as NDT{}

class "<<InformationObjectClass>>\n NDTService " as NDTService {}

class "<<InformationObjectClass>>\n NDTReport" as NDTReport{}

ManagedEntity "1" *-- "*" NDT : <<names>>

NDT "1" *-- "*" NDTService : <<names>>

NDT "1" *-- "1*" NDTReport : <<names>>

NDTReport "*" <-left-> "1" NDTService

note left of ManagedEntity

Represents SubNetwork,

ManagedElement or

ManagedFunction

end note

@enduml

End of changes