3GPP TSG-SA5 Meeting #159	S5-250189

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel19 TS28.105 New use case on ML inference emulation environment selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR to introduce the agreed ML inference emulation environment selection use case and requirements in TR28.858 clause 5.1.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case of ML inference emulation environment selection is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.x (new), 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

6.3	AI/ML inference emulation

6.3.1	Description

Before the ML model is applied in the production network, the MnS inference consumer may want to receive results of inference in one or more environments that emulate (to different extents) the expected inference characteristics, in a process that may be termed as Inference emulation. The Inference emulation phase enables this.

6.3.2	Use cases

6.3.2.1	AI/ML inference emulation

After the ML model is validated and tested during development, the MnS consumer may wish to receive information from an inference emulation process that indicates if the ML model or the associated ML inference function is working correctly under certain runtime context.

The management system should have the capabilities enabling an MnS consumer:

-	request an inference emulation function to provide emulation reports; and

-	to receive the results from running inference through an AI/ML inference emulation environment available at the emulation MnS producer.

6.3.2.x	ML inference emulation environment selection

ML emulation involves applying the ML model in an emulation environment to verify whether its performance meets the expected inference characteristics. Considering the diversity of ML inference scenarios, one or more emulation environments are provided, each differing in terms of emulation scope, emulation performance, and other factors.

The management system should have the capability to enable an MnS consumer to select the appropriate emulation environment and provide the necessary configuration properties related to that environment. The configuration information may include defining the scope of the emulation , the emulation time, and other relevant parameters.

6.3.3	Requirements for Managing AI/ML inference emulation

Table 6.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_EMUL-1: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to receive reporting about the ML inference emulation. | AI/ML Inference emulation (clause 6.3.2.1) |
| REQ-AI/ML_EMUL-2: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to request an inference emulation function to provide inference emulation reports on an ML model or inference Function. | AI/ML Inference emulation (clause 6.3.2.1) |
| REQ-EMUL_SEL-1 | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to select the emulation environment. | ML inference emulation environment selection (clause 6.3.2.x) |

End of changes