3GPP TSG-SA5 Meeting #159 	S5-250253

Sophia-Antipolis, France, 17 – 21 February 2025

# Source:	Ericsson

# Title:	PM Counter Metadata

# Document for:	Discussion and Endorsement

Agenda Item:	6.19.12

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 28.622: "Subscriber and equipment trace; Trace data definition and management".

[2]	3GPP TS 28.522: "Management and Orchestration: 5G Performance Measurements".

[3]	3GPP TS 32.401: "Performance Management(PM): Concept and requirements".

[4]	3GPP TS 32.404: "Performance Management(PM): Definitions and template".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators(KPI) ".

# 3	Rationale

## 3.1	Present:

In current 3GPP TS 28.622[1], The PerfMetric Job IOC represents a performance metric production job. It can be name-contained by SubNetwork, ManagedElement, or ManagedFunction. The SupportedPerfMetricGroup <<dataType>> captures a group of supported performanceMetrics, and associated (production and monitoring) granularity periods and reporting methods that are supported for the specified performance metric group. The attribute performanceMetrics defines the performance metrics to be produced and the attribute granularityPeriod defines the granularity period to be applied.

Performance metrics include measurements defined in 3GPP TS 28.552[2] and KPIs defined in TS 28.554[5]. TS 28.552[2] follows the template defined in TS 32.404[4].

Also in 3GPP TS 32.401[3] we have the below collection methods defined.

These collection methods are:

- Cumulative Counter (CC): The NE maintains a running count of the event being counted. The counter is reset to a well-defined value (usually "0") at the beginning of each granularity period.

- Status Inspection (SI): Network elements maintain internal counts for resource management purposes. These counts are read at a predetermined rate, the rate is usually based upon the expected rate of change of the count value. Status inspection measurements shall be reset at the beginning of the granularity period and will only have a valid result at the end of the granularity period.

- Gauge: Gauges represent dynamic variables that may change in either direction. Gauges can be integer or real valued. If a gauge is required to produce low and high tide marks for a granularity period (e.g. minimum and maximum call duration), then it shall be reinitialised at the beginning of each granularity period. If a gauge is required to produce a consecutive readout over multiple granularity periods (e.g. cabinet temperature), then it shall only be reinitialised at the start of a recording interval (see definition of "recording interval" in clause 5. 4.1. 3).

- Discrete Event Registration (DER): Data related to a particular event is captured. Every nth event is registered, where n can be 1 or larger. The value of n is dependent on the frequency of occurrence of the event being measured.

- DER measurements shall be reset at the beginning of each granularity period and will only have a valid result at the end of the granularity period.

- Object Mapping (OM): The 3GPP entity maps the measured object(s) for the measurements, that are collected by a method defined by another SDO, from the non-3GPP defined MOI(s) to 3GPP defined MOI(s) representing the managed 3GPP NF(s), and generates the 3GPP measurement(s) for the 3GPP defined MOI with or without processing of the mapped (source) measurements.

The measurement result data can be collected in a non-3GPP defined NE of the network with a number of collection methods which are not defined in this document. These collection methods are referred to as “externally defined collection methods”

This following item describes the collection method to be used for collecting measurements result data from a non-3GPP defined NE.

- Transparent Forwarding (TF): The non-3GPP defined NE maintains a count based on the NE’s “externally defined collection method”. The 3GPP system maintains a measurement count that is a snapshot/reading of the non-3GPP defined NE count at each granularity period.

## 3.2	Problem Statement:

- Missing machine readability Aggregation types information (measurement method) in the template

  - Currently, in 3GPP, the template includes the collection method but doesn’t address the Measurement Method, such as the aggregation of samples performed by the counter, like Sum, Min, Max, Avg or Last_update.

  - A counters aggregation is mentioned within its description, indicating whether it performs a sum, average, percentage etc. However, for automated tools, this aggregation is crucial for machine readability, particularly when designing user-defined KPI.

  - For example, if we have a counter calculating ConnTime and user needs to compile a user-defined KPI based on a specific Use case, it is essential to have an attribute in the template that specifies the aggregation method for this counter, as the same counter can support multiple aggregation types (measurement methods), such as Max, Min, Avg or Sum.

  - In such cases, the absence of machine-readable format attribute could prevent automated tools from supporting specific counters/user-defined KPIs. So, in order to enhance efficiency and automation, the counter’s aggregation should be explicitly included in the template.

Problem Statement: Many tools within the Management system, as well as those connected northbound, rely on automated processes to prepare user-defined KPIs. Therefore, it is crucial to provide this information in a machine-readable format to support the compilation of KPIs similar to what is available for the collection method.

- Missing machine readability resetAtGranPeriod info in the template

  - In the 3GPP specs, the collection method CC states that counters reset to a well=defined value (usually “0”), at the beginning of each GP. Additionally, for Gauge, it mentions that counters will or will not reset based on the counter. However, it is crucial to clearly specify in the specification which counters are reset, and which does not.

  - This distinction is particularly important for tools, especially automated systems, to accurately interpret the mechanism. To facilitate this, machine-readable information should incorporate into the template, enabling tools to process and understand this behaviour effectively.

  - Assuming that all counters within a specific aggregation type (or collection method) are reset to a well-defined value can lead to inaccurate performance evaluations, as certain counters within same aggregation type may have different reset values, which are intentionally designed to address specific network performance scenarios. This information, utilized by the tools, is critical, and any misinterpretation could result in incorrect performance assessments.

  - Few HO Counters (CC/Sum) related with primary band/Additional bands doesn’t reset at GP in NR.

  - Certain counters of a specific collection method (e.g., Guage) and Aggregation type (measurement method) that typically do not reset at the end of a GP are reset in certain cases in NR. For example, counters that calculate the number of UE’s in RRC_Connected state, do reset at the end of the GP.

  - This information is essential because certain counters are specifically designed to address particular use cases based on requirements. Network tools rely on this data to evaluate performance, such as conducting delta analysis and other assessments.

Problem statement: Most counter measurements are reset at the end of the GP, however, some counters do not, as they are designed to support specific uses cases. This information is crucial for tools to interpret the counter measurements behavior and accurately present the results.

## 3.3	Potential Solutions

The current template defined in TS 32.404[4] captures various performance capabilities essential for understanding counter functionalities. Similarly, it is equally important to include the additional details proposed in this DP such as aggregation types (measurement methods), reset at GP, and bin range - as they provide crucial insights into measurement behaviour.

Therefore, it is proposed to include in the template as below:

- add Aggregation type in a sub-bullet

  - Aggregation type: Specifies the measurement method used by the counter measurement (Sum, Avg, etc.) and this can be included as a sub-bullet within Collection Method as b.1

  - The following is the example of the updates in template:

    - b) Collection Method: Contains the form in which measurement data is obtained.

    - b.1) Aggregation:  Average/Sum/Percentage/Min/Max/Peak etc.…

- add resetAtGP in a sub-bullet

  - resetAtGP: Specifies the information if the counter measurement shall be reset at every GP or not, this can be included as new sub-bullet in the template within Condition as c.1

  - The following is the example of the updates in template:

    - c) Condition: Contains the condition which causes the measurement result data to be updated

    - c.1) ResetInterval: number of minutes. Value 0 means the measurement is never reset.

# 4	Detailed proposal

It is proposed to endorse the proposal A / B as specified in section 3.3:

- Update the measurement template in TS32.404/28.558

- Update the measurement definition in TS28.552/TS28.558 where it is applicable