3GPP TSG-SA5 Meeting #159	S5-250376

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.277 | CR | 0054 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging-related information in support of ProSe Multi-Hop UE-to-Network relay communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.4.5 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ProSe Multi-Hop UE-to-Network relay communication standardized in SA2 enables communication between a remote UE and the network via one or more relay UEs. Charging for this communication is based on the specific communication chain used, including the number of hops and the intermediate relays involved. Consequently, charging functions require more detailed information to ensure accurate billing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance charging mechanisms for ProSe Multi-Hop UE-to-Network relay communication by adding the necessary charging-related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accurate billing for ProSe Multi-Hop UE-to-Network relay communication is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, 6.5.2.X(new), 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.5.2.1	Definition of ProSe Information

The fields of the ProSe specific charging information used for 5G ProSe is provided within the Service Specification Information.

The detailed structure of the ProSe Information can be found in table 6.5.2.1.1.

Table 6.5.2.1.1: Structure of ProSe information

| Information Element | Category | Description |
| --- | --- | --- |
| Announcing PLMN ID | OC | Described in table 6.3.1.2.1. |
| Announcing UE HPLMN Identifier | OM | Described in table 6.3.1.2.1. |
| Announcing UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitoring UE HPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitoring UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoverer UE HPLMN Identifier | OM | Described in table 6.3.1.2.1. |
| Discoverer UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoveree UE HPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoveree UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitored PLMN Identifier | OC | Described in table 6.3.1.2.1. |
| ProSe Application ID | Oc | Described in table 6.3.1.2.1. |
| Application ID | Oc | Described in table 6.3.1.2.1. |
| Application Specific Data | Oc | Described in table 6.3.1.2.1. |
| ProSe functionality | Oc | This IE holds the ProSe functionality UE is requesting, e.g. direct discovery (Announcing, Monitoring, or Match Report), direct communication. |
| ProSe Event Type | Oc | Described in table 6.3.1.2.1. |
| Direct Discovery Model | Oc | Described in table 6.3.1.2.1. |
| Validity Period | Oc | Described in table 6.3.1.2.1. |
| Role of UE | Oc | Described in table 6.3.1.2.1. |
| ProSe Request Timestamp | Oc | Described in table 6.3.1.2.1. |
| PC3 Protocol Cause | Oc | Described in table 6.3.1.2.1. |
| Monitoring UE Identifier | Oc | Described in table 6.3.1.2.1. |
| Requestor PLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Requested Application Layer User ID | Oc | Described in table 6.3.1.2.1. |
| Requested PLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Time Window | Oc | Described in table 6.3.1.2.1. |
| Range Class | Oc | Described in table 6.3.1.2.1. |
| Proximity Alert Indication | Oc | Described in table 6.3.1.2.1. |
| Proximity Alert Timestamp | Oc | Described in table 6.3.1.2.1. |
| Proximity Cancellation Timestamp | Oc | Described in table 6.3.1.2.1. |
| Hop Count | Oc | This value reflects the number of Relays for the 5G ProSe Remote UE to reach the network in ProSe multi-hop UE-to-Network relay communication. |
| Relay IP address | Oc | Described in table 6.3.1.2.1. |
| ProSe UE-to-Network Relay UE ID | Oc | Described in table 6.3.1.2.1. |
| ProSe Destination Layer-2 ID | Oc | The identifier of a link-layer that identifies a device or a group of devices that are recipients of ProSe communication frames. |
| Multihop Relay Information | Oc | This field holds a list of Multihop Relay Information defined in clause 6.5.2.x |
| PFI Container information | OC | This field holds a list of PFI data container information defined in clause 6.5.2.3. |
| Transmission Data Container | OC | Described in table 6.3.1.2.1. |
| Local Sequence Number | Oc | Described in table 6.3.1.2.1. |
| Change Time | Oc | Described in table 6.3.1.2.1. |
| Coverage status | Oc | Whether UE is served by NG-RAN or not, i.e. in coverage, out of coverage. |
| User Location Information | Oc | The location of the UE, e.g. NCGI. |
| Data Volume Transmitted | Oc | Described in table 6.3.1.2.1. |
| Change Condition | Oc | Described in table 6.3.1.2.1. |
| VPLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Usage information report sequence number | Oc | Described in table 6.3.1.2.1. |
| Radio Resources indicator | Oc | Described in table 6.3.1.2.1. |
| Radio Frequency | Oc | Described in table 6.3.1.2.1. |
| PC5 Radio Technology | OM | Described in table 6.3.1.2.1. |
| Reception Data Container | Oc | Described in table 6.3.1.2.1. |
| Local Sequence Number | Oc | Described in table 6.3.1.2.1. |
| Change Time | Oc | Described in table 6.3.1.2.1. |
| Coverage status | Oc | Described in table 6.3.1.2.1. |
| User Location Information | Oc | The location of the UE, e.g. NCGI. |
| Data Volume Received | Oc | Described in table 6.3.1.2.1. |
| Change Condition | Oc | Described in table 6.3.1.2.1. |
| VPLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Usage information report sequence number | Oc | Described in table 6.3.1.2.1. |
| Radio Resources indicator | Oc | Described in table 6.3.1.2.1. |
| Radio Frequency | Oc | Described in table 6.3.1.2.1. |
| PC5 Radio Technology | OM | Described in table 6.3.1.2.1. |

# * * * * Second change * * * *

6.5.2.X	Definition of Multihop Relay Information

Multihop Relay Information specific charging information used for 5G ProSe Multi-hop UE-to-Network Relay Communication charging is provided within the Multihop Relay Information described in table 6.5.2.X.1.

Table 6.5.2.X.1: Structure of Multihop Relay Information

| Information Element | Category | Description |
| --- | --- | --- |
| Intermediate Relay IP address | Relay IP address | IP address of Intermediate Relay |
| ProSe UE-to-Network Intermediate Relay UE ID | ProSe UE-to-Network Relay UE ID | ProSe UE-to-Network Intermediate Relay UE ID |
| ProSe Destination Layer-2 ID | ProSe Destination Layer-2 ID | ProSe Destination Layer-2 ID |

# * * * * Third change * * * *

6.5.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data for 5G ProSe converged charging.

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.5.3.1 illustrates the basic structure of the supported fields in the Charging Data Request for 5G ProSe converged charging.

Table 6.5.3.1: Supported fields in Charging Data Request message

| Information Element |  | Node Type | Direct Discovery |  | Direct Communication |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | --E |  | IUTE |  |  |
| Subscriber Identifier |  |  | --E |  | IUTE |  |  |
| Tenant Identifier |  |  | --E |  | IUTE |  |  |
| NF Consumer Identification |  |  | --E |  | IUTE |  |  |
| NF Functionality |  |  | --E |  | IUTE |  |  |
| NF Name |  |  | --E |  | IUTE |  |  |
| NF Address |  |  | --E |  | IUTE |  |  |
| NF PLMN ID |  |  | --E |  | IUTE |  |  |
| Invocation Timestamp |  |  | --E |  | IUTE |  |  |
| Invocation Sequence Number |  |  | --E |  | IUTE |  |  |
| One-time Event |  |  | --E |  | --E |  |  |
| One-time Event Type |  |  | --E |  | --E |  |  |
| Notify URI |  |  | I-- |  | I-- |  |  |
| Triggers |  |  | --E |  | IUTE |  |  |
| Multiple Unit Usage |  |  | --E |  | IUTE |  |  |
| ProSe Information |  |  |  |  |  |  |  |
| Announcing PLMN ID |  |  | ---E |  | - |  |  |
| Announcing UE HPLMN Identifier |  |  | ---E |  | - |  |  |
| Announcing UE VPLMN Identifier |  |  | ---E |  | - |  |  |
| Monitoring UE HPLMN Identifier |  |  | ---E |  | - |  |  |
| Monitoring UE VPLMN Identifier |  |  | ---E |  | - |  |  |
| Discoverer UE HPLMN Identifier |  |  | ---E |  | - |  |  |
| Discoverer UE VPLMN Identifier |  |  | ---E |  | - |  |  |
| Discoveree UE HPLMN Identifier |  |  | ---E |  | - |  |  |
| Discoveree UE VPLMN Identifier |  |  | ---E |  | - |  |  |
| Monitored PLMN Identifier |  |  | ---E |  | - |  |  |
| ProSe Application ID |  |  | ---E |  | - |  |  |
| Application ID |  |  | ---E |  | IUTE |  |  |
| Application Specific Data |  |  | - |  | IUTE |  |  |
| ProSe functionality |  |  | ---E |  | IUTE |  |  |
| ProSe Event Type |  |  | ---E |  | - |  |  |
| Direct Discovery Model |  |  | ---E |  |  |  |  |
| Validity Period |  |  | ---E |  |  |  |  |
| Role of UE |  |  | ---E |  |  |  |  |
| ProSe Request Timestamp |  |  | ---E |  |  |  |  |
| PC3 Protocol Cause |  |  | ---E |  |  |  |  |
| Monitoring UE Identifier |  |  | ---E |  |  |  |  |
| Requested Application Layer User ID |  |  | ---E |  |  |  |  |
| Requested PLMN Identifier |  |  | ---E |  |  |  |  |
| Time Window |  |  | ---E |  |  |  |  |
| Range Class |  |  | ---E |  |  |  |  |
| Proximity Alert Indication |  |  | ---E |  |  |  |  |
| Proximity Alert Timestamp |  |  | ---E |  |  |  |  |
| Proximity Cancellation Timestamp |  |  | ---E |  |  |  |  |
| Relay IP address |  |  | - |  | IUTE |  |  |
| ProSe UE-to-Network Relay UE ID |  |  | - |  | IUTE |  |  |
| ProSe Destination Layer-2 ID |  |  | - |  | IUTE |  |  |
| Multihop Relay Information |  |  | - |  | IUTE |  |  |
| PFI Container information |  |  | - |  | IUTE |  |  |
| Transmission Data Container |  |  | - |  | IUTE |  |  |
| Reception Data Container |  |  | - |  | IUTE |  |  |

Table 6.5.3.2 illustrates the basic structure of the supported fields in the Charging Data Response for 5G ProSe converged charging.

Table 6.5.3.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | Direct Discovery | Direct Communication |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | --E | IUTE |
| Invocation Timestamp |  | --E | IUTE |
| Invocation Result |  | --E | IUTE |
| Invocation Result |  | --E | IUTE |
| Failed parameter |  | --E | IUTE |
| Failure Handling |  | --E | IUTE |
| Invocation Sequence Number |  | --E | IUTE |
| Session Failover |  | - | IUTE |
| Supported Features |  | - | IUTE |
| Triggers |  | I--E | IUTE |
| Multiple Unit Usage |  | --E | IUTE |
| Result Code |  | --E | IUTE |
| Rating Group |  | --E | IUTE |
| Granted Unit |  | --E | IUTE |
| Tariff Time Change |  | --E | IUTE |
| Time |  | --E | IUTE |
| Total Volume |  | --E | IUTE |
| Uplink Volume |  | --E | IUTE |
| Downlink Volume |  | --E | IUTE |
| Service Specific Units |  | --E | IUTE |
| Validity Time |  | --E | IUTE |
| Final Unit Indication |  | --E | IUTE |
| Time Quota Threshold |  | --E | IUTE |
| Volume Quota Threshold |  | --E | IUTE |
| Unit Quota Threshold |  | --E | IUTE |
| Quota Holding Time |  | --E | IUTE |
| Triggers |  | --E | IUTE |

# * * * * End of changes * * * *