3GPP TSG-SA5 Meeting #159	S5-250377

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	China Telecom

# Title:	DP on charging aspects for 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay

# Document for:	Discussion

Agenda Item:	7.4.5

1	Decision/action requested

The group is asked to discuss and endorse this proposal.

2	References

[1]	3GPP TS 32.277 Charging Management; Proximity-based Services (ProSe) charging

[2]	3GPP TS 23.304 Proximity based Services (ProSe) in the 5G System (5GS)

3	Rationale

3.1 Introduction and Background

Layer-3 multi-hop communication between 5G networks and UEs was introduced in Release 19, with architectures and solutions defined in TS 23.304 [2]. This scenario involves one or more intermediate UE-to-Network relays between a 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay. Figure 4.2.7.1A-1 from TS 23.304 [2] (shown below) illustrates this architecture.

TS 23.304 Figure 4.2.7.1A-1: Reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay

TS 32.277 [1] specifies two procedures relevant to charging: ProSe Direct Discovery and ProSe Direct Communication. The impact of ProSe multi-hop UE-to-Network relay communication on these procedures is analyzed below.

3.2 5G ProSe Multi-hop UE-to-Network Relay Discovery

Two models (Model A and Model B) are defined for multi-hop UE-to-Network discovery in TS 23.304 [2].

- Model A: Relays announce themselves, and intermediate relays propagate these announcements, including information about previous relays in the chain. This allows the remote UE to learn about multiple relays and their paths, enabling it to select the optimal relay based on criteria like hop count and QoS.

- Model B: The Remote UE initiates multi-hop discovery by sending a solicitation message with a hop limit. This solicitation travels through intermediate relays, each adding its ID to the path. The UE-to-Network Relay, upon receiving the solicitation, responds and sends the response back along the established path. The Remote UE then selects the optimal path from the received responses

The table below compares UE-to-Network Relay Discovery and Multi-Hop UE-to-Network Relay Discovery features (applicable to both Model A and B)."

Table 3.2-1 UE-to-Network Delay Feature Comparison for Discovery

| Feature | U2N Relay Discovery | Multi-Hop U2N Relay Discovery | Required enhancements |
| --- | --- | --- | --- |
| Relays Involved | Only one UE-to-Network Relay is involved. | Multiple relays (including intermediate relays) | Intermediate relays should be included in the relay discovery procedure |
| Path Discovery | Direct connection; no path discovery needed. | Remote UE discovers the best path through multiple relays | - |
| Hop Count | Not applicable. | Hop count is crucial for path selection and limiting the chain length. | - |
| Message Content | Focuses on the single relay's info. | Includes path information (relay IDs, hop count). | - |
| Discovery Process | Direct announcement and selection. | Involves propagation of announcements by intermediate relays. | - |

Proposal: It is proposed to show the intermediate relays in the message flows of ProSe direct discovery information.

3.3 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay

In scenarios of 5G ProSe Multi-hop layer-3 UE-to-Network Relay, the Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure, The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication. The table below compares UE-to-Network Relay communication and Multi-Hop UE-to-Network Relay communication features (applicable to Model B)."

Table 3.3-1 UE-to-Network Delay Feature Comparison for Communication

| Feature | U2N Relay Communication | Multi-Hop U2N Relay Communication | Required enhancements |
| --- | --- | --- | --- |
| Relays Involved | One UE-to-Network Relay | Multiple relays (including intermediate relays) | Intermediate relays should be included in the relay communication procedure |
| Path Info | Not needed | Required | Path Info should be included in ProSe Info for charging |
| Connection | Direct connection to the relay | Connection via a path of intermediate relays | Connection hops should be included in ProSe info for charging |
| Addressing | Relay assigns addresses to Remote UE | Relay assigns addresses to intermediate relays and Remote UE | - |
| Packet Forwarding | Relay handles forwarding | Intermediate relays and relays handle forwarding | Intermediate relays may report the usage info and trigger the charging procedure |

Path information is crucial for charging, as different communication paths with various relay nodes and QoS levels may result in charging differences. Relay node IDs provide more detailed charging data, and reporting this information to charging functions is helpful. Additionally, as multi-hop communication increases resource consumption, with resource consumption typically increasing as the number of hops grows, the remote UE should report the hop count for more accurate charging.

Proposal 2: It is proposed to add path information in charging information.

Proposal 3: It is proposed to include intermediate relays in the multi-hop relay communication procedure.

4	Detailed proposal

Proposal: It is proposed to consider the above proposals.