3GPP TSG SA WG5 Meeting #159	S5-250398

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei

# Title:	New charging solution for DC application usage by volume

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the follow update in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes new solution for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

5.2.4.y	Solution #y: DC application usage charging by volume per application DC

5.2.4.y.1	General

This solution #x resolves the key issues #2 for REQ_NG_RTC_CH-DCAPP-02. The SMF charging specified in TS 32.255 [10] can support the volume based charging for IMS services, as specified in clause 5.1.18, which may be extended to support the identification of volume for IMS DC application.

5.2.4.y.2	Description

An IMS session may contain one or more IMS application DC(s), which can be identified by the application binding information in the SDP, e.g. "a=3gpp-req-app" attribute with "req-app-id" and "app-dc-info" parameter, as specified in TS 26.114 [8] clause 6.2.13. The DC application usage volume can be differentiated for an IMS application DC.

According to TS 23.228 [5] clause 4.6.1, the P-CSCF can derive the session information that is relevant for Policy and Charging Control from the SDP contained in the SIP signalling and forwards it to the PCF over Rx or N5 interface, which may identify the IMS application DC information.

According to the application detection and control in PCC rule for PDU session management over N7 interface, the PCF may assign a service identifier for an application DC. When SMF receives the PCC rule with service identifier for an application DC, it can support the volume based charging for IMS application DC according to the application based charging principle specified in TS 32.255 [10] clause 5.1.17.

End of changes