3GPP TSG-SA5 Meeting #159	S5-250454

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0238 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.105 Reinforcement Learning UC and Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute India |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case for Managing ML models in use in a live network is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides the use case and requirements for Managing ML models in use in a live network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Sepcification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.5.x	Managing ML models in use in a live network

#### 6.5.x.1	Description

##### 6.5.x.1.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once an ML trained model has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations or replacing current ML model with an earlier model one that was performing better.

##### 6.5.x.1.2	Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model and these KPIs would need to be collectable by the network operator.

#### 6.5.x.2	Potential requirements

REQ-DATA-ACT-1: The 3GPP management system should have a capability enabling an authorized consumer to identify an ML Model that caused performance measurement and/or KPI degradation.

REQ-DATA-ACT-2: The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by an ML model.

REQ-DATA-PERF-1: The 3GPP management system should be able to collect, from the network, performance data pertaining to Network Functions actively utilizing ML models.

REQ-DATA-PERF-2: The 3GPP management system should support performance KPIs that can be monitored for performance assurance purpose.