3GPP TSG-SA5 Meeting #159	S5-250483

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0031 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.540 corrections for MR-DC related description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some incorrect descriptions: - The abbreviation of MR-DC is not align with RAN3. - MR-DC with the 5GC including three archiectures as defined in TS 37.340, The NR-NR Dual Connectivity is missing. - The title of RIM requirements is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Abbreviations of MR-DC. Update the description of MR-DC. Correct the requirements title of RIM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect descriptions will make confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2, 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core network

5GS	5G System

AMF	Access and Mobility Management Function

EN-DC	E-UTRA-NR Dual Connectivity

EPS	Evolved Packet System

MR-DC	Multi-Radio Dual Connectivity

NG-RAN	NG Radio Access Network

NR	New Radio

PCF	Policy Control Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

Next modified section

4.2	MR-DC

Besides single connectivity operation, E-UTRAN, NR and NG-RAN support Multi-RAT Dual Connectivity (MR-DC) operation defined in 3GPP TS 37.340 [5], which can be further divided into following two categories based on connected core network types.

1)	MR-DC with the EPC via EN-DC, the NR node in EN-DC is called en-gNB.

2)	MR-DC with the 5GC via either NG-RAN E-UTRA-NR Dual Connectivity (NGEN-DC) or NR-E-UTRA Dual Connectivity (NE-DC) or NR-NR Dual Connectivity (NR-DC).

There are different user plane connectivity options of the master node and secondary node involved in MR-DC (see detail in 3GPP TS 37.340 [5]).

Next modified section

5.2	Requirements for management of MR-DC

The following specific requirements apply to MR-DC management, including management of NR in EN-DC and other MR-DC operations.

REQ-MRDC_NRM-CON-001: The NRM definitions shall support management of NR node in EN-DC operation (en-gNB).

REQ-MRDC_NRM-CON-002: The NRM definitions shall support management of en-gNB with different user plane connectivity options defined in 3GPP TS 37.340 [5].

REQ-MRDC_NRM-CON-003: The NRM definitions shall support management of NG-RAN nodes in NGEN-DC operation.

REQ-MRDC_NRM-CON-004: The NRM definitions shall support management of different user plane connectivity options in NGEN-DC operation defined in 3GPP TS 37.340 [5].

REQ-MRDC_NRM-CON-005: The NRM definitions shall support management of NG-RAN nodes in NE-DC operation.

REQ-MRDC_NRM-CON-006: The NRM definitions shall support management of different user plane connectivity options in NE-DC operation defined in 3GPP TS 37.340 [5].

REQ-MRDC_NRM-CON-007: The NRM definitions shall support management of NG-RAN nodes in NR-DC operation.

REQ-MRDC_NRM-CON-008: The NRM definitions shall support management of different user plane connectivity options in NR-DC operation defined in 3GPP TS 37.340 [5].

Next modified section

5.7	Requirements Remote Interference Management

REQ-RIM_NRM-CON-001: The NRM definitions should support Remote Interference Management.

End of modified sections