3GPP TSG-SA5 Meeting #159	S5-250542

Sophia Antipolis, France, 17 - 21 February 2025

# Source:                         NTT DOCOMO, China Mobile

# Title:	pCR TR28.869 VNF configuration management

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add solution and evaluation for Cloud-native VNF configuration management.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

### 5.1.1	Use case #1: Cloud-native VNF configuration management

#### 5.1.1.1	Description

Besides allocation and configuration of the necessary resources, to bring up a cloud native VNF into operation, i.e. full instance provisioning and configuration of the corresponding VNF instances needs to take place. Configuration actions can also be performed during the cloud native VNF instance operation.

An operator needs to be able to manage and orchestrate through the 3GPP management system the configuration of the cloud native VNF instances.

The operator needs to be able to perform through the 3GPP management system, operations related to configuration management for VNF application parameters and VNF non-application parameters like:

- set and query configuration information,

- backup configuration information prior to deployment,

- rollback configuration information (e.g., after reverting software to a previous stable version),

- remove unnecessary configuration files from the configuration repository (e.g., very old versions),

- enable configuration validation, ensuring that configuration files adhere to standardized schemas and models.

#### 5.1.1.2	Potential requirements

REQ-CVNF_CM-1 The 3GPP management system should have the capability to configure cloud-native VNF instances.

REQ-CVNF_CM-2 The reference point between the 3GPP management system and external OAM entities should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of configuring cloud-native VNF instances.

REQ-CVNF_CM-3 The 3GPP management system should have the capability to perform configuration management operations for cloud-native VNF instances.

REQ-CVNF_CM-4 The reference point between the 3GPP management system and external OAM entities should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of supporting configuration management operations for cloud-native VNF instances.

Editor's Note: The use of the term cloud native network function and cloud native VNFs in the present document is FFS.

2nd Change

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	VNF Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of the VNF Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. Some key functionalities supported by the VNF Configuration Manager function are the capability to convey configuration information to one or more cloud-native VNF/VNFC instances, the capability to perform pre-configuration actions (e.g. create configuration backup) and post-configuration actions (e.g. rollback running configuration) and the capability to query configuration information of cloud-native VNF/VNFC instances.

From 3GPP management system perspective, MnS provisioning handles configuration of managed NFs defined by 3GPP during NF instance creation or NF instance modification. In both cases MnS provisioning service,  includes the necessary configuration related to the behavior and role of a NF (e.g., configuration of a gNB CU UP function). The 3GPP management system fully understands the semantics of this kind of configuration since it is needed to ensure that the 3GPP mobile network operates as intended by the network operator.

The VNF Configuration Manager does not understand the semantics of the configuration information that is conveyed to the cloud-native VNF/VNFC instances (both VNF application configuration parameters and VNF non-application configuration parameters).

Figure 5.1.1.3.1-1 depicts the interaction and reference point between 3GPP management system and the VNF Configuration Manager.

Figure 5.1.1.3.1-1: Interaction and reference point between 3GPP management system and VNF Configuration Manager

NOTE:	As specified in ETSI GS NFV-IFA 049 [2], the VNF Configuration Manager is a VNF generic OAM function, while close mapping between VNF generic OAM functions and Platform as a Service (PaaS) Services exists. According to ETSI GS NFV-IFA 049 [2], any VNF generic OAM function can be understood as a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

The present solution addresses the potential requirements REQ-CVNF_CM-1, REQ-CVNF_CM-2, REQ-CVNF_CM-3 and REQ-CVNF_CM-4.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.2	Network Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution considers the use of the Network Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. The Network Configuration Manager function can be used to set network configuration information related to one or more VNF/VNFC instances. For example, the Network Configuration Manager can be used to configure the VNF Connection Points (CPs), VNF networking for VNF instances in a service mesh, etc.

Figure 5.1.1.3.2-1 depicts the interaction and reference point between 3GPP management system and the Network Configuration Manager.

Figure 5.1.1.3.2-1: Interaction and reference point between 3GPP management system and Network Configuration Manager

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.3	Configuration Server

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of a Configuration Server introduced in ETSI GR NFV-EVE 022 [44]. The Configuration Server can be understood as a logically centralized storage for configuration, without limiting the implementation of the storage. In addition, Configuration Server services can be used to store and fetch configuration data, convert configuration data formats, validate configuration data schema and apply version control for configuration data. The interface exposed and operations supported by the Configuration Server are defined in ETSI GS NFV-IFA 049 [2].

NOTE:	Implementation of the Configuration Server, e.g. structure of configuration data, format of storage is not in scope of the present solution.

Figure 5.1.1.3.3-1 depicts the interaction and reference point between 3GPP management system and the Configuration Server.

Figure 5.1.1.3.3-1: Interaction and reference point between 3GPP management system and Configuration Server

As depicted in Figure 5.1.1.3.3-1, a consumer of the Configuration Server interfaces could be other VNF generic OAM function such as the VNF Configuration Manager defined in ETSI GS NFV-IFA 049 [2] or the 3GPP management system.

The solution enables the 3GPP management system having the capability to handle configuration data, like distributing, fetching and transforming configuration data, for one or multiple cloud-native VNFs.

The present solution addresses the potential requirements REQ-CVNF_CM-3 and REQ-CVNF_CM-4.

This potential solution related to option 2 in clause 5.1.5

##### 5.1.1.3.4	Using the existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for VNF application specific parameters configuration purposes.

- The 3GPP management system configures VNF with VNF application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]).

- The 3GPP management system interacts with NFV-MANO to configure VNF non-application parameters (as defined in clause 5.1.18 of TS 28.531[7]).

This solution is not using the VNF Generic OAM functions described in ETSI ISG NFV-IFA049 [2].

This potential solution related to option 1 in clause 5.1.5.

The existing 3GPP provisioning management service and the available NRMs cannot be used to support VNF non-application specific parameters configuration purposes.

The existing 3GPP provisioning management service and the available NRMs do not support functionalities related to configuration management (e.g., configuration backup, configuration rollback etc.).

2nd Change

##### 5.1.1.3.5	Proposed updates in the 3GPP management system

VNF generic OAM functions described in clause 5.1.1.3.1, clause 5.1.1.3.2 and clause 5.1.1.3.3 can be used to support configuration management for both VNF application parameters and VNF non-application parameters.

See discussion and Figure X-2 in Annex X for example interactions between VNF generic OAM functions to support configuration management.

From a 3GPP management system point of view, to enable Cloud-native VNF configuration management different options can be considered in the normative phase.

- Configuration management operation#1: Request for configuration backup

Option#1.1: According to this option the request to perform configuration backup is enabled by an additional attribute to the generic NRM specified in TS 28.622 [ref-ts28.622]. New attributes related to backup information can be name-contained by ManagedElement or ManagedFunction or Subnetwork. The MnS consumer needs to be able to request configuration backup operation for the VNF application specific parameters and/or the VNF non-application specific parameters.

    - For example, for the case of a ManagedFunction,a new attribute is proposed to be added named configBackupInfo in the ManagedFunction IOC. The attribute properties will be investigated in the normative phase.

  - Option#1.2: According to this option configuration backup capability is introduced to specific 3GPP NF IOCs. For example, an additional attribute configBackupInfo can be added in the AmfInfo <<dataType>> of the AMFFunction IOC specified in TS28.541 [17]. The attribute properties will be investigated in the normative phase.

Editor’s note: additional options might be considered.

From an MnS provisioning service point of view, according to TS 28.532 [10], the createMOI operation is invoked by MnS consumers to request a MnS producer to create a managed object instance. In the input parameters "attributeListIn" attributes related to configuration backup specified by the managed object class definition (generic NRM or specific IoCs) of the class to be created can be described by the consumer. If the MnS producer performs backup operation by assigning values that are not known to the MnS consumer (e.g., the IP address of the repository used to store configuration backup) then the attributeListOut needs to be updated accordingly.

See Annex X and Annex Y for example flows for performing configuration backup during NF deployment instance creation and NF deployment instance modification, using VNF generic OAM functions. Parameterization in the updated IOC(s) can enable options like "backupBeforeModification" or  "backupAfterModification".

- Configuration management operation#2: Request for configuration rollback

  - Option#2.1: Similar to option#1.1. Configuration rollback operations are applicable to the case of   modifyMOIAttributes request from MnS provisioning point of view.

  - Option#2.2: Similar to option#1.2 Configuration rollback operations are applicable to the case of   modifyMOIAttributes request from MnS provisioning point of view.

- Configuration management operation#3: Request for configuration of cloud native VNF non-application specific parameters configuration purposes.

- Option#3.1: Update the vnfParametersList attribute in ManagedFunction IoC specified in TS 28.622, with an additional attribute vnfConfig. Reference can be made to the vnfConfigurableProperty attribute of the VnfInfo as specified in ETSI GS NFV-IFA 008 [9]. Configuration management operation#4: Request for configuration validation

  - Option#4.1: According to this option the request to perform configuration validation is enabled by an additional attribute to the generic NRM specified in TS 28.622 [ref-ts28.622]. New attributes related to configuration validation can be name-contained by ManagedElement or ManagedFunction or Subnetwork. The MnS consumer needs to be able to request configuration validation operation for the VNF application specific parameters and/or the VNF non-application specific parameters.

    - For example, for the case of a ManagedFunction,a new attribute is proposed to be added named configValidationInfo in the ManagedFunction IOC. The attribute properties will be investigated in the normative phase.

  - Option#4.2: According to this option configuration backup capability is introduced to specific 3GPP NF IoCs. For example, an additional attribute configValidationInfo can be added in the AmfInfo <<dataType>> of the AMFFunction IOC specified in TS28.541 [17]. The attribute properties will be investigated in the normative phase.

Editor’s note: additional operations related to configuration management might be added.

3nd Change

#### 5.1.1.4	Evaluation of solutions

According to the updates proposed in clause 5.1.1.3.5, the NRM extensions and procedure needed to enable configuration management operations for VNF application parameters and VNF non-application parameters are described.

Based on the analysis in clause 5.1.1.3.5, the following attributes “configBackupInfo”, “vnfConfig” and “configValidationInfo” are proposed to be considered in the normative phase and these are common parts of NRM extension proposed and all the solutions described. Based on the proposed updated NRM:

- solution 1 (clause 5.1.1.3.1): The MnS producer can interact with the VNF configuration manager function defined in ETSI GS NFV-IFA 049 [2] to:

  - set and query VNF configuration information,

  - backup configuration information prior to deployment,

  - rollback configuration information (e.g., reverting software to a previous stable version),

  - enable configuration validation, ensuring that configuration files adhere to standardized IoCs.

- solution 2 (clause 5.1.1.3.2): The MnS producer can interact with the Network configuration manager function defined in ETSI GS NFV-IFA 049 [2] to:

  - set and query VNF network configuration information,

- solution 3 (clause 5.1.1.3.3): The MnS producer can interact with the Configuration server defined in ETSI GS NFV-IFA 049 [2] to:

  - set and query VNF network configuration information,

- solution 4 (clause 5.1.1.3.4): The MnS producer can:

  - set and query VNF network configuration information,

  - backup configuration information prior to deployment,

  - rollback configuration information (e.g., reverting software to a previous stable version),

  - enable configuration validation, ensuring that configuration files adhere to standardized IoCs.

The updates proposed in clause 5.1.1.3.5 together with solution 1, 2 and 3 like also solution 4 can be combined to provide a complete solution for configuration management. Additional operations like removing unnecessary configuration files will be also considered during the normative phase.

4th Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification".

[3]	ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV);Architectural Framework; Report on VNF generic OAM functions".

[4]	3GPP TR 28.834: "Technical Specification Group Services and System Aspects; Study on management of cloud-native Virtualized Network Functions (VNF)".

[5]	SP-230764 New WID on Management of cloud-native Virtualized Network Functions.

[6]	3GPP TS 28.526: "Technical Specification Group Services and System Aspects; Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[7]	3GPP TS 28.531: "Technical Specification Group Services and System Aspects; Management and orchestration; Provisioning".

[8]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[9]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[10]	3GPP TS 28.532: "Technical Specification Group Services and System Aspects; Management and orchestration; Generic management services".

[11]	ETSI GR NFV 003 (V1.8.6): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[12]	3GPP TS 28.555: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

…

NOTE:	Available at https://github.com/cncf/toc/blob/main/DEFINITION.md.

[43]	"The twelve-factor app".

NOTE:	Available at https://12factor.net.

[44]	ETSI GR NFV-EVE 022: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on VNF configuration".

[ref-ts28.622]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

End of Changes