3GPP TSG-SA5 Meeting #159	S5-250606

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add concepts and background

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add concepts and background to TS 28.579[1].

# 4	Detailed proposal

It is proposed to add concepts and background to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]                      3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[Z]	3GPP TS 28.532: "Management and orchestration; Generic management services".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

# 4 Concepts and background

## 4.X Overview

The present document specifies a generic approach to exposing management services (MnSs) to external MnS consumers through the common API framework (CAPIF). CAPIF is a framework comprising common API aspects (e.g., onboarding, publishing, and discovery) required to expose service APIs (see TS 23.222[Y]). An external MnS consumer is a MnS consumer that discovers and consumes management services using CAPIF.

As defined in clause 4.2 of 3GPP TS 28.533 [Z], a management service (MnS) is defined by different components, i.e., MnS component type A (management operations or notifications), MnS component type B (managed objects), and MnS component type C (performance and fault data). When exposing management services using CAPIF, it’s crucial to determine which components of the MnS (i.e., either A and B or A, B, and C) will be published and to configure the external MnS consumers with the proper access control rights within the management system.

End of Changes