3GPP TSG-SA5 Meeting #159	S5-250608

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case and requirements for logging the management service API invocations to the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the following proposals

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add a use case “Logging the management service API invocations to the CCF” and the corresponding requirements to TS 28.579[1].

# 4	Detailed proposal

It is proposed to add the following use case to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Z]                       3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

## X.Y Logging the management service API invocations to the CCF

### X.Y.1 Definition

When exposing service APIs to external MnS consumers, the NOP can benefit from monitoring information related to the service API invocations. The monitoring information can include:

- The invoked service API details (e.g., the name, resource(s), and operations)

- The identity of the external MnS consumer who performed the service API invocation (i.e., the API invoker ID)

- The result of the service API invocation (e.g., success or failure)

- The time and duration of the service API invocation

To monitor the service API invocations within the management system, the AEF of the MSED can support the logging functionality, which involves the following:

- Creation of the service API invocation log(s) using the “CAPIF_Logging_API_Invocation_API” data type (see clause 8.7 of 3GPP TS 29.222 [Z]).

- Sending the created service API invocation log(s) to the CCF via the CAPIF-3 interface.

Authorized consumers of the service API invocation logs (e.g., the AMF of the MSED and the charging function (CHF)) can access the logs for auditing and charging purposes.

### X.Y.2 Requirements

REQ-MExpo-Log-Z1: The 3GPP management system shall have the capability to create logs based on the service API invocations by the external MnS consumers.

REQ-MExpo-Log-Z2: The 3GPP management system shall have the capability to log the service API invocations to the CCF.

End of Changes