3GPP TSG-SA5 Meeting #159	S5-250646

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0373 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 CR TS 28.532 Correction of the event time format & correction of the “notificationType” for the “MoiChange” data type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-02-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Incorrect event time format. The specification states that for the event time, the following shall be true, “The semantics of Generalised Time specified by RFC 3339 [50] shall be used here”. However, the event time value does not follow the standardized format as per RFC 3339. - Incorrect “notificationType” for the “MoiChange” data type |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the event time values to follow the standarized internet date time as specified in RFC 3339 and correction of the “notificationType” for the “MoiChange” data type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Risk of having the event time mis-interpreted in different regions or countries since it does not follow the standarized date time format. - Inconsistent specfications can lead to confusion of the readers |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 12.1.1.4.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin Change

#### 12.1.1.4	Data type definitions

##### 12.1.1.4.1	General

This clause defines the data types used by the Provisioning MnS. Table 12.1.1.4.1-1 specifies the data types defined in the present document and Table table 12.1.1.4.1-2 the data types imported.

Table 12.1.1.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| CmNotificationTypes | 12.1.1.4.4.3 | Notification type (notifyMOICreation, etc.) |
| SourceIndicator | 12.1.1.4.4.4 | Indicates the source of the operation that led to the generation of the notification. |
| ScopeType | 12.1.1.4.4.5 | Scope type of a scope |
| Operation | 12.1.1.4.4.6 | Enum with "create", "delete" and "replace" |
| Insert | 12.1.1.4.4.8 | Enum with "before" and "after" |
| PatchOperation | 12.1.1.4.4.7 | Enum with "add", "replace", "remove", "copy", "move" and "test" |
| Resource | 12.1.1.4.1a.1 | Used for resource representations |
| Scope | 12.1.1.4.1a.2 | Used in the query part of HTTP GET and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the authority and path component of the URI |
| CorrelatedNotification | 12.1.1.4.1a.3 | Describes the correlated notifications of a single source |
| MoiChange | 12.1.1.4.1a.4 | Single MOI change reported by notifyMOIChanges |
| NotifyMOICreation | 12.1.1.4.1a.5 | Used in the request body of HTTP POST for the notification type notifyMOICreation |
| NotifyMOIDeletion | 12.1.1.4.1a.6 | Used in the request body of HTTP POST for the notification type notifyMOIDeletion |
| NotifyMOIAttributeValueChanges | 12.1.1.4.1a.7 | Used in the request body of HTTP POST for the notification type notifyMOIAttributeValueChanges |
| NotifyMOIChanges | 12.1.1.4.1a.8 | Used in the request body of HTTP POST for the notification type notifyMOIChanges |
| NotifyEvent | 12.1.1.4.1a.10 | Used in the request body of HTTP POST for the notification type notifyEvent |
| PatchItem | 12.1.1.4.1a.9 | Specifies a patch item of a patch document |

Table 12.1.1.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| DateTime | TS 28.623 [44] | Date and time |
| Dn | TS 28.623 [44] | DN type |
| SystemDN | TS 28.623 [44] | systemDN type |
| Uri | TS 28.623 [44] | URI type |
| AttributeNameValuePairSet | TS 28.623 [44] | Set of attribute name/value pairs |
| AttributeValueChangeSet | TS 28.623 [44] | Set of attribute names with their old and new values |
| Filter | TS 28.623 [44] | Filter type |
| NotificationId | TS 28.623 [44] | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| NotificationType | TS 28.623 [44] | Notification type |
| NotificationHeader | TS 28.623 [44] | Notification header |
| ErrorResponse | TS 28.623 [44] | Used in the response body of multiple HTTP methods in case of error |

##### 12.1.1.4.1a	Structured data types

###### 12.1.1.4.1a.1	Type Resource

Table 12.1.1.4.1a.1 -1: Definition of type Resource

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| id | string | Identifier of the resource object | M |
| objectClass | string | Object class of the resource object | O |
| objectInstance | Dn | Object instance of the resource object | O |
| attributes | object | "attributes" (JSON) object whose members are the IOC attributes (except for "id”, "objectClass” and "objectInstance"). | M |
| n/a | map(array(object)) | Name contained objects | M |

This definition of "Resource" does not specify any attributes or name contained objects. Resource representations with specific attributes and name contained objects are contained in the NRM definitions. These definitions should be used in implementations of the Provisioning MnS instead of this generic definition.

###### 12.1.1.4.1a.2	Type Scope

Table 12.1.1.4.1a.2-1: Definition of type Scope

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| scopeType | ScopeType | Used in the query component of HTTP GET and HTTP DELETE together with scopeLevel to extend the set of targeted resources beyond the base resource identified with the authority and path component of the URI | M |
| scopeLevel | integer | Used in the query component of HTTP GET and HTTP DELETE together with scopeType to extend the set of targeted resources beyond the base resource identified with the path component of the URI | M |

###### 12.1.1.4.1a.3	Type CorrelatedNotification

Table 12.1.1.4.1a.3 -1: Definition of type CorrelatedNotification

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| source | Dn | Source of the correlated notifications | M |
| notificationIds | array(NotificationId) | Notification identifiers of correlated notifications of that source | M |

###### 12.1.1.4.1a.4	Type MoiChange

Table 12.1.1.4.1a.4 -1: Definition of type MoiChange

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| op | Operation | Operation associated to the reported change ("add", "remove , "replace"). | M |
| path | string | URI path component segments specifying when appended to "href" the created, deleted or updated resource or secondary resource | M |
| insert | Insert | Indicates whether the new attribute element was added before or after the attribute element specified by "path", only valid for attributes with the property isOrdered=True. It can take the values "before" and "after". If missing, it defaults to "before". The "insert" attribute shall be supported only when changes from YANG defined NRMs are reported. For JSON defined NRMs the attribute shall not be supported. | CM |
| value | any type | New value of the created or updated resource or secondary resource. Optional old value of the deleted resource or secondary resource | M |
| oldValue | any type | Old value of the updated secondary resource | O |

The properties "op", "path" and "value" shall use the 3GPP JSON Patch format (TS 32.158 [15]) for reporting NRM changes. The "merge" operation specified by 3GPP JSON Patch is not supported in "notifyMOIChanges". The "move", "copy" and "test" operations specified by JSON Patch are not supported either.

The "oldValue" is an optional extension for "notifyMOIChanges" allowing to report also the value that the attribute had before replacing the value with the new value, that is contained in "value".

The following example notification (where JSON is expressed in YAML notation) reports an object creation

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1

value:

id: 1,

objectClass: ClassA,

attributes:

attrA: 123

attrB:

subAttrB1: ABC

subAttrB2: 56

The following example reports the deletion of that object.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1

The following example reports the addition of a new attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrC

value: xyz

The following example reports the deletion of the attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrC

The following example reports a value change for the simple attribute "attrA".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

When the old value is reported as well, the notification looks like.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

oldValue: 123

The following example reports a value change for the complex attribute "attrB".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

The previous two notifications can be combined into a single notification as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

Note the operation "replace" has replace semantics and not merge semantics. The following notification reports the value change of the attribute field "attrB:subAttrB1" to "def" and the deletion of the attribute field "attrB:subAttrB2".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: def

The value change of the attribute field "attrA:subAttrB1" is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA/subAttrB1

value: def

Assume "attrD" is a JSON array with simple elements, then the creation of this multi-valued attribute is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 1

- 2

- 3

Its deletion is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrD

The complete replacement of the array is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 11

- 21

- 31

The following example reports the second item in the array changed to "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrD/1

value: 22

Note the array index of the second item is "1".

Assume now "attrE" is a JSON array with complex array items, for example.

[{subItemE1: 11, subItemD2: abc}, {subItemE1: 21, subItemE2: def}, {subItemE1: 31, subItemE2": ghi}.

A value change to

[{subItemE1: 11, subItemE2: abc}, {subItemE1: 21, subItemE2: xyz}, {subItemE1: 31, subItemE2": ghi}.

is reported by

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/1/subItemE2

value: xyz

When "subItemE2" is defined as array item key at stage 2, then "attrE" should contain a JSON map.

attrE:

11:

subItemE2: abc

21:

subItemE2: def

31:

subItemE2: ghi

The same change as above is now reported by the notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/21/subItemD2

value: xyz

When all attributes of an object have been updated with a new value, the MnS producer may use a compact format reporting that the "attributes" container was updated completely.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } } ] }

Note that clause 4.3 of IETF RFC 6902 [36] does not consider it as an error if an attribute value is replaced with exactly the same value. For that reason, it would not be an error if in the example above an attribute value is included in the "value" property that did not change value. A MnS producer may consider this compact format hence also for the case that not all attributes of an object have been updated.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "moiChange", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" } } ] }

To allow the MnS consumer to understand which attributes have been updated, the MnS producer may decide to send the following notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" }, "oldValue": { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

###### 12.1.1.4.1a.5	Type NotifyMoiCreation

Table 12.1.1.4.1a.5 -1: Definition of type NotifyMoiCreation

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOICreation") | M |
| eventTime | DateTime | Event (MOI creation) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | The attributes (name/value pairs) of the created MOI. | O |

The following example shows a notification reporting the creation of an object with two attributes "attrA" and "attrB". Note that the notification includes the name/value pairs representing the attributes of the created object only and not the complete object representation.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeList": { "attrA": "valueAttrA", "attrB": "valueAttrB" } }

The creation of an empty object not containing any attribute values is reported as follows.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeList": { } }

###### 12.1.1.4.1a.6	Type NotifyMoiDeletion

Table 12.1.1.4.1a.6 -1: Definition of type NotifyMoiDeletion

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIDeletion") | M |
| eventTime | DateTime | Event (MOI deletion) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | Attributes (name/value pairs) of the deleted MOI. | O |

The following example demonstrates the deletion of an object. The message body includes the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1" }

The message body may include the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeList": { "attrA": "valueAttrA", "attrB": "valueAttrB" } }

###### 12.1.1.4.1a.7	Type NotifyMoiAttributeValueChanges

Table 12.1.1.4.1a.7 -1: Definition of type NotifyMoiAttributeValueChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIAttributeValueChanges") | M |
| eventTime | DateTime | Event (MOI attribute value changes) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeListValueChanges | AttributeValueChangeSet | List with names of changed attributes, together with new value and optionally old value | M |

The following example notification reports the modification of the attribute values for "attrA" and "attrB".

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } ] }

In addition to the new values, the old values may be included in the notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" }, { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

In the example above the attribute values are of simple type. For attributes of structured type, the question is if all attribute fields of an attribute, where at least one attribute field changed its value, need to be reported or only those attribute fields that changed value. Assume an attribute field that did not change value is not included in the notification. In this case the MnS consumer receiving the notification cannot tell if the attribute field was deleted or if the attribute field did not change value. It is not possible to distinguish these two cases based on the information in the notification. For that reason, always all attribute fields of an attribute need to be included in the notification. For structured attributes with many attribute fields this may not be very efficient.

In the next example "attrA" is a structured attribute with the attribute fields "attrFieldAA" and "attrFieldAB". The attribute field "attrFieldAA" changed value, the attribute field "attrFieldAB" did not change value. The attribute value change notification reporting this change may look as follows.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": { "attrFieldAA": "newValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } }, { "attrA": { "attrFieldAA": "oldValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } } ] }

Note also that for multi-valued attributes all attribute elements of the new value need to be reported. It is not possible to report only added, deleted, or modified attribute elements. Furthermore, the notification does not allow reporting of deleted attributes. When this is required the MnS producer needs to include always all attributes of the object in the notification. It is outside the present document how the MnS producer signals to the MnS consumer if all attributes or only the changed ones are included in the attribute value change notification. The notification itself does not include this information.

###### 12.1.1.4.1a.8	Type NotifyMoiChanges

Table 12.1.1.4.1a.8 -1: Definition of type NotifyMoiChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of a common ancestor resource (object) of the resources for which changes are reported. A MnS producer may set this attribute always to the parent of the root resource in the MIB. | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4]. | M |
| notificationType | NotificationType | Notification type (notifyMOIChanges) | M |
| eventTime | DateTime | Event (NRM updates) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| moiChanges | array(MoiChange) | MOI changes to be reported | M |

###### 12.1.1.4.1a.9	Type PatchItem

Table 12.1.1.4.1a.9 -1: Definition of type PatchItem

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| op | PatchOperation | Patch operation. | M |
| from | string | Present only for "copy" and "move" operations, identifies the value to be copied or moved to the location specified by path. | M |
| path | string | Path specifying the patched value. | M |
| value | any type | New value for the resource identified by "path". | M |

###### 12.1.1.4.1a.10	Type NotifyMoiEvent

Table 12.1.1.4.1a.10 -1: Definition of type NotifyEvent

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyEvent") | M |
| eventTime | DateTime | Date and time of the event | M |
| systemDN | SystemDN | It carries the DN of producer of the notification. | M |
| specificProblem | SpecificProblem | It indicates a problem detected | M |
| additionalText | string | It carries additional information. | O |
| additionalInformation | AttributeNameValuePairSet | It carries additional information. | O |

The following is an example of the notifyEvent notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: myMns.mytelecom.com Content-Type: application/json { "href": "http://myNode.com/ManagedElement=ME1 ", "notificationId": 123456789, "notificationType": "notifyEvent", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=myNode.com,ManagedElement=ME1,MnsAgent=MA1", "specificProblem": "Restart", "additionalText": "Restart due to overheating", "additionalInformation": { "temperature": "94.7", "trendIndication": "MORE_SEVERE" } }

##### 12.1.1.4.2	Void

##### 12.1.1.4.3	Void

##### 12.1.1.4.4	Simple data types and enumerations

12.1.1.4.4.1	General

This clause defines simple data types and enumerations that are used by the data structures defined in the previous clauses.

12.1.1.4.4.2	Simple data types

Table 12.1.1.4.3.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| n/a | n/a | n/a |

12.1.1.4.4.3	Enumeration CmNotificationTypes

Table 12.1.1.4.4.3-1: Enumeration CmNotificationTypes

| Enumeration value | Description |
| --- | --- |
| notifyMOICreation | Notification type is notifyMOICreation |
| notifyMOIDeletion | Notification type is notifyMOIDeletion |
| notifyMOIAttributeValueChanges | Notification type is notifyMOIAttributeValueChange |
| noitifyMOIChanges | Notification type is notifyMOIChanges |
| notifyEvent | Notification type is notifyEvent |

12.1.1.4.4.4	Enumeration SourceIndicator

Table 12.1.1.4.4.4-1: Enumeration SourceIndicator

| Enumeration value | Description |
| --- | --- |
| RESOURCE_OPERATION | The notification was generated in response to an internal operation of the resource. |
| MANAGEMENT_OPERATION | The notification was generated in response to a management operation applied across the managed object boundary external to the managed object |
| SON_OPERATION | The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . |
| UNKNOWN | It is not possible to determine the source of the operation. |

12.1.1.4.4.5	Enumeration ScopeType

Table 12.1.1.4.4.4.1-1: Enumeration ScopeType

| Enumeration value | Description |
| --- | --- |
| BASE_ONLY | Selects only the base resource. The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_ALL | Selects the base resource and all of its subordinate resources (incl. the leaf resources). The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_NTH_LEVEL | Selects all resources on the level, which is indicated by the "scopeLevel" parameter, below the base resource. The base resource is at "scopeLevel" zero. |
| BASE_SUBTREE | Selects the base resource and all of its subordinate resources down to and including the resources on the level indicated by the "scopeLevel" parameter. The base resource is at "scopeLevel" zero. |

12.1.1.4.4.6	Enumeration Operation

Table 12.1.1.4.4.4.6-1: Enumeration Operation

| Enumeration value | Description |
| --- | --- |
| add | Create operation |
| remove | Delete operation |
| replace | Replace operation |

###### 12.1.1.4.4.7	Enumeration PatchOperation

Table 12.1.1.4.4.4.7-1: Enumeration PatchOperation

| Enumeration value | Description |
| --- | --- |
| add | Add operation |
| replace | Replace operation |
| remove | Remove operation |
| copy | Copy operation |
| move | Move operation |
| test | Test operation |

12.1.1.4.4.8	Enumeration Insert

Table 12.1.1.4.4.4.8-1: Enumeration Insert

| Enumeration value | Description |
| --- | --- |
| before | Specifies the new attribute element is inserted before the attribute element identified by the "path" attribute of "MoiChange". |
| after | Specifies the new attribute element is inserted after the attribute element identified by the "path" attribute of "MoiChange". |

End Change