3GPP TSG-SA5 Meeting #159	S5-250651

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0262 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.105 Use Case and Requirements for ML model training for multiple contexts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Use Case and Requirement of ML model training for multiple contexts has been discussed and agreed (in TR 28.858). This pCR is to normatively propose the use case and requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add New Use Case and Requirements for ML model training for multiple contexts |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2.x(new), 6.2b.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Change

### 6.2b.2.x	ML model training for multiple contexts

#### 6.2b.2.x.1	Description

ML models can be trained for similar contexts allowing for efficient management of these ML models. If an ML model that is trained for a particular context needs to be updated, it can be realized using an ML model that was trained for different context as the baseline.

#### 6.2b.2.x.2	Use cases

##### 6.2b.2.x.2.1	ML model training for multiple contexts

Although the ML model may provide an AI/ML inference service for multiple scenarios, there are similarities in the contexts where ML models operate and perform AI/ML inferences. For e.g. two ML model instances for the same inference type in urban or rural areas would have significant overlap in their contexts with respect to their type of learning, performance characteristic, task solving, and training and inference time. The context similarity can be leveraged in forming a cluster of ML models, where ML model instances in the cluster are trained either from the same previously trained ML model or from an ML model previously trained for another similar context as the baseline. The training of an ML model for multiple contexts allows for efficiency by cluster training rather than individually training each one of them. ML training needs to support the capability to train the cluster of ML models from the same baseline ML model or from an ML model previously trained for another similar context as the baseline. As input to the training, the clustering criteria needed to distinguish the ML model instances i.e. which ML models can form the cluster and trained together indicating similarities in context, learning paradigm, evaluation performance metrics, task type, application domain, training and inference time etc may be provided by the MnS consumer.

In the case of degradation of ML models, updating of ML models is expected to be triggered. For ML models created through cluster training, the retraining of a degraded ML model could be triggered to start from another member of the cluster, i.e. start from an ML model with another context to create a new ML model with the desired context.

End of First Change

Start of Second Change

### 6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_CLUSTER-TRAIN-1 | The MLT MnS producer should have a capability for an authorized MnS consumer to request as per clustering criteria the training of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model. | ML model training for multiple contexts (clause 6.2b.2.x) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |  |  |

End of Changes