3GPP TSG-SA5 Meeting #159	S5-250656

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Samsung

# Title:	Rel-19 pCR TS 28.567 Add use-case and Requirements for Performance monitoring of Closed control loop

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

The group is asked to discuss and approval

# 2	References

[1]	3GPP TR 28.867: "Study on closed control loop management"

# 3	Rationale

This contribution proposes to add in TS 28.567 the following:

- Use-case on Performance Evaluation of a Closed Control  Loop, as the use-case was studied in Clause 5.11.1 of TR 28.867 and recommended for normative phase in Clause 5.11.4 of TR 28.867.

- Requirements for use-case on Performance Evaluation of a Closed Control  Loop as it was studied in Clause 5.11.2 and recommended for normative phase in Clause 5.11.4 of TR 28.867

# 4	Detailed proposal

1st Change

## X	Performance Evaluation of a Closed Control Loop

### X.1	Description

The advanced monitoring functionalities of a CCL can provide real-time insights into the performance and outcomes of a CCL. The monitoring activity for a Closed Control Loop may result in further actions that happen in the operation phase, e.g., evaluate and update, in order to change the closed control loop settings and improve its performance. So, there is a need to evaluate the performance of a Closed Control Loop itself. Such metrics are important to understand and change a CCL's behaviour and to improve its performance to pursue the assigned goal(s).

For example, certain performance aspects of a CCL can be very crucial to know in order to evaluate and decide upon a CCL's performance, such as the number of breached goals, time taken to meet a breached goal, number of conflicts occurred by a CCL etc. With the knowledge of such performance aspects of an existing CCL, a MnS consumer can more effectively update or create a new CCL.

An operator can also compare different CCLs based on these performances and choose the best one for its network deployment.

2nd Change

### X.2	Potential Requirements

REQ-CCL-PERF-1: The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of occurrences of an assurance goal breach.

REQ-CCL-PERF-2: The 3GPP management system should be able to obtain a CCL's performance with respect to the time taken by CCL to meet a breached goal.

REQ-CCL-PERF-3: The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of conflicts occurred by a CCL.

End of changes