3GPP TSG-SA5 Meeting #159	S5-250702

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Architecture updates

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

[2]	3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture; Phase 3

# 3	Rationale

The architecture in the TR 23.700-29 [2] are only informative, this means that other architectures are possible and including these in the study may limit the solutions.

# 4	Detailed proposal

First change

## 4.3	Store and Forward Satellite operation

As specified in the TS 22.261 [2], the new capability that S&F operation is an operation mode of a 5G system with satellite-access, allows the satellite to store and forward data when satellite connectivity is intermittently/temporarily unavailable (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network). The following from Figure J-1 of TS 22.261 [2] shows the high-level network architecture for S&F operation.

Figure 4.3-1: Illustration of "S&F Satellite operation" modes in a 5G system with satellite access

The conclusions in TR 23.700-29 [3], have two informative architectures for the support S&F operation:

-	Split MME architecture, where the eNB and part of MME functions on the satellite, and other network functions are deployed on the ground, including part of MME functions and HSS.

-	full CN onboard the satellite architecture, where the eNB and whole CN including MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. The implementation of the proxies and the interface between them is out of 3GPP scope.

End of changes