3GPP TSG-SA5 Meeting #159	S5-250732

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei, Ericsson, Nokia

# Title:	Update evaluation and conclusion for Topic 3

# Document for:	Approval

Agenda Item:	7.5.2

1 	Discussion

This is a pCR to add the evaluation on the solutions about API charging in TR 28.849.

2	References

[1]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

3	Rationale

This pCR proposes to introduce the Evaluation on the solutions of API charging.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.849 [1].

First change

6.3.6	Evaluation

6.3.6.1	Solutions evaluation for Key issue #3.1

Solution #3.1 provides the CAPIF Core Function (CCF) capability to support CAPIF Service APIs invocation/notification charging. A new charging specification is required focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3] for the CAPIF service API invocation/notification charging.

Solution #3.2 provides the capability to support charging of API invocation, by using the current Northbound Application Program Interfaces (APIs) charging architecture and information. The clarification about the relationship between CAPIF charging and NEF charging in the TS 32.254 [3] is required.

Solution #3.x provides the capability to support charging of API Invocation Management Lifecycle (creation/deletion/access/reading) for the API Provider. A new charging specification is required focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3].

Next change

6.3.7	Conclusion

From solution #3.2 it is concluded that the current Northbound Application Program Interfaces (APIs) charging architecture and information can be reused with some extension. All solutions add CAPIF Core Function (CCF) as the CTF, therefore having the CTF in CAPIF Core Function (CCF) seems to be the preferred solution and adding triggers and charging information according to solutions #3.1. It is concluded the solution #3.1 and solution #3.2 are selected to be normative work, by extending in the TS 32.254 [3].

End of change