3GPP TSG SA WG5 Meeting #159	S5-250737

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei, CMCC

# Title:	New solutions for UAV information

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide new solutions for Topic 1 in UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

To distinguish UAV from other types of 3GPP UE, one possible way is to introduce a generic UE type with enumerated values. This solution allows future extensibility in case more types of UE require charging differentiation.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.1	Topic 1：Converged charging with UAV Information

5.1.1	Use cases

5.1.1.1	Use case #1a: Converged charging with UAV Information

As defined in TS 23.256 [3], the UAV is associated with the following identifiers in the 3GPP system:

-	CAA-level UAV Identity: A UAV is assigned a CAA-level UAV Identity by functions in the aviation domain (e.g. USS). The UAV provides the CAA-level UAV Identity to the 3GPP system during UUAA procedures.

-	3GPP UAV ID: A 3GPP UAV ID is associated to the UAV by the 3GPP system in the subscription information and is used by the 3GPP system to identify the UAV. GPSI in the format of External Identifier is used as the 3GPP UAV ID.

The UAV, as a 3GPP UE configured for UAS services, can be identified by the UAV identifier, which makes it possible for charging differentiation.

For the scenario of using 3GPP network as the transport network for supporting UAS services, the charged party and charging party can be:

-	Charged party: UAS-SC who accesses the 3GPP network.

-	Charging party: UAS-MNO who provides UAS services to UAS-SC.

The potential charging requirements for this Use Case are: REQ-CH_ UAS_ID-01.

Next change

5.1.4.x	Solution #1.x: Generic User Information to identify UAV

5.1.4.x.1	General description

This solution #1.x addresses Key Issue#1a. To distinguish the UAV or any other future proofed 3GPP UE types for charging differentiation, a generic IE can be introduced to hold the specific type of a UE.

As per TS 23.256 [3] clause 5.2.2 and clause 5.2.3, AMF / SMF can receive the UAV information (e.g. CAA-Level UAV ID or the UAV) in the UE Registration / PDU Session Establishment request. In this case, both AMF and SMF can identify the specific type of the UE, e.g. whether it is a UAV or any other specific type of UE.

The User Information IE, which are part of the PDU session charging information in TS 32.255 [10] and Registration charging information in TS 23.256 [12], can be extended to include a new sub IE, e.g. User Identifier Ext, to indicate the specific type of UE defined in the 3GPP system. Table 5.1.4.x.1-1 demonstrate the example of the extended User Information.

Table 5.1.4.x.1-1: Extension to the User Information IE

| Information Element | Category | Description |
| --- | --- | --- |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. The detail identification of the wireline access is specified in clause 4.7.7 of TS 23.316 [203]. |
| unauthenticated Flag | OC | This field indicates the served SUPI is not authenticated. |
| Roamer In Out | OC | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| User Identifier Ext | OC | This field indicates the specific type of UE defined in the 3GPP system, e.g. UAV when charging for the UAV scenario. This field can be used to provide additional information for user identifier. |

End of change