3GPP TSG-SA5 Meeting #159	S5-250823

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0180 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 add stage3 solution for mobility performance analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, stage 3 only defines the general IOC for MDA report, and the solution for MDA report of each use case is not clear. In this proposal, the MDA output of mobility performance analysis is used as an example to describe the solution in stage 3. After the conclusion is reached, the solution for stage 3 of all MDA types can be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the stage3 solution for the MDA output of mobility performance analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of mobility performance anlaysis will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MdaReport.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1539 at commit aa8091e956ab960223d2e2d8c6d7fb74c5b79a6e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28104_MdaReport.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA Report

version: 19.0.0

description: >-

OAS 3.0.1 specification of the MDA Report

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA Report

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAOutputs:

type: object

properties:

mDAType:

type: string

readOnly: true

mDAOutputList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputEntry'

analyticsWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindowRo'

confidenceDegree:

type: number

format: float

MDAOutputEntry:

oneOf:

- $ref: '#/components/schemas/MobilityPerformanceAnalysis'

- type: object

properties:

mDAOutputIEName:

type: string

readOnly: true

mDAOutputIEValue: {}

#-------- Definition of MDA Report --------------------------------------------

MDAReport:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mDAReportID:

type: string

mDAOutputs:

type: array

items:

$ref: '#/components/schemas/MDAOutputs'

mDARequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#-------- Definition of specific MobilityPerformanceAnalysis MDAoutput -----------------------------

MobilityPerformanceAnalysis:

description: >-

This data type is the "MDAOutputs" data type to represent MnS consumer's request for coverage problem analysis

type: object

properties:

mDAType:

enum:

- MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS

mDAOutputList:

type: array

items:

type: object

allOf:

- $ref: '#/components/schemas/MobilityPerformanceIssueIdentifier'

- $ref: '#/components/schemas/MobilityPerformanceIssueRootCause'

- $ref: '#/components/schemas/MobilityPerformanceIssueLocation'

#-------Definition of the MobilityPerformanceAnalysis MDAoutput ---------#

MobilityPerformanceIssueIdentifier:

description: >-

This data type is the "MDAOutputEntry" data type with specialisations for mobilityPerformanceIssueIdentifier. It describes the mobility Performance Issue Identifier.

type: object

properties:

mDAOutputIEName:

type: string

enum:

- mobilityPerformanceIssueIdentifier

mDAOutputIEValue:

type: integer

MobilityPerformanceIssueRootCause:

description: >-

This data type is the "MDAOutputEntry" data type with specialisations for mobilityPerformanceIssueRootCause. It describes the mobility Performance Issue Root Cause.

type: object

properties:

mDAOutputIEName:

type: string

enum:

- mobilityPerformanceIssueRootCause

mDAOutputIEValue:

type: string

enum:

- TooLongMobilityInterruptionTime

- PoorCoverageOfTheCelledge

- InappropriateHandoverParameters

- Other

MobilityPerformanceIssueLocation:

description: >-

This data type is the "MDAOutputEntry" data type with specialisations for MobilityPerformanceIssueLocation. It describes

the Mobility Performance Issue Location.

type: object

properties:

mDAOutputIEName:

type: string

enum:

- MobilityPerformanceIssueLocation

mDAOutputIEValue:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change